5	PROGRAMMING	5.1-1

ISSUE	1	18/5/99

5.1	INTRODUCTION	1.0	18/5/99	5.1-1

5.1.1	TO ENTER THE PROGRAMMING MODE	5.1-1

5.1.2	HOW TO PROGRAM	5.1-1

5.1.3	EXAMPLE OF ADMIN. PROGRAM	5.1-2

5.1.4	BUTTON DEFINITION	5.1-3

5.2	ADMIN PROGRAMMING INDEX	1.0	18/5/99	5.2-1

5.2.1	DEFAULT VALUES	5.2-2

5.3	DATABASE INITIALIZATION (MAIN-MENU1) 	1.0	18/5/99	5.3-1

5.4	STATION BASE PROGRAM (MAIN-MENU2) 	1.0	18/5/99	5.4-1

5.4.1	STATION ATTRIBUTES (SUB-MENU1)	5.4-1

5.4.2	STATION ISDN ATTRIBUTES (SUB-MENU 2)	5.4-3

5.4.3	STATION COS  (SUB-MENU 3)	5.4-4

5.4.4	CO LINE GROUP ACCESS   ( SUB-MENU 4 )	5.4-5

5.4.5	PRESET CALL FORWARD  ( SUB-MENU 5 )	5.4-5

5.4.6	FLEX BUTTON ASSIGNMENT ( SUB-MENU 6 )	5.4-6

5.4.7	WARM LINE SELECTION  ( SUB-MENU 7 )	5.4-8

5.4.8	PAGE ZONE  ( SUB-MENU 8 )	5.4-9

5.4.9	STATION LANGUAGE CODE  ( SUB-MENU 9 )	5.4-9

5.5	CO LINE PROGRAMMING ( MAIN-MENU 3 ) 	1.0	18/5/99	5.5-1

5.5.1	CO LINE ATTRIBUTE ( SUB-MENU 1 )	5.5-1

5.5.2	ISDN CO LINE ATTRIBUTE ( SUB-MENU 2 )	5.5-3

5.5.3	CO Line Group Assignment ( SUB-MENU 3 )	5.5-4

5.5.4	DISA TYPE ( SUB-MENU 4 )	5.5-5

5.5.5	CO FLASH TIMER ( SUB-MENU 5 )	5.5-6

5.5.6	CO LOOP SUPERVISION TIMER ( SUB-MENU 6 )	5.5-7

5.5.7	CO RING ASSIGNMENT ( SUB-MENU 7 )	5.5-8

5.6	ISDN PROGRAM ( MAIN-MENU 4 ) 	1.0	18/5/99	5.6-1

5.6.1	TEI TYPE  ( SUB-MENU 1 )	5.6-2

5.6.2	CALLING SUB-ADDRESS (SUB-MENU 2)	5.6-2

5.6.3	INCOMING ZERO INSERTION  ( SUB-MENU 3 )	5.6-3

5.6.4	OUTGOING ZERO INSERTION  ( SUB-MENU 4 )	5.6-3

5.6.5	OUTGOING DIGIT CHECK  ( SUB-MENU 5 )	5.6-3

5.6.6	MY AREA CODE  ( SUB-MENU 6 )	5.6-4

5.6.7	MY NATION CODE ( SUB-MENU 7 )	5.6-4

5.6.8	INTERNATIONAL CODE ( SUB-MENU 8 )	5.6-4

5.6.9	ISDN COLP TABLE ( SUB-MENU 9 )	5.6-5

5.6.10	ISDN MSN TABLE ( SUB-MENU 10 )	5.6-5

5.6.11	DID DIGITS CONVERSION ( SUB-MENU 11 )	5.6-6

5.6.12	TRANSFER CODE (STA) (SUB-MENU 12 )	5.6-7

5.6.13	ISDN SUPPLEMENTARY SERVICES (SUB-MENU 13 )	5.6-7

5.6.14	ISDN B-CHANNEL LOOP BACK	5.6-7

5.7	SYSTEM BASE PROGRAMMING ( MAIN-MENU 5 ) 	1.0	18/5/99	5.7-1

5.7.1	SYSTEM ATTRIBUTE  ( SUB-MENU 1 )	5.7-4

5.7.2	ATTENDANT STATION  ( SUB-MENU 2 )	5.7-6

5.7.3	SET SYSTEM DATE/TIME  ( SUB-MENU 3 )	5.7-6

5.7.4	LCD DISPLAY MODE ( SUB-MENU 4 )	5.7-7

5.7.5	PBX ACCESS CODES ( SUB-MENU 5 )	5.7-8

5.7.6	DID/DISA DESTINATION ( SUB-MENU 6 )	5.7-9

5.7.7	DISA RETRY COUNTER ( SUB-MENU 7 )	5.7-10

5.7.8	SMDR ATTRIBUTE ( SUB-MENU 8 )	5.7-11

5.7.9	DIAL SPEED RATIO   ( SUB-MENU 9 )	5.7-12

5.7.10	ALARM ATTRIBUTE  ( SUB-MENU 10 )	5.7-13

5.7.11	EXTERNAL CONTROL CONTACT  ( SUB-MENU 11 )	5.7-14

5.7.12	BGM(Background Music) TYPE  ( SUB-MENU 12 )	5.7-15

5.7.13	MOH(Music On Hold) TYPE  ( SUB-MENU 13 )	5.7-15

5.7.14	SYSTEM GAIN CONTROL ( SUB-MENU 14 )	5.7-16

5.7.15	AUTO FAX TRANSFER ( SUB-MENU 15 )	5.7-18

5.7.16	EXTERNAL PAGE PORT ASSIGNMENT( SUB-MENU 16 )	5.7-18

5.7.17	DID/DISA FORWARD TO DVIB ( SUB-MENU 17 )	5.7-18

5.8	SYSTEM TIMER PROGRAMMING ( MAIN-MENU 6 ) 	1.0	18/5/99	5.8-1

5.8.1	SYSTEM TIMER 1 (SUB-MENU 1 )	5.8-1

5.8.2	SYSTEM TIMER 2 (SUB-MENU 2 )	5.8-2

5.9	TOLL TABLE ( MAIN-MENU 7 ) 	1.0	18/5/99	5.9-1

5.9.1	TOLL TABLES	5.9-1

5.10	AUTHORIZATION CODE TABLE ( MAIN-MENU 8 ) 	1.0	18/5/99	5.10-1

5.11	FLEXIBLE NUMBERING PLAN ( MAIN-MENU 9 ) 	1.0	18/5/99	5.11-1

5.12	LCR PROGRAM ( MAIN-MENU 10 ) 	1.0	18/5/99	5.12-1

5.13	STATION GROUP PROGRAM ( MAIN-MENU 11 ) 	1.0	18/5/99	5.13-1

5.14	DATABASE PRINT ( MAIN-MENU 12 ) 	1.0	18/5/99	5.14-1

# PROGRAMMING

## INTRODUCTION

The Aria-16 Key Telephone System can be programmed to meet each customer's individual need. All programming is done at station 100 (DKT 1) using KD-36D, KD/E-36D or LKD-30D digital key telephone.

Upon entering the program mode, the key telephone at station 100 cannot operate as a normal telephone but as a programming instrument with all of the buttons redefined. The keys of the dial pad are used to enter the various data fields and to enter numerical information. The 24 buttons located at the top of the phone (Flex Buttons) are used to indicate the specific data field and to enter information. Sometimes [SPEED] button is used to delete the data.

See tables in text for default data. If this pre-programming suits the customer, additional Admin program is not necessary. To change admin data, the user enters the admin programming mode and selects the program code. During Admin programming, other keysets in the system operate normally.

When Admin programming, LCD and LED’s indicate current programmed data and status. If the programmer enters correct data, then LCD and LED’s show the entered data and the data is stored in the temporary buffer area. Real system databases is not changed and has no effect on telephone operation unless permanent updating procedure is executed.  Pressing [HOLD/SAVE] button, all data in the temporary buffer (same as LCD and LED’s show their status) is saved into permanent memory. Tones are provided to let the programmer know whether data entry is correct (confirmation tone) or not  (error tone).

### TO ENTER THE PROGRAMMING MODE

- Lift handset or press the [MON] button on the admin station, and hear ICM dial tone (optional),

- Press [TRANS/PGM] button and dials *# (confirmation tone heard),

- Enter Admin. Password if the password has been set. This procedure places the station into the Admin. programming mode (confirmation tone heard).  And LCD will displays the first step of Admin. programming.(below LCD)

ADMIN PROGRAM START PRESS FLEX KEY(1-11)

### HOW TO PROGRAM

- There are 11 main-menus in the Admin. Programming mode.

- Each main program has several sub-menus and each sub-menu can have several terminal menus(sub-

menus of a sub-menu), but some sub-menus don’t have any terminal-menus(It means that sub-menus are

terminal-menus in this case). Refer the followings.

| MAIN-MENU |  A main-menu has several sub-menus.  To select a sub-menu, press a flex-key responding to the sub-menu number. |
| --- | --- |



| SUB-MENU |  A sub-menu may have several terminal-menus, but some sub-menus may not.  To select a terminal-menu, press a flex-key responding to the terminal-menu number. |
| --- | --- |



| TERMINAL-MENU |  Enter data at the terminal-menu with dialling digits or flex-keys. |
| --- | --- |

### EXAMPLE OF ADMIN. PROGRAM

PROCEDURE

| ADMIN PROGRAM START PRESS FLEX KEY(1-11) | - When the user enter Admin. mode, LCD displays the first message. Press a flex-key to select a main-menu among 11 main-menus. - (If the user program does not include LCR-package, the user cannot enter main-menu 10) |
| --- | --- |
| STA PGM ENTER STA RANGE | - The user selected the 2nd main-menu, “STATION BASE PROGRAM”, by pressing the 2nd flex-key in step 1. - Dial station range. (The second station number must be more than or equal to the first station number) |
| STA PGM,100-100 PRESS FLEX KEY(1-8) | - And then, press a flex key(1-8) or /. -  Flex key(1-8) : Select a sub-menu of STA BASE PROGRAM. -   : Move to the next main-menu, CO LINE BASE PROGRAM. -  : Move to the previous main-menu, INITIALIZATION. |
| - STA PGM,100-115,ATTRIBUT - PRESS FLEX KEY(1-10) | - If the user press a flex key 1, the LCD displays as the left side. |
| - STA PGM,100-115, DND - YES | - If the user press a flex key 1, the LCD displays as the left side. - And then press a flex key 1 to change the current value. Press [HOLD/SAVE] to update the database permanently. - - / : Go to the other same level menu. - - [TRANS/PGM] : Go to the Admin. start menu. - - [REDIAL] : Go to the 1 level upper menu. |

### BUTTON DEFINITION

Buttons are used for the following purposes in the Admin. program mode.

- [HOLD/SAVE] : Update the system database permanently.

- [TRANS/PGM] : Move to the start of admin program menu.

- [REDIAL] : Move to the 1 level upper level menu.

- (SUB-MENU  MAIN-MENU  or  TERMINAL-MENU  SUB-MENU)

- : Move to the next menu in the same level.(Round-robin)

- (Ex : DND(1st  terminal-menu of STATION ATTRIBUTE)  SPEED DIAL ACCESS(2nd terminal-menu

- of STATION ATTRIBUTE)

- : Move to the previous menu in the same level.(Round-robin)

- (Ex 1 : INITIALIZATION(1st main-menu)  PRINT DATABSE(last main-menu)

- Ex 2 : STATION CLASS OF SERVICE(3rd sub-menu of 2nd main-menu)  ISDN STATION ATTRIBUTE

- (2nd sub-menu of 2nd main-menu) )

- [SPEED] : Clear database of the current  terminal menu.

- FLEX-KEY :  1. Used for selecting a menu (main/sub/terminal) responding to the number.

2. Also used as toggle key for entering data, if entered value has only 2 case number

(i.e: Yes/No, Enable/Disable)

- DIGIT :  Used for entering data by dialling.

## ADMIN PROGRAMMING INDEX

| FLEX KEY | MAIN MENU | FLEX KEY | SUB MENU |
| --- | --- | --- | --- |
| 1 | INITIALIZATION | 1 | Station Database Initialization |
|  | (DB INIT) | 2 | CO Line Database Initialization |
|  |  | 3 | ISDN Tables Database Initialization |
|  |  | 4 | System Feature Database Initialization |
|  |  | 5 | System Timer Database Initialization |
|  |  | 6 | Toll Table Database Initialization |
|  |  | 7 | Authorization Code Table Database Initialization |
|  |  | 8 | Flexible Station Number Initialization |
|  |  | 9 | Flexible Button Program Initialization |
|  |  | 10 | LCR Database Initialization (LCR package only-MEMU) |
|  |  | 11 | All Database Initialization |
| 2 | STATION BASE PROGRAM | 1 | Station Attribute |
|  | (STA PGM) | 2 | ISDN Station Attribute |
|  |  | 3 | Station Class-Of-Service |
|  |  | 4 | CO Line Group Access |
|  |  | 5 | Preset Call Forward |
|  |  | 6 | Flex Buttons Assignment |
|  |  | 7 | Warm Line Selection |
|  |  | 8 | Page Zone |
| 3 | CO LINE BASE PROGRAM | 1 | CO Line Attribute |
|  | (CO PGM) | 2 | ISDN CO Line Attribute |
|  |  | 3 | Group Assignment |
|  |  | 4 | DISA Type |
|  |  | 5 | CO Flash Timer |
|  |  | 6 | CO Loop Supervision Timer |
|  |  | 7 | CO Line Ring Assignment |
| 4 | ISDN PROGRAM | 1 | TEI Type |
|  | (ISDN PGM) | 2 | Calling Sub-Address |
|  |  | 3 | Incoming Zero Insertion |
|  |  | 4 | Outgoing Zero Insertion |
|  |  | 5 | Outgoing Check Digit |
|  |  | 6 | My Area Code |
|  |  | 7 | My Nation Code |
|  |  | 8 | COLP Table (Entry no:01-10) |
|  |  | 9 | MSN Table (Entry no:01-24) |
|  |  | 10 | DID Digit Conversion Table |
| 5 | SYSTEM BASE PROGRAM | 1 | System Attribute |
|  | (SYS PGM) | 2 | Attendant Assignment |
|  |  | 3 | Setting System Date/Time |
|  |  | 4 | LCD Date/Time Display Mode |
|  |  | 5 | PBX Access Code |
|  |  | 6 | DID/DISA Destination |
|  |  | 7 | DISA Retry Counter |
|  |  | 8 | SMDR Attributes |
|  |  | 9 | Pulse Dial / Speed Ratio |
|  |  | 10 | Alarm Attributes |
|  |  | 11 | External Control Contact |
|  |  | 12 | BGM(BackGround Music) Type |
|  |  | 13 | MOH(Music On Hold) Type |
|  |  | 14 | System Gain Control |
| 6 | SYSTEM TIMERS | 1 | Exclusive Hold Recall Timers |
|  | (SYS TMR) | 2 | System Hold Recall Timer |
|  |  | 3 | Transfer Hold Recall Timer |
|  |  | 4 | I-Recall Timer |
|  |  | 5 | Attendant Recall Timer |
|  |  | 6 | CO Ring Detect Timer |
|  |  | 7 | CO Ring Stop Detect Timer |
|  |  | 8 | Pause Timer |
|  |  | 9 | CO Release Guard Timer |
|  |  | 10 | CO Dial Delay Timer |
|  |  | 11 | Preset Forward Timer |
|  |  | 12 | Call Forward Busy/No Answer Timer |
|  |  | 13 | DID/DISA No Answer Timer |
|  |  | 14 | Unsupervised Conference Timer |
|  |  | 15 | MSG Wait Reminder Tone Timer |
|  |  | 16 | Hook Switch Flash Timer |
|  |  | 17 | Door Open Timer(Enhanced) |
|  |  | 18 | Inter Digit Timer |
|  |  | 19 | Warm Line Timer |
|  |  | 20 | SMDR Timer |
|  |  | 21 | Wake-up Fail Ring Timer |
| 7 | TOLL TABLE | 1 | Toll Exception Table – Allow A (Entry no:01-20) |
|  |  | 2 | Toll Exception Table – Deny A (Entry no:01-10) |
|  |  | 3 | Toll Exception Table – Allow B (Entry no:01-20) |
|  |  | 4 | Toll Exception Table – Deny B (Entry no:01-10) |
|  |  | 5 | 1. Canned Toll Table –Allow (Entry no:01-10) |
|  |  |  | 2. Canned Toll Table –Deny (Entry no:01-10) |
| 8 | AUTHORIZATION CODE TABLE (AUTHOR CODE TABLE) | - | Authorization Code Table (Entry no:01-30) |
| 9 | FLEXIBLE NUMBERING PLAN | 1 | Numbering Plan Type(Extension / Basic) |
|  |  | 2 | Station Number Digit Size (3 / 2) |
|  |  | 3 | Flexible Station Number Assignment |
| 10 | LCR PROGRAM | 1 | LCR Access Mode |
|  | - MEMU needed | 2 | Day Zone |
|  |  | 3 | Time Zone of Day Zone |
|  |  | 4 | LDT Table |
|  |  | 5 | DMT Table |
| 12 | PRINT DATABASE | 1 | Station Database Print |
|  |  | 2 | CO Line Database Print |
|  |  | 3 | ISDN Database Print |
|  |  | 4 | System Feature Database Print |
|  |  | 5 | System Timer Database Print |
|  |  | 6 | Toll Table Print |
|  |  | 7 | Authorization Code Table Print |
|  |  | 8 | Flexible Station Number Print |
|  |  | 9 | LCR Database Print |
|  |  | 10 | All Database Print |

### DEFAULT VALUES

| NO | SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | STATION ATTRIBUTE | 1 | DND ( Do-Not-Disturb ) | YES / NO | YES |  |
|  |  | 2 | Speed Dial Access | YES / NO | YES |  |
|  |  | 3 | Page Access | YES / NO | YES |  |
|  |  | 4 | Call Forward Access | YES / NO | YES |  |
|  |  | 5 | PLA (Preferred Line Answer) | YES / NO | YES |  |
|  |  | 6 | Speaker-Phone/Headset | SP / HEAD | Speaker-Phone |  |
|  |  | 7 | Auto Speaker Selection | YES / NO | YES |  |
|  |  | 8 | ICM Box Signaling | YES / NO | YES |  |
|  |  | 9 | Automatic Hold | YES/NO | Station1:YES Station2-16:NO |  |
|  |  | 10 | Data Line Security | YES/NO | NO |  |
|  |  | 11 | DVIB access | YES/NO | NO |  |
| 2 | ISDN STATION ATTRIBUTE | 1 | Sub Address | YES/NO | NO | If this field is set to YES, then Aria sends the called extension number in called party sub address information of setup message to ISDN extension. |
|  |  | 2 | Long/Short | Long/Short | Short |  |
|  |  | 3 | MSN | YES/NO | NO | If this field is set to YES, then Aria sends the called extension number in called party number information of setup message to ISDN extension. |
|  |  | 4 | COLR | YES/NO | NO |  |
|  |  | 5 | COLP | YES/NO | NO |  |
|  |  | 6 | CLIP | YES/NO | NO |  |
| 3 | STATION COS | - | Day COS/Night COS | 11 - 77 | 11 | 2 Digit 1st Digit-Day COS 2nd Digit-Night COS |
| 4 | STATION CO Group Access | 1- 4 | CO Line Group | 1-4 | 1-4 | 1st Flex : CO GRP 1 : 4th Flex : CO GRP 4 |
| 5 | Preset Call Forward | - |  | Station No (2Digit or 3Digit) | Not Assigned | The ISDN station or ICM box cannot be assigned as preset call forward station. |
| 6 | Flex Buttons |  |  | 1-6 |  | Type |
|  | Assignment |  |  | -- |  | 1: User Button |
|  |  |  |  | 1-6 |  | 2: CO x |
|  |  |  |  | 1-4 |  | 3: Reserved |
|  |  |  |  | - |  | 4: LOOP |
|  |  |  |  | STA No. |  | 5: STA xxx |
|  |  |  |  | SPD Bin No. (01-99) |  | 6: SPD xx |
| 7 | Warm Line Selection | - |  | 1-4 | Not Assigned | 1:Flex Button 1) 2:CO Line 3:CO Group 4:Station |
| 8 | Page Zone | - |  | 1-2 | 1 | 1:Page Zone 1 2:Page Zone 2 |

Table 5.2.1-1   STATION BASE PROGRAM  (MAIN MENU 2)

- Under following conditions, flex button cannot be assigned as warm line.

- If any station among entered range is SLT or 2B-SLT, you cannot save input value.

- If you entered the bigger flex button number than stations have, you cannot save input value.

| NO | SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CO LINE | 1 | CO Line Type | PABX / CO | CO |  |
|  | ATTRIBUTE | 2 | CO Line Signal Type | DTMF / Pulse | DTMF |  |
|  |  | 3 | Universal Night Answer (UNA) | Yes / No | NO |  |
|  |  | 4 | Flash Type | Ground / Loop | Loop |  |
|  |  | 5 | DISA Account | Yes/No | No |  |
|  |  | 6 | Loop Supervision | Yes/No | No |  |
|  |  | 7 | DID/DISA Voice announcement | Yes/No | No |  |
| 2 | ISDN CO LINE ATTRIBUTE | 1 | COLP | 00-10 | 00 | 00:Not Assigned 01-10:indicate an entry of Outgoing COLP table |
|  |  | 2 | DID_RN | 00-99 | 00 | 00:Do not ignore 01-99:indicate an ignored called party number (DID_RN : DID Remove number from called party information) |
|  |  | 3 | Type of Calling Number | 0-8 | 2 | 0:Unknown number 1:International type 2:National number 3:Not used 4:Subscriber number 5:Not used 6:Not used 7:Not used 8:No send COLP |
|  |  | 4 | DID Line Use | Yes/No | No |  |
| 3 | CO Line Group Assign | - |  | 1 - 4 | 1 |  |
| 4 | DISA Type | - |  | U/Day/Night/Both | U | 0/1/2/3 |
| 5 | CO Flash Timer | - |  | 000 - 300 | 050 | 10 msec base |
| 6 | CO Loop Supervision Timer | - |  | 0 - 20 | 0 | 100 msec base |
| 7 | CO Line Ring Assignment | - |  | No/Day/Night/Both | Station 1:All ring Station2-16:No ring | Flex key : Round-robin |

Table 5.2.1-2   CO LINE BASE PROGRAM  (MAIN MENU 3)

| FLEX KEY | SUB MENU | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | TEI Type | - | Auto/Fixed | Auto (3 Ports) | After you change TEI, press RESET button on MBU to operate with the new TEI type. |
| 2 | Calling Sub-Address | - | Yes/No | No | If this field is set to YES, then Aria sends the calling extension number in calling party sub address information of setup message. |
| 3 | Incoming Zero Insertion | - | Yes/No | No |  |
| 4 | Outgoing Zero Insertion | - | Yes/No | Yes |  |
| 5 | Outgoing Check Digit | - | 0 – 9 | 0 |  |
| 6 | My Area Code | - | - | Not Assigned |  |
| 7 | My Nation Code | - | - | Not Assigned |  |
| 8 | COLP TABLE | - | Entry No (01-10) | Not Assigned | Max : 10 digit |
| 9 | MSN TABLE | - | Entry No (01-24) | Not Assigned | Flex key 1: CO line no(1-6) Flex key 2: Station no(2digit or 3digit) Flex key 3: MSN no(1-8) Flex key 4: Telephone no(max:20digit) |
| 11 | DID DIGITS CONVERSION | 1. DID Received Digit from PX | 2 - 4 | 3 |  |
|  |  | 2. DID First Two Digits Conversion | DID Received Digit No | *** | 0 - 9 # : Ignore Digit * : using received digit |
|  |  | 3. DID Second Digit Conversion | 10 Digits | 1234567890 |  |
| 10 | MSN TABLE | - | Entry No (01-24) | Not Assigned | Flex key 1: CO line no(1-6) Flex key 2: Station no(2digit or 3digit) Flex key 3: MSN no(1-8) Flex key 4: Telephone no(max:20digit) |
| 13 | ISDN Supplementary service | 1. Hold/Retreive | Keypad/ Function | Keypad |  |
|  |  | 2. 3Party Conf. | Keypad/ Function | Keypad |  |
|  |  | Hold Code | 10 digits | None |  |
|  |  | Retrieve Code | 10 digits | None |  |
|  |  | 3 Party Conf Code | 10 digits | None |  |
| 14 | Loop Back |  | On/Off | Off |  |

Table 5.2.1-3   ISDN PROGRAM  (MAIN MENU 4)

| NO | SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | SYSTEM | 1 | Hold Preference | SYS/EXC | SYS |  |
|  | ATTRIBUTES | 2 | Privacy | YES/NO | YES |  |
|  |  | 3 | External Night Ring | YES/NO | NO |  |
|  |  | 4 | Multi-line Conference | YES/NO | NO |  |
|  |  | 5 | CO Line Choice | Last Choice / Round-robin | Last Choice |  |
|  |  | 6 | Override 1st CO Group | Yes/No | No |  |
| 2 | ATTENDANT ASSIGNMENT | - |  | STA No (2Digit or 3Digit) | 100 (1st Station) | ICM box cannot be assigned as attendant station. |
| 3 | SET SYSTEM DATE/TIME | - |  |  |  |  |
| 4 | LCD DISPLAY MODE | - |  | 1-4 | 3 (12-DD/MM/YY) | 1:24- DD/MM/YY 2:24- MM/DD/YY 3:12- DD/MM/YY 4:12- MM/DD/YY |
| 5 | PABX ACCESS CODE | - |  | MAX 2 Digits | Not Assigned | Max : 4 PBX Access Code |
| 6 | DID/DISA | 1 | Busy Destination | Tone/ATD | Tone |  |
|  | DESTINATION | 2 | Error Destination | Tone/ATD | Tone |  |
| 7 | DISA RETRY COUNTER |  | DISA Retry Counter | 1-9 | 3 |  |
| 8 | SMDR | 1 | SMDR Print Enable | YES/NO | NO |  |
|  | ATTRIBUTES | 2 | SMDR Recording Call Type | All Call/Long Distance | All Call |  |
|  |  | 3 | Print Incoming Call | YES/NO | NO |  |
|  |  | 4 | SMDR Print Baud Rate (RS-232C) | 1-8 | 6 | 1:300Bps 2:600Bps 3:1200Bps 4:2400Bps 5:4800Bps 6:9600Bps 7:19200Bps 8:38400Bps |
| 9 | DIAL SPEED RATIO | - |  | 60/40 or 66/33 | 66/33 (10 PPS Only) |  |
| 10 | ALARM | 1 | Alarm Enable | YES/NO | NO |  |
|  | ATTRIBUTE | 2 | Alarm Contact Type | Close/Open | Close |  |
|  |  | 3 | Alarm Signal Mode | Repeat/Once | Repeat |  |
| 11 | External Control Contact | 1 | First Contact | 1-3 (Loud Bell /Door/Ext. Page) | None |  |
|  |  | 2 | Second Contact |  | None |  |
| 12 | BGM Type | - | - | 1-3 | 1 | 1:Inetrnal 2:External 3:Both |
| 13 | MOH Type | - | - | 1-3 | 1 | 1:Inetrnal 2:External 3:DVIB MOH |
| 14 | SYSTEM GAIN CONTROL | 1 | Digital Keyset Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 2 | SLT Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 3 | CO Line Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 4 | Digital CO Line (DCO) Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 5 | DVU Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.MUSIC2 |
|  |  | 6 | DTMF Rx Gain | 00-60 |  | 1.SLT |
|  |  |  |  |  |  | 2.CO |
|  |  |  |  |  |  | 3.DCO |
| 15 | Auto Fax Transfer | - | Fax CO Line | 1-6 | None |  |
| 16 | External Page port Assignment | - | - | 102-115 | None | Only SLT port |
| 17 | DID/DISA Forward to DVIB | 1 | Busy | Yes/No | No |  |
|  |  | 2 | No Answer | Yes/No | No |  |

Table 5.2.1-4   SYSTEM BASE PROGRAM  (MAIN MENU 5)

| FLEX KEY | SUB MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Exclusive Hold Recall Timer | 000 - 300 | 060 | 1 sec base |
| 2 | System Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
| 3 | Transfer Recall Timer | 000 - 300 | 030 | 1 sec base |
| 4 | I-Hold Recall Timer | 000 - 300 | 030 | 1 sec base |
| 5 | Attendant Recall Timer | 00 - 60 | 01 | 1 min base |
| 6 | CO Ring Detect Timer | 1 - 9 | 2 | 100 msec base |
| 7 | CO Ring Stop Detect Timer | 1-15 | 6 | 1 sec base |
| 8 | Pause Timer | 1 - 9 | 1 | 1 sec base |
| 9 | CO Release Guard Timer | 01 - 15 | 01 | 1 sec base |
| 10 | CO Dial Delay Timer | 00 - 99 | 05 | 100 msec base |
| 11 | Preset Call Forward Timer | 00 - 99 | 10 | 1 sec base |
| 12 | Call Forward Busy/No Answer Timer | 00 - 99 | 10 | 1 sec base |
| 13 | DID/DISA No Answer Timer | 00 - 99 | 20 | 1 sec base |
| 14 | Unsupervised Conference Timer | 00 - 99 | 10 | 1 min base |
| 15 | Message Reminder Tone Timer | 00 - 60 | 00 | 1 min base |
| 16 | Hook Switch Flash Timer | 001 - 250 | 050 | 10 msec base |
| 17 | Door Open Timer | 05 - 99 | 20 | 100 msec base |
| 18 | Inter-Digit Timer | 01 - 20 | 05 | 1 sec base |
| 19 | Warm Line Timer | 01 – 20 | 05 | 1 sec base |
| 20 | SMDR Start Timer | 000 - 250 | 000 | 1 sec base |
| 21 | Wake-up Fail Ring Timer | 00 - 99 | 20 | 1 sec base |
| 22 | Fax Tone Detect Timer | 01-10 | 05 | 1 sec base |
| 23 | Fax CO Call Timer | 1-5 | 1 | 1 min base |
| 24 | DVIB User Record Timer | 10-255 | 20 | 1 sec base |
| 25 | DVIB Station Forward Answer Timer | 4-40 | 4 | 1 sec base |
| 26 |  |  |  |  |
| 27 | DVIB Valid User Message Timer | 0-10 | 4 | 1 sec base |

Table 5.2.1-5   SYSTEM TIMER PROGRAM  (MAIN MENU 6)

| FLEX KEY | SUB MENU | TERMINAL MENU | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Allowed Table A | Entry No(01 – 20) |  | Max : 8 digit |
| 2 | Denied Table A | Entry No(01 – 10) | Not | Max : 8 digit |
| 3 | Allowed Table B | Entry No(01 – 20) | Assigned | Max : 8 digit |
| 4 | Denied Table B | Entry No(01 – 10) |  | Max : 8 digit |
| 5 | Canned Toll Table : Allow | Entry No(01 – 10) |  | Max : 8 digit |
|  | Canned Toll Table : Deny | Entry No(01 – 10) |  | Max : 8 digit |

Table 5.2.1-6   TOLL TABLE (MAIN MENU 7)

| SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| AUTHORIZATION CODE TABLES | - | Entry No (01-30) | 5 Digit | Not Assigned |  |

Table 5.2.1-7   AUTHORIZATION CODE TABLE (MAIN MENU 8)

| FLEX KEY | SUB MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | NUMBERING PLAN TYPE | Extension / Basic | Basic |  |
| 2 | STA NUMBER DIGIT SIZE | 3 / 2 | 3 |  |
| 3 | FLEXIBLE STATION NUMBER | FLEX KEY 1-16 | 100-115 | Each flex key denotes corresponding station. |

Table 5.2.1-8   FLEXIBLE STATION NUMBERING (MAIN MENU 9)

| - FLEX KEY | - SUB MENU | - Range | - Default | - Remark |
| --- | --- | --- | --- | --- |
| - 1 | - LCR Access | - 3-way toggle - - DISABLE - - OVERRIDE - - ALWAYS | - Disable |  |
| - 2 | - Day Zone | - F1 : Zone 1. Day : 1 - 7 | - 1-7 | - Each day belongs to only one day zone. |
|  |  | - F2 : Zone 2. Day : 1 - 7 | - - |  |
|  |  | - F3 : Zone 3. Day : 1 - 7 | - - |  |
| - 3 | - Time Zone | - F1 : Zone 1. Time : 00 – 24 | - 00-24 | - Each hour belongs to only one time zone. |
|  |  | - F2 : Zone 2. Time : 00 – 24 | - - |  |
|  |  | - F3 : Zone 3. Time : 00 – 24 | - - |  |
| - 4 | - LDT - (Leading Digit Table) | - 00 – 99 - (total:100) | - None | - F1: LCR type, BOTH / INT / COL. - F2: CD 12 digits - F3: DMT index. 6 digits. Total three DMT - indices for each time zone1/2/3 -  F1 – Day zone 1 index to the DMT. -  F2 – Day zone 2 index to the DMT. -  F3 – Day zone 3 index to the DMT. - (For detail, refer to Table 6.2.33) |
| - 5 | - DMT - (Digit Modification Table) | - 00 – 99 - (total:100) | - None | - F1: A (Added digit stream). 25digits. - F2: RP (Removal Position). 2digits. - F3: RN (Removal Number of digits of - CODE in each table) 2 digits - F4: AP (Add Position). 2digits. - F5: CG (CO group). 1digit. - F6: ALT (Alternative DMT index). 2digits. - (For detail, refer to Table 6.2.34) |

Table 5.2.1-9   LCR PROGRAM  (MAIN MENU 10)

| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Hunt Group Program | 1 | Type |  | Cir/Term/Ucd/VM /Ring/None | None |  |
|  |  | 2 | Group Member |  | Station No. | None | Max 6 station |
|  |  | 3 | Attribute |  |  | None |  |
|  |  |  | Cirm/ Term | 1.No Answer Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 2.Ring Timer | 0-99 | 60 | 1 second |
|  |  |  | UCD | 1.First Announcement Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 2.Second Announcement Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 3.Announcement Repeat Timer | 0-99 | 15 | 1 second |
|  |  |  | VM | 1.Ring Timer | 0-99 | 99 | 1 second |
|  |  |  |  | 2.Put Mail Index | 1-4 | 1 |  |
|  |  |  |  | 3.Get Mail Index | 1-4 | 2 |  |
| 2 | VM Dialing Table | 1 | Put Mail |  | 1-2 | Prefix : P## Suffix : - | Flex 1 for Prefix, 2 for Suffix. Max 12 digits. |
|  |  | 2 | Get Mail |  | 1-2 | Prefix : P# Suffix : - |  |
|  |  | 3 | Busy |  | 1-2 | Prefix : P## Suffix : - |  |
|  |  | 4 | No Answer |  | 1-2 | Prefix : P## Suffix : - |  |
|  |  | 5 | Error |  | 1-2 | Prefix : P## Suffix : - |  |
|  |  | 6 | DND |  | 1-2 | Prefix : P## Suffix : - |  |
|  |  | 7 | Disconnect |  | - | ***** |  |

Table 5.2.1-10  STATION GROUP PROGRAM  (MAIN MENU 11)

| FLEX KEY | SUB-MENU | DATA | REMARK |
| --- | --- | --- | --- |
| 1 | Station Database Print | - | All Station(1-16) DB Print |
| 2 | CO Line Database Print | - | All CO(1-6) DB Print |
| 3 | ISDN Database Print | - |  |
| 4 | System Feature Database Print | - |  |
| 5 | System Timer Print | - |  |
| 6 | Toll Tables Print | - |  |
| 7 | Authorization Code Table Print | - |  |
| 8 | Flexible Station Numbering Print | - |  |
| 9 | LCR Database Print | - | Printed only when LCR feature is included. |
| 10 | Hunt/VM Group Print | - |  |
| 11 | All Database Print | - | Print Above 9 menu serially. |

Table 5.2.1-11  DATABASE PRINT  (MAIN MENU 12)

## DATABASE INITIALIZATION (MAIN-MENU1)

The System has been pre-programmed with certain features that are called default data (Table A1.6.1 - A1.6.8). These features are loaded into memory when the system is initialized. The system should always be initialized when installed or at any time the database has been corrupted. To initialize the system to default values, proceed as follows.

PROCEDURE

| DB INIT PRESS FLEX KEY (1-11) | - Press a flex key(1-11) or []/[]. -  Press one of flex key(1-11) as described at Table A2.0.1 -  [] : Change to the next main menu, STATION BASE PROGRAM. -  [] : Change to the previous main menu, PRINT DATABASE. |
| --- | --- |
| - DB INIT,STATION - PRESS [HOLD/SAVE] KEY | - Press [HOLD/SAVE] or []/[]. When pressing [HOLD/SAVE] button, the selected database is initialized with default data.  [] : Change to the next sub menu, CO LINE DATABASE INITIALI- ZATION.  [] : Change to the previous sub menu, ALL DATABASE INITIALI- ZATION. |

| MAIN MENU | FLEX KEY | SUB MENU | REMARK |
| --- | --- | --- | --- |
| DATABASE | 1 | Station Database Initialization |  |
| INITIALIZATION | 2 | CO Line Database Initialization |  |
|  | 3 | ISDN Database Initialization |  |
|  | 4 | System Feature Database Initialization |  |
|  | 5 | System Timer Database Initialization |  |
|  | 6 | Toll Table Database Initialization |  |
|  | 7 | Authorization Code Table Initialization |  |
|  | 8 | Flexible Station Number Initialization |  |
|  | 9 | Flexible Button Program Initialization |  |
|  | 10 | LCR Database Initialization | Exist only when LCR feature is included. |
|  | 11 | All Database Initialization |  |

Table 5.2.1-12  Button Configuration of Initialization (MAIN MENU 1)

## STATION BASE PROGRAM (MAIN-MENU2)

The user can enter STATION BASE PROGRAM, one of main menus, by pressing 2nd FLEX BTN or []/[] at the beginning of the ADMIN. PROGRAM.

### STATION ATTRIBUTES (SUB-MENU1)

PROCEDURE

| ADMIN PROGRAM START PRESS FLEX KEY(1-11) | - Select a main-menu 2 by pressing 2nd flex key. |
| --- | --- |
| - STA PGM - ENTER STA RANGE | - Enter 6 digits for station range (Ex:100-112). Or enter 4 digits if station number is 2digit. The second station number must be more than or equal to the first station number |
| - STA PGM,100-112 - PRESS FLEX KEY(1-8) | - Press a flex key 1. The user can press a flex key (1-8) or []/[]. |
| - STA PGM,100-112,ATTRIBUT - PRESS FLEX KEY(1-10) | - Press a flex key 1. The user can press a flex key(1-10) or []/[]. -  Press one of flex key(1-10) to enter a sub-program of STATION - ATTRIBUTES.(next LCD) -  [] : Change to the next sub-menu, ISDN STATION ATTRIBUTES. -  [] : Change to the previous main menu, PAGE ZONE. |
| - STA PGM,100-112 DND - YES | - To change the value, press the 1st flex key as Table2.1.1. The 1st flex key acts as toggle setting. |
| - STA PGM,100-112 DND - YES | - Press [HOLD/SAVE] or []/[].  [HOLD/SAVE] : Confirm and update the DND status of the station in the range, then LCD displays the current DND status.  []/[] : Change the current menu to the next/previous terminal menu.  [TRANS/PGM] : Go to the start of the Admin. program.  [REDIAL] : Go to the 1 level upper menu. |

 LCD Display of Terminal Menu in STATION ATTRIBUTES

| LCD Display |  |  | FLEX KEY |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| STA PGM,100-112 DND YES |  |  | 1. Do Not Disturb (DND) |  |  |  |
| STA PGM,100-112 SPEED YES |  |  | 2. Speed Dial Access |  |  |  |
| STA PGM,100-112 PAGE YES |  |  | 3. Page Access |  |  |  |
| STA PGM,100-112 FWD YES |  |  | 4. Call Forward Access |  |  |  |
| STA PGM,100-112 PLA YES |  |  | 5. Preferred Line Answer (PLA) |  |  |  |
| STA PGM,100-112 SPEAKER YES |  |  | 6. Speaker-Phone/Headset |  |  |  |
| STA PGM,100-112 AUTO SP YES |  |  | 7. Auto Speaker Selection |  |  |  |
| STA PGM,100-112 ICM BOX YES |  |  | 8. ICM Box Signalling |  |  |  |
| STA PGM,100-112 A HOLD NO |  |  | 9. Automatic Hold |  |  |  |
| STA PGM,100-112 SECURE NO |  |  | 10. Data Line Security |  |  |  |
| STA PGM,100-112 DVU ACC NO |  |  | 11. DVIB Access |  |  |  |
| SUB MENU | FLEX KEY | TERMINAL MENU |  | DEFAULT (LED) | REMARK |  |
| STATION ATTRIBUTE | 1 | DND |  | YES (ON) | Enables DND to be activated by the station. |  |
|  | 2 | Speed Dial Access |  | YES (ON) | Allows access to system speed dial by the station. |  |
|  | 3 | Page Access |  | YES (ON) | Allows access to paging by the station. |  |
|  | 4 | Call Forward |  | YES (ON) | Enables Call Forward to be activated by the station. |  |
|  | 5 | PLA |  | YES (ON) | The allowance to answer calls by simply lifting handset or pressing [MON] button with the answering priority. |  |
|  | 6 | Speaker Phone |  | YES (ON) | Operate with Speakerphone. |  |
|  | 7 | Auto Speaker Selection |  | YES (ON) | Allows accessing a CO line or place a DSS call by pressing appropriate {CO} or {DSS} button without lifting handset or pressing the [MON] button. |  |
|  | 8 | ICM Box Signalling |  | YES (ON) | Allows receiving ICM box signal. |  |
|  | 9 | Automatic Hold |  | Station1:YES Station2-16:NO | While on a CO line, the station user seizes another CO line by depressing the {CO} button. The first CO line goes on Hold automatically. |  |
|  | 10 | Data Line Security |  | NO (OFF) | The Allowance to protect from override and camp-on, when busy state. |  |
|  | 11 | DVIB Access |  | NO (OFF) | Enable DVIB access to each station |  |

Table 5.4.1-13 STATION ATTRIBUTES (SUB-MENU 1 of STATION BASE PROGRAM)

### STATION ISDN ATTRIBUTES (SUB-MENU 2)

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-8) | - To select STATION ISDN ATTRIBUTE, press a flex key 2. |
| --- | --- |
| - STA PGM,100-112,ISDN STA - PRESS FLEX KEY(1-6) | - To select SUB ADDR menu, press flex key 1. |
| - STA ISDN,SUB ADDR - NO | - LCD will show the current status. To program SUB ADDRESS, press the 1st flex key for toggle setting. - (YES(LED On), NO(LED Off)) |
| - STA ISDN,SUB ADDR - YES | - LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next terminal menu, LONG/SHORT.  [] : Move to the previous terminal menu, CLIP. |

| FLEX KEY | FIELD | DEFAULT | REMARK |
| --- | --- | --- | --- |
| 1 | Sub Address | NO | This field determines that ISDN station will use Sub Address or not. ON:YES, OFF:NO |
| 2 | Long/Short | SHORT | This field determines that ISDN station acts in Short passive mode or not.. ON:LONG, OFF:SHORT |
| 3 | MSN | NO | This field determines that ISDN station will use MSN or not. ON:YES, OFF:NO |
| 4 | COLR | NO | Restricts sending Connected Line Number to PX |
| 5 | COLP | NO | Allows printing Connected Line Number |
| 6 | CLIP | NO | This field determines whether the caller’s ID is printed or not in PRIB/BRIB/STIB(T-mode). |

Table 5.4.2-14 ISDN STATION ATTRIBUTES (SUB-MENU 2 of STATION BASE PROGRAM)

### STATION COS  (SUB-MENU 3)

Default gives all stations COS 1 for day and night operation. Each station must be assigned a class of service which governs that station's toll restriction for the day and night operation. (Table A2.3.1)

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-8) | - Select COS menu by pressing a flex key 3. |
| --- | --- |
| - STA PGM,100-112, COS - 11 DIAL 2 DIGITS(D+N) | - Dial 2 digit(11-77) for Day/Night COS. LCD displays the current COS. -  [] : Move to the next terminal menu, CO LINE GROUP ACCESS. -  [] : Move to the previous terminal menu, ISDN STATION ATTRIBUTE. |
| - STA PGM,100-112, COS - 22 DIAL 2 DIGITS(D+N) | - Press [HOLD/SAVE] or []/[]. [HOLD/SAVE] updates database permanently. LCD displays the current COS. |

| CLASS | REMARK |
| --- | --- |
| STA COS 1 | No restrictions are placed at the station for dialling. |
| STA COS 2 | The assignments in the Exception Table A are monitored for allow and deny numbers. |
| STA COS 3 | The assignments in the Exception Table B are monitored for allow and deny numbers. |
| STA COS 4 | The assignments in the Exception Table A & B are monitored for allow and deny numbers. |
| STA COS 5 | The leading digits can not be a Long Distance code. |
| STA COS 6 | The leading digits can not be a Long Distance code. Only 8 digits maximum can be dialled. |
| STA COS 7 | Not use a CO Line, Only allowed to make an intercom call and to receive any calls. |

Table 5.4.3-15 Station Class-Of-Service (Sub-Menu 3 of STATION BASE PROGRAM)

### CO LINE GROUP ACCESS   ( SUB-MENU 4 )

If you want to change CO line group access of some stations, do this programming. Default gives to access CO Line Group 1-4 for all stations.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-8) | - Select CO LINE GROUP ACCESS menu by pressing a flex key 4. |
| --- | --- |
| - STA,100-112,CO GROUP ACC - 12_4 FLEX KEY(1-4) | - Press flex keys(1-4) for toggle setting. LED's of flex key(1-4) displays the current status of the first station in the range. |
| - STA,100-112,CO GROUP ACC - 1234 FLEX KEY(1-4) | - Press [HOLD/SAVE] button for updating database permanently. |

| BTN | TERMINAL MENU | REMARK |
| --- | --- | --- |
| 1-4 | - | ACCESS AUTHORITY of CO GROUP 1-4 (YES:LED-ON, NO:LED-OFF) |

Table 5.4.4-16 Button Configuration For CO Group Access  (Sub-Menu 4 of STATION BASE PROGRAM)

### PRESET CALL FORWARD  ( SUB-MENU 5 )

A station can be programmed so that incoming Co lines can be forwarded to a preset station if the first station does not answer after Call Forward No Answer Timer. Default assigns no stations.

The maximum allowed number of preset call forward chain is 5.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-8) | - Select PRESET CALL FORWARD menu by pressing a flex key 5. |
| --- | --- |
| - STA,100-112, PRESET FWD - ___ DIAL STA NO | - Enter the station number want to forward. (Ex.101). LCD shows current status of the station. - To clear the Preset Call Forward station, press [SPEED] button. |
| - STA,100-112, PRESET FWD - 101 DIAL STA NO | - Press [HOLD/SAVE] button for updating database permanently. If current preset call forward chain number is 5, the user cannot save his input. |

### FLEX BUTTON ASSIGNMENT ( SUB-MENU 6 )

Flex button program is supported up to 24.

Each Flex Button of key station can be assigned as one of the following.

| DIGIT(to dial) | TYPE | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | User Button | - | User can program by button programming procedure. |
| 2 | {CO x} Button | 1 - 6 | CO number |
| 3 | {CO Grp x} | 1 - 4 | CO Group ( Not available yet) |
| 4 | {LOOP} | - |  |
| 5 | {STA xxx} | 100 - 115 | Station No.(Station number range can be changed) |
| 6 | {SPD xx} | 01 -99 | Speed Bin |

Table 5.4.6-17   Button Type For Flexible Button Assignment ( Sub-Menu 6 of STATION BASE PROGRAM )

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-8) | - Select FLEX BUTTON ASSIGNMENT menu by pressing a flex key 6. |
| --- | --- |
| - STA,100-112, BUTTON PGM - PRESS FLEX KEY(1-24) | - Press a flex key(1-24) to be programmed. |
| - STA,100-112, BUTTON PGM - 01:CO1 DIAL TYPE(1-6) | - The LCD will display the current status of the pressed flex key. - Dial a digit(1-6) , then LCD will display the changed type. |
| - STA,100-112, BUTTON PGM - 01:CO DIAL NO(1-6) | - If the pressed type in step-4 needs additional data(i.e. Digit 2,3,5,6), dial additional data. ( See Table A2.6.1) |
| - STA,100-112, BUTTON PGM - 01:CO2 DIAL TYPE(1-6) | - Press [HOLD/SAVE] button for updating database permanently. |

| BTN | 2 Button | 8 Button | 12 Button | 24 Button | 30 Button | 44 Button |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | {CO 1} | {CO 1} | {CO 1} | {CO 1} | {CO 1} | {CO 1} |
| 2 | {LOOP} | {CO 2} | {CO 2} | {CO 2} | {CO 2} | {CO 2} |
| 3 |  | {CO 3} | {CO 3} | {CO 3} | {CO 3} | {CO 3} |
| 4 |  | {LOOP} | {CO 4} | {CO 4} | {CO 4} | {CO 4} |
| 5 |  | {STA 100} | {STA 100} | {STA 100} | {STA 100} | {STA 100} |
| 6 |  | {STA 101} | {STA 101} | {STA 101} | {STA 101} | {STA 101} |
| 7 |  | {STA 102} | {STA 102} | {STA 102} | {STA 102} | {STA 102} |
| 8 |  | {STA 103} | {STA 103} | {STA 103} | {STA 103} | {STA 103} |
| 9 |  |  | {STA 104} | {STA 104} | {STA 104} | {STA 104} |
| 10 |  |  | {STA 105} | {STA 105} | {STA 105} | {STA 105} |
| 11 |  |  | {STA 106} | {STA 106} | {STA 106} | {STA 106} |
| 12 |  |  | {STA 107} | {STA 107} | {STA 107} | {STA 107} |
| 13 |  |  |  | {STA 108} | {STA 108} | {STA 108} |
| 14 |  |  |  | {STA 109} | {STA 109} | {STA 109} |
| 15 |  |  |  | {STA 110} | {STA 110} | {STA 110} |
| 16 |  |  |  | {STA 111} | {STA 111} | {STA 111} |
| 17 |  |  |  | {STA 112} | {STA 112} | {STA 112} |
| 18 |  |  |  | {STA 113} | {STA 113} | {STA 113} |
| 19 |  |  |  | {STA 114} | {STA 114} | {STA 114} |
| 20 |  |  |  | {STA 115} | {STA 115} | {STA 115} |
| 21 |  |  |  | Empty | Empty | Empty |
| 22 |  |  |  | Empty | Empty | Empty |
| 23 |  |  |  | Empty | Empty | Empty |
| 24 |  |  |  | Empty | Empty | Empty |
| 25 - 44 | - | - | - | - | Empty | Empty |

Table 5.4.6-18  Initial Button Configurations   ( Sub-Menu 6 of STATION BASE PROGRAM )

| FLEX KEY | TYPE | DATA |
| --- | --- | --- |
| 1-24 | 1 : User Button | - |
|  | 2 : CO | CO Line 1-6 |
|  | 3 : CO GROUP | CO Group 1-4 (Not available yet) |
|  | 4 : LOOP | - |
|  | 5 : STA xxx | Station No(2digits or 3digits) |
|  | 6 : SPD xx | Speed Bin 01-99 |

Table 5.4.6-19  Button Configuration For Flexible Button Assignments (Sub-Menu 6 of STATION BASE PROGRAM)

### WARM LINE SELECTION  ( SUB-MENU 7 )

Idle Line Selection for a station is which is assigned to warm line, is activated when user takes no action for Warm Line Timer after lifting handset or pressing [MON] button. Warm line is programmable at Sub-menu 7 of STATION BASE PROGRAM. All stations are not assigned any Idle Line Selection by default.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-8) | - Select WARM LINE SELECTION menu by pressing a flex key 7. |
| --- | --- |
| - STA PGM,100-112, WARM LIN - CO1 DIAL TYPE(1-4) | - LCD will show the current status. Dial type and number serially. (Refer Table A2.7.1) The changed status will be displayed on the LCD. -  Press [SPEED] button to delete Warm Line Selection. -  SLT can’t program Warm Line Selection. |
| - STA PGM,100-112, WARM LIN - COGRP3 DIAL TYPE(1-4) | - Press [HOLD/SAVE] button for updating database permanently. |

| DIGIT | ITEM | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | Flex Button | 01-44 | To activate a feature on a flex button as if pressed. |
| 2 | CO Line | 1-6 | To seize a CO Line. |
| 3 | CO Group | 1-4 | To seize a CO Line Group. |
| 4 | Station | STA NUM | To call an another station. |

Table 5.4.7-20 Button Configuration for Warm Line Selection ( Sub-menu 7 of STATION BASE PROGRAM )

### PAGE ZONE  ( SUB-MENU 8 )

Each station is assigned to paging zones. A station can be in any or no zone at all. International all call is defined as the sum of all zones. If station is not in any internal zone, it will not receive any page announcement. In Aria16, system supports 2 internal paging zones. Default assigns all stations to internal page zone 1.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select PAGE ZONE menu by pressing a flex key 8. |
| --- | --- |
| - STA PGM,100-112,PAGE ZON - 1_ PRESS FLEX KEY(1-2) | - LCD will show the current status. Press flex key(1-2) to change page zone. - The flex key acts in toggle. |
| - STA PGM,100-112,PAGE ZON - 12 PRESS FLEX KEY(1-2) | - Press [HOLD/SAVE] button for updating database permanently. |

### STATION LANGUAGE CODE  ( SUB-MENU 9 )

Each station can change the language of its LCD display between eight (8) different languages..

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select LANGUAGE CODE menu by pressing a flex key 9. |
| --- | --- |
| - STA PGM, LANGUAGE CODE - 1: ENGLISH DIAL DIGIT:1-8 | - LCD will show the language - Dial digit for relevant language |
| - STA PGM, LANGUAGE CODE - 1: ENGLISH DIAL DIGIT:1-8 | - Press [HOLD/SAVE] button for updating database permanently. |

| Digit | Language |
| --- | --- |
| 1 | English |
| 2 | Swedish |
| 3 | Norwegian |
| 4 | Italian |
| 5 | Dutch |
| 6 | Finnish |
| 7 | Spanish |
| 8 | Danish |

## CO LINE PROGRAMMING ( MAIN-MENU 3 )

When programming, LCD and LED’s indicate current programmed data and programming status. If the programmer enters correct data, then LCD and LED's show the entered data and the data is stored in the temporary buffer area. To save data permanently, press the [HOLD/SAVE] button, then all data in the temporary buffer (same as LCD and LED’s show their status) are stored into system memory.

### CO LINE ATTRIBUTE ( SUB-MENU 1 )

PROCEDURE

| CO PGM ENTER CO RANGE (1-6) | (1) Enter CO line range (Ex.1-6) |
| --- | --- |
| CO PGM,1-6 PRESS FLEX KEY (1-7) | (2) Press a flex button 1 to enter CO LINE ATTRIBUTE menu.  [] : Move to the next main-menu , ISDN PROGRAM. [] : Move to the previous main-menu, STATION BASE PROGRAM. |
| CO PGM,1-4, ATTRIBUTE PRESS FLEX KEY (1-6) | (3) Press a flex button 1 to enter LINE TYPE of CO LINE ATTRIBUTE menu. |
| CO ATTR,1-4 LINE TYPE CO | (4) To change CO LINE TYPE, press flex key 1 for toggle setting. LCD displays the current status. |
| CO ATTR,1-4 LINE TYPE PBX | (5) Press [HOLD/SAVE] button for updating database permanently. |

 LCD Display of Terminal Menu in CO LINE ATTRIBUTE

| LCD Display | FLEX KEY |
| --- | --- |
| CO ATTR,1-6 LINE TYPE CO | 1. CO Line Type |
| CO ATTR,1-6 SIGNAL TYPE DTMF | 2. Signal Type |
| CO ATTR,1-6 UNA NO | 3. Universal Night Answer(UNA) |
| CO ATTR,1-6 FLASH TYPE LOOP | 4. CO Flash Type |
| CO ATTR,1-6 DISA ACCOUNT NO | 5. DISA Account |
| CO ATTR,1-6 LOOP SUPERVS NO | 6. Loop Supervision |
| CO ATTR,1-6 LOOP SUPERVS NO | 7. DID/DISA Announcement |

| FLEX KEY | TERMINAL MENU | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CO Line Type | PBX/CO | CO(OFF) | When marked PBX, a 1 or 2 digit dial code may be entered after which toll restriction is applied. |
| 2 | CO Dial Type | DTMF/Pulse | DTMF(ON) | Each CO line can be programmed to be either DTMF(tone) or dial pulse. |
| 3 | UNA | YES/NO | NO(OFF) | The allowance of Universal Night Answer service. |
| 4 | CO Flash Type | Ground/ Loop | LOOP(OFF) | Flash types of CO Line are Loop Flash (timed-break) and Ground Flash. |
| 5 | DISA Account | YES/NO | NO(OFF) | When accessed another CO line in the system by DISA line, you should enter authorization code if this flag is set. |
| 6 | Loop Supervision | YES/NO | NO(OFF) |  |
| 7 | DID/DISA Annc. | YES/NO | NO |  |

Table 5.5.1-21  Button Configuration for CO LINE Attribute ( Sub-Menu 1 of CO LINE BASE PROGRAM )

### ISDN CO LINE ATTRIBUTE ( SUB-MENU 2 )

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | (1) Enter CO line Range (ex. 1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY (1-7) | (2) Press flex button 2, to select CO LINE ISDN ATTRIBUTE. |
| CO PGM,1-4,ISDN ATTRIBUT PRESS FLEX KEY(1-4) | (3) Press flex button 1, to select ISDN COLP menu of ISDN ATTRIBUTE. |
| CO PGM,ISDN ATTR,COLP 0 DIAL COLP NO(0-10) | (4) LCD will show the current status. Dial COLP number, then entered data will be displayed on LCD. |
| CO PGM,ISDN ATTR,COLP 3 DIAL COLP NO(0-10) | (5) Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | COLP 1) | 00-10 | 00 | 00 : Not assign. 01-10 : Indicate a entry of Outgoing COLP table (ADM 4-8). |
| 2 | DID_RN | 00-99 | 00 | 00 : Do not ignore, 01-99 : Indicate a ignored called party number. (DID_RN: DID Remove number from called party information) |
| 3 | TYPE OF CALLING NUMBER | 0-8 | 2 | 0 : Unknown number 1 : International type 2 : National number 3 : Not used 4 : Subscriber number type 5 : Not used 6 : Not used 7 : Not used 8 : No send COLP |
| 4 | DID Use | YES/NO | NO |  |

Table 5.5.2-22  ISDN CO Line Attribute ( Sub-menu 2 of CO LINE BASE PROGRAM )

1) COLP : COnnected Line identification Presentation

### CO Line Group Assignment ( SUB-MENU 3 )

CO groups should be assigned according to CO Type, etc. Default gives CO Group 1 for all stations.

There can be 4 CO groups.

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | (1) Enter CO line range (Ex.1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY(1-7) | (2) Press flex button 3 to enter CO LINE GROUP ASSIGNMENT menu of CO LINE BASE PROGRAM. |
| CO PGM,1-4, GROUP ASSIGN GRP1 DIAL GRP NO(1-4) | (3) To program, dial a digit appropriating to CO group number. |
| CO PGM,1-4, GROUP ASSIGN GRP2 DIAL GRP NO(1-4) | (4) Press [HOLD/SAVE] button for updating database permanently. |

### DISA TYPE ( SUB-MENU 4 )

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | (1) Enter CO line range (Ex.1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY(1-7) | (2) Press a flex button 4 to enter DISA TYPE menu of CO LINE BASE PROGRAM. |
| CO PGM,1-4, DISA TYPE NO DIAL TYPE(0-3) | (3) To program, Dial a digit (0-3).  0: No DISA, 1: Day DISA 2: Night DISA, 3: Day/Night DISA |
| CO PGM,1-4, DISA TYPE DAY DIAL TYPE(0-3) | (4) Press [HOLD/SAVE] button for updating database permanently |

| SUB MENU | DEFAULT | REMARK |
| --- | --- | --- |
| DISA TYPE | NO DISA | Each CO line in the system can be programmed as DISA (Direct Inward System Access) line and DISA types are like this. - 0(No DISA:U) / 1(Day Only:D) / 2(Night Only:N) / 3(Day-Night Both:B) |

Table 5.5.4-23   CO DISA TYPE ( Sub-Menu 4 of CO LINE BASE PROGRAM )

### CO FLASH TIMER ( SUB-MENU 5 )

User can initiate an open loop or ground flash across a CO line for receiving CO dial tone or transferring a PABX. In this case, flash maintains for CO Flash Timer. Default of CO Flash Timer is 50.

PROCEDURE

| CO PGM, ENTER CO RANGE(1-6) | (1) Enter CO line range (Ex.1-4) |
| --- | --- |
| CO PGM, 1-4 PRESS FLEX KEY(1-7) | (2) Press flex button 5 to enter CO FLASH TIMER menu of CO LINE BASE PROGRAM. LCD shows the current flash timer. |
| CO PGM,1-4, FLASH TIME 010*10ms DIAL TIME:0-300 | (3) Enter CO Flash Timer value (000-300), then entered data will be displayed on LCD. |
| CO PGM,1-4, FLASH TIME 300*10ms DIAL TIME:0-300 | (4) Press [HOLD/SAVE] button for updating database permanently. |

### CO LOOP SUPERVISION TIMER ( SUB-MENU 6 )

The system can recognize the status of a CO line (normally open or closed) by loop supervision timer.

Default of CO Loop Supervision Timer is 0.

PROCEDURE

| CO PGM, ENTER CO RANGE(1-6) | (1) Enter CO line range (Ex.1-4) |
| --- | --- |
| CO PGM, 1-4 PRESS FLEX KEY(1-7) | (2) Press flex button 6 to enter CO LOOP SUPERVISION TIMER menu. LCD shows the current timer value. |
| CO PGM,1-4, LOOP SUPR TMR 00*100ms DIAL TIME(0-20) | (3) Enter Loop Supervision Timer value (00-20), then entered data will be displayed on LCD. |
| CO PGM,1-4, LOOP SUPR TMR 20*100ms DIAL TIME(0-20) | (4) Press [HOLD/SAVE] button for updating database permanently. |

### CO RING ASSIGNMENT ( SUB-MENU 7 )

The system can assign the status of CO rings.

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | (1) Enter CO line range (Ex.1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY (1-7) | (2) Press flex button 7 to enter CO RING ASSIGNMENT menu of CO LINE BASE PROGRAM. |
| CO PGM,1-4, RING ASSIGN 101:NO FLEX KEY(1-20) | (3) Each flex button(1-16) means each station in order. If you want to know the current CO ring assignment of any station, press the appropriate flex button. Then LCD will display current station number and ring assignment of station. If you want to change the CO ring assignment, press the flex button again. The flex button acts in 4-way toggle. (No ring, Day ring, Night ring, All ring) |
| CO PGM,1-4, RING ASSIGN 101:DAY FLEX KEY(1-20) | (4) Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- |
| 1 - 16 | Station1 – Station16 | Station 1: ALL RING | 0 : NO RING |
|  |  | Station 2 – 16 : NO RING | 1 : DAY RING |
|  |  |  | 2 : NIGHT RING |
|  |  |  | 3 : ALL RING |

Table 5.5.7-24  CO LINE RING ASSIGNMENT ( Sub-Menu 7 of CO LINE BASE PROGRAM )

## ISDN PROGRAM ( MAIN-MENU 4 )

| FLEX KEY | SUB MENU | TERMINAL MENU |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TEI Type | - |  | AUTO/FIXED | AUTO | After you change TEI, press RESET button on MBU to operate with the new TEI type. |
| 2 | CALLING SUB- ADDRESS | - |  | YES/NO | NO | If this field is set ON, then Aria send the calling extension number in calling party sub address information of setup message. |
| 3 | INCOMING ZERO INSERTION | - |  | YES/NO | NO |  |
| 4 | OUTGOING ZERO INSERTION | - |  | YES/NO | YES |  |
| 5 | OUTGOING CHECK DIGIT | - |  | 0 – 9 | 0 |  |
| 6 | MY AREA CODE | - |  | MAX 5 Digits | Not Assigned | To delete MY AREA CODE, press [SPEED] key. |
| 7 | MY NATION CODE | - |  | MAX 5 Digits | Not Assigned | To delete MY NATION CODE, press [SPEED] key. |
| 8 | INTERNATIONAL CODE | - |  | MAX 4 Digits | Not Assigned |  |
| 9 | COLP TABLE | - |  | 01 – 10 (Entry) | Not Assigned | Max : 10 Digits |
| 10 | MSN TABLE | - |  | 01 – 24 (Entry) | Not Assigned | Flex key 1:CO line no(1-6) Flex key 2:Station no(2digit or 3digit) Flex key 3:MSN no(1-8) Flex key 4:Telephone no(max:20digit) |
| 11 | DID DIGIT CONVERSION | 1. DID Received Digit from PX |  | 2 - 4 | 3 |  |
|  |  | 2. DID First Two Digits Conversion |  | DID Received Digit No | *** | 0 - 9 # : Ignore Digit * : using received digit |
|  |  | 3. DID Second Digit Conversion |  | 10 Digits | 1234567890 |  |
| 12 | TRANSFER CODE | - |  | MAX 2 Digits | *# |  |
| 13 | ISDN Supplementary service | 1. Service Type | 1. Hold/ Retrieve | Keypad/ Function | Keypad |  |
|  |  |  | 2. 3 Party Conf | Keypad/ Function | Keypad |  |
|  |  | 2. Hold Code |  | 10 digits | None |  |
|  |  | 3. Retrieve Code |  | 10 digits | None |  |
|  |  | 4. 3 Party Conf Code |  | 10 digits | None |  |
| 14 | LOOP BACK | - |  | ON/OFF | OFF | Not Used |

Table 5.5.7-25  ISDN Program ( Main Menu 4 )

### TEI TYPE  ( SUB-MENU 1 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key 1 to select TEI Type menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, TEI TYPE AAA PRESS FLEX KEY(1-3) | (2) LCD will show the current status. To program TEI Type, press the flex key 1-3 for toggle setting. Each flex key means each ISDN port. - FIXED (LED On), AUTO (LED Off) |
| ISDN PGM, TEI TYPE AFA PRESS FLEX KEY(1-3) | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , CALLING SUB-ADDRESS. [] : Move to the previous sub-menu, DID DIGIT CONVERSION. |

NOTE: After you change TEI type, press RESET button on MBU to operate with the new TEI type.

Note:

AAA: = 1st A = MBU ISDN BRI port, 2nd A = 1st STIB/STIB2 ISDN port, 3rd A = 2nd STIB2 ISDN port.

### CALLING SUB-ADDRESS (SUB-MENU 2)

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, CALL SUB ADDR NO | (2) LCD will show the current status. To program Calling SUB Address, press the flex key 1 for toggle setting. - YES(LED On), NO(LED Off) |
| ISDN PGM, CALL SUB ADDR NO | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , INCOMING ZERO INSERTION [] : Move to the previous sub-menu, TEI TYPE. |

### INCOMING ZERO INSERTION  ( SUB-MENU 3 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, INCOME ZERO INS NO | (2) LCD will show the current status. To program, press the flex key 1 for toggle setting. - YES(LED On), NO(LED Off) |
| ISDN PGM, INCOME ZERO INS NO | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , OUTGOING ZERO INSERTION [] : Move to the previous sub-menu, CALL SUB ADDR. |

### OUTGOING ZERO INSERTION  ( SUB-MENU 4 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, OUTGO ZERO INS YES | (2) LCD will show the current status. To program, press the flex key 1 for toggle setting. - YES(LED On), NO(LED Off) |
| ISDN PGM, OUTGO ZERO INS YES | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , OUTGOING DIGIT CHECK [] : Move to the previous sub-menu, INCOMING ZERO INSERTION. |

### OUTGOING DIGIT CHECK  ( SUB-MENU 5 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, OUTGO DIGIT CHK 0 DIAL DIGIT (0-9) | (2) LCD will show the current status. To program, dial number of digits for outgoing digit check. - Dial digits 0-9 (1 digit) |
| ISDN PGM, OUTGO DIGIT CHK 0 DIAL DIGIT (0-9) | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , MY AREA CODE [] : Move to the previous sub-menu, OUTGOING ZERO INSERTION. |

### MY AREA CODE  ( SUB-MENU 6 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, MY AREA CODE _ _ _ _ _ DIAL DIGIT (MAX:5) | (2) LCD will show the current status. To program, dial digits for my area code . - Dial digits 0-9 (5 digit) |
| ISDN PGM, MY AREA CODE _ _ _ _ _ DIAL DIGIT (MAX:5) | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , MY NATION CODE [] : Move to the previous sub-menu, OUTGOING DIGIT CHECK. |

### MY NATION CODE ( SUB-MENU 7 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, MY NATION CODE _ _ _ _ _ DIAL DIGIT (MAX:5) | (2) LCD will show the current status. To program, dial digits for my nation code. - Dial digits 0-9 (5 digit) |
| ISDN PGM, MY AREA CODE _ _ _ _ _ DIAL DIGIT (MAX:5) | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , INTERNATIONAL CODE. [] : Move to the previous sub-menu, MY AREA CODE.. |

### INTERNATIONAL CODE ( SUB-MENU 8 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-11) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, INTERNATIONAL CODE _ _ _ _ _ DIAL DIGIT (MAX:4) | (2) LCD will show the current status. To program, dial digits for INTERNATIONAL code. - Dial digits 0-9 (4 digit) |
| ISDN PGM, INTERNATIONAL CODE _ _ _ _ _ DIAL DIGIT (MAX:4) | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next sub-menu , ISDN COLP TABLE. [] : Move to the previous sub-menu, MY NATION CODE.. |

### ISDN COLP TABLE ( SUB-MENU 9 )

If COLP is not activated, the Aria will send COLP information to PX when a called party the incoming call according to the type of calling party number.

PROCEDURE

| ISDN PGM, COLP TABLE DIAL ENTRY NO(01-10) | (1) Dial entry number(01-10) to program a table. |
| --- | --- |
| ISDN PGM, COLP TABLE 01:1234E DIAL NO | (2) Dial COLP number (max:10digit).   To delete installed COLP number, press [SPEED] button without entering any data. |
| ISDN PGM, COLP TABLE 01:1234E DIAL NO | (3) Press [HOLD/SAVE] button for updating database permanently. |

### ISDN MSN TABLE ( SUB-MENU 10 )

By default, ISDN MSN Tables are not assigned at all.

PROCEDURE

| ISDN PGM, MSN TABLE DIAL ENTRY NO(01-24) | (1) Dial 2 digit entry number. |
| --- | --- |
| MSN 1:CO1 STA112 MSN0 TEL:5675543665E | (2) Press one of the flex key(1-4) to select entered data type.  Flex key 1: CO line number (1-6)  Flex key 2: Station number (2digit or 3digit)  Flex key 3: MSN no (1-8)  Flex key 4: Telephone number (max:20digit) |
| MSN TBL 1 CO1 DIAL CO NO(1-6) | (3)  If you press flex key 1 in step 2, you select CO line number. Dial CO line number (1-6). |
| MSN TBL 1 STA112 DIAL STA NO |  If you press flex key 2 in step 2, you select Station number. Dial Station number (2digit or 3digit). |
| MSN TBL 1 MSN:1 DIAL MSN NO(1-8) |  If you press flex key 3 in step 2, you select MSN number. Dial MSN number (1-8). |
| MSN TBL 1:TEL NO(20DGT) 5675543665E |  If you press flex key 4 in step 2, you select telephone number. Dial telephone number (max:20digit). |
| MSN TBL 1 CO1 DIAL CO NO(1-6) | (4) To delete installed number, press [SPEED] button without entering any data. |
| MSN 1:CO2 STA113 MSN1 TEL:12345673665E | (5) Press [HOLD/SAVE] button for updating database permanently. |

### DID DIGITS CONVERSION ( SUB-MENU 11 )

With Direct Inward Dialling (DID) a user dials through a public telephone network. To complete the connection, the DID trunk receives two, three or four digits from the central office, and routes the call to the proper corresponding station. The DID dialled digits may be modified by stripping up to two leading digits, by substitution of two leading digits (after stripping).

If station number size is changed, the 3 terminal menu setting of DID DIGITS CONVERSION is re-initialized according to the station number size automatically.

PROCEDURE

| ISDN PGM, DID DIGIT TABLE PRESS FLEX KEY (1-3) | (1) To program DID DIGITS CONVERSION TABLE, press one of the flex buttons(1-3) as. LCD shows current status of DID Digit Conversion. |
| --- | --- |
| DID TBL: RECEIVE DIGIT 3 DIAL NO (2-4) | (2) To assign Received digit number from PBX, enter digit number data, then entered data will be displayed on LCD. |
| DID TBL: CONVERT DIGITS #** DIAL DIGITS | To assign Converted DID digits, press digits(combination of 0-9, *, #), then entered digits will be displayed on LCD. The number of digits to be dialled depends on the receive digit number. |
| 1234567890 :2nd DGT CONV 1234567890 DIAL 10DIGIT | To assign DID 2nd digit conversion table, enter 10 digits (Ex : 1 2 2 4 5 6 7 8 9 0 ). |
| 1234567890 :2nd DGT CONV 1224567890 DIAL 10DIGIT | (3) Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DID Received Digit No From PX | 3 | 2 - 4 |  |
| 2 | DID Digit Conversion | *** | d, *, # | d : digit ( 0 - 9 ) # : ignore digits * : any kind of digit |
| 3 | DID 2nd Digit Conversion | (1 2 3 4 5 6 7 8 9 0) |  | 10 Digits |

Table 5.6.11-26  Button Configuration For DID Digit Conversion (Sub-Menu 10 of ISDN PROGRAM)

### TRANSFER CODE (STA) (SUB-MENU 12 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-14) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN TRANSFER CODE * # , DIAL CODE (2 DIGIT) | (2) LCD will show the current status. To program, dial digits for ISDN TRANSFER code. - Dial digits 0-9, *# (2 digit) |
| ISDN TRANSFER CODE * # , DIAL CODE (2 DIGIT) | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently. |

### ISDN SUPPLEMENTARY SERVICES (SUB-MENU 13 )

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-14) | (1) Press flex key to select menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, SUPPLEMENTARY PRESS FLEX KEY (1-4) | (2) LCD will show the current status. To program, press flex keys as in table following |
| ISDN PGM, SUPPLEMENTARY PRESS FLEX KEY (1-4) | (3) LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently. |

|  | FLEX KEY | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
|  | 1. Hold/Retreive | Keypad/ Function | Keypad |  |
|  | 2. 3Party Conf. | Keypad/ Function | Keypad |  |
|  | Hold Code | 10 digits | None |  |
|  | Retrieve Code | 10 digits | None |  |
|  | 3 Party Conf Code | 10 digits | None |  |

### ISDN B-CHANNEL LOOP BACK

NOT USED.

## SYSTEM BASE PROGRAMMING ( MAIN-MENU 5 )

To change system parameters, select the main-menu5 by pressing the 5th flex-key. When programming, LCD and LED’s indicate current programmed data and programming status. If the programmer enters correct data, then LCD and LED’s show the entered data and the data is stored in the temporary buffer area. After pressing the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD/LED’s show their status) are copied into permanent system memory.

| ADMIN PROGRAM START PRESS FLEX KEY(1-11) | - If you select SYSTEM BASE PROGRAM, following message will be - displayed on LCD. |
| --- | --- |
| SYS PGM PRESS FLEX KEY(1-14) | - Press one of the FLEX key (1-14) which you want. Each flex key number means sub menu number. - If you press [] or [] button, same level menu message will be displayed on LCD one after another. |

 LCD Display of Sub Menu in SYSTEM BASE PROGRAM

| LCD Display | FLEX KEY |
| --- | --- |
| SYS PGM, ATTRIBUTE PRESS FLEX KEY (1-6) | - System Attribute |
| - SYS PGM,ATTENDANT ASSIGN - 100 DIAL STA NO | - Attendant Assignment |
| - SYS PGM,SET DATE/TIME - 98/04/22 13:21 (10DIGIT) | - Setting System DATE / TIME |
| - SYS PGM,LCD DISPLAY MODE - 12/DDMMYY DIAL MODE(1-4) | - LCD DATA/TIME Display Mode |
| - SYS PGM,PBX CODE - PRESS FLEX KEY (1-4) | - PBX Access Code |
| - SYS PGM,DID/DISA DEST - PRESS FLEX KEY(1-2) | - DID/DISA Destination |
| - SYS PGM, DISA RETRY COUNT - 3 DIAL DIGIT(1-9) | - DISA Retry Counter |
| - SYS PGM,SMDR ATTRIBUTE - PRESS FLEXKEY(1-4) | - SMDR Attribute |
| - SYS PGM, CO SPEED RATIO - 66/33 | - Pulse Dial / Speed Ratio |
| - SYS PGM, ALARM ATTRIBUTE - PRESS FLEX KEY (1-3) | - Alarm Attribute |
| - SYS PGM, EXT CONTACT - PRESS FLEX KEY (1-3) | - External Control Contact |
| - SYS PGM, BGM TYPE - INTERNAL DIAL DIGIT(1-3) | - BGM(Background Music) Type |
| - SYS PGM, MOH TYPE - INTERNAL DIAL DIGIT(1-3) | - MOH(Music-On-Hold) Type |
| - SYS PGM,SYSTEM GAIN - PRESS RX-DEV TYPE:FLX1-6 | - System Gain |
| - SYS PGM,FAX TRANS CO LIN - 0 DIAL DIGIT (3-6) | - Auto FAX transfer |
| - SYS PGM,EXT PAGE STATION - _ _ _ DIAL STA NO | - External Page port (SLT) |
| - SYS , DID/DISA FWD TO DVU - PRESS FLEX KEY (1-2) | - DID/DISA FWD to DVU |

| NO | SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | SYSTEM | 1 | Hold Preference | SYS/EXC | SYS |  |
|  | ATTRIBUTES | 2 | Privacy | YES/NO | YES |  |
|  |  | 3 | External Night Ring | YES/NO | NO |  |
|  |  | 4 | Multi-line Conference | YES/NO | NO |  |
|  |  | 5 | CO Line Choice | Last Choice / Round-robin | Last Choice |  |
|  |  | 6 | Override 1st CO Group | Yes/No | No |  |
| 2 | ATTENDANT ASSIGNMENT | - |  | STA No (2Digit or 3Digit) | 100 (1st Station) | ICM box cannot be assigned as attendant station. |
| 3 | SET SYSTEM DATE/TIME | - |  |  |  |  |
| 4 | LCD DISPLAY MODE | - |  | 1-4 | 3 (12-DD/MM/YY) | 1:24- DD/MM/YY 2:24- MM/DD/YY 3:12- DD/MM/YY 4:12- MM/DD/YY |
| 5 | PBX ACCESS CODE | - |  | MAX 2 Digits | Not Assigned | Max : 4 PBX Access Code |
| 6 | DID/DISA | 1 | Busy Destination | Tone/ATD | Tone |  |
|  | DESTINATION | 2 | Error Destination | Tone/ATD | Tone |  |
| 7 | DISA RETRY COUNTER |  | DISA Retry Counter | 1-9 | 3 |  |
| 8 | SMDR | 1 | SMDR Print Enable | YES/NO | NO |  |
|  | ATTRIBUTES | 2 | SMDR Recording Call Type | All Call/Long Distance | All Call |  |
|  |  | 3 | Print Incoming Call | YES/NO | NO |  |
|  |  | 4 | SMDR Print Baud Rate (RS-232C) | 1-8 | 6 | 1:300Bps 2:600Bps 3:1200Bps 4:2400Bps 5:4800Bps 6:9600Bps 7:19200Bps 8:38400Bps |
| 9 | DIAL SPEED RATIO | - |  | 60/40 or 66/33 | 66/33 (10 PPS Only) |  |
| 10 | ALARM | 1 | Alarm Enable | YES/NO | NO |  |
|  | ATTRIBUTE | 2 | Alarm Contact Type | Close/Open | Close |  |
|  |  | 3 | Alarm Signal Mode | Repeat/Once | Repeat |  |
| 11 | External Control Contact | 1 | First Contact | 1-3 (Loud Bell /Door/Ext. Page) | None |  |
|  |  | 2 | Second Contact |  | None |  |
| 12 | BGM Type | - | - | 1-3 | 1 | 1:Inetrnal 2:External 3:Both |
| 13 | MOH Type | - | - | 1-3 | 1 | 1:Inetrnal 2:External 3:DVIB MOH |

| 14 | SYSTEM GAIN CONTROL | 1 | Digital Keyset Rx Gain | 00-60 |  | 1.DKTU |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 2 | SLT Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 3 | CO Line Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 4 | Digital CO Line (DCO) Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 5 | DVU Rx Gain | 00-60 |  | 1.DKTU |
|  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  | 5.MUSIC2 |
|  |  | 6 | DTMF Rx Gain | 00-60 |  | 1.SLT |
|  |  |  |  |  |  | 2.CO |
|  |  |  |  |  |  | 3.DCO |
| 15 | Auto Fax Transfer | - | Fax CO Line | 1-6 | None |  |
| 16 | External Page port Assignment | - | - | 100-115 | None | Only SLT port |
| 17 | DID/DISA Forward to DVIB | 1 | Busy | Yes/No | No |  |
|  |  | 2 | No Answer | Yes/No | No |  |

### SYSTEM ATTRIBUTE  ( SUB-MENU 1 )

PROCEDURE

| SYS PGM,ATTRIBUTE PRESS FLEX KEY (1-6) | - Press one of the flex buttons(1-6) to select a sub menu of SYSTEM ATTRIBUTE. |
| --- | --- |
| SYS ATTR, HOLD TYPE SYSTEM | (2) When you press flex button 1 in SYSTEM ATTRIBUTE menu, the current status will be displayed on the LCD. To program, use the 1st flex key for toggle setting (Refer to Table A6.1.1). The changed status will be displayed on the LCD. ( SYSTEM (LED On) , EXCLUSIVE (LED Off) ). |
|  | - If you want to go to another menu in SYSTEM ATTRIBUTE, press [] or [] Button. |
| SYS ATTR,HOLD TYPE EXCLUSIVE | - Press [HOLD/SAVE] button for updating database permanently. |

 LCD Display of Terminal Menu in SYSTEM ATTRIBUTES

| LCD Display | FLEX KEY |
| --- | --- |
| SYS ATTR, HOLD TYPE SYSTEM | - HOLD Preference |
| - SYS ATTR, AUTO PRIVACY - YES | - Auto Privacy |
| - SYS ATTR, EXT NIGHT RING - NO | - External Night Ring |
| - SYS ATTR, MULTI-LINE CONF - YES | - Multi-Line Conference |
| - SYS ATTR, CO LINE CHOICE - LAST | - CO Line Choice |
| - SYS ATTR, OVR 1ST CO GRP - NO | - Override 1st CO Group |

| FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | HOLD Preference | System/ Exclusive | SYSTEM (ON) | If system hold is preferred, depress [HOLD/SAVE] button and twice for exclusive hold. |
| 2 | Auto Privacy | YES / NO | YES(ON) | The system can be programmed to override CO line call to gain access to the conversation. If privacy is disabled, a station privileged to override in PGM 11 joins an existing call in progress. |
| 3 | External Night Ring | YES/NO | NO(OFF) | When CO lines are marked UNA, ring will be sent to LBC1 when an incoming call occurs on those lines during night service. |
| 4 | Multi-Line Conference | YES/NO | NO(OFF) | The system allows a conference with multiple CO lines. |
| 5 | CO Line Choice | Last Choice / Round-robin | Last Choice (ON) | The method of a CO line seizing on a CO line group access. |
| 6 | Override 1st Co Group | YES/NO | NO(OFF) |  |

Table 5.7.1-27   Button Configuration For System Attribute ( Sub-Menu 1 of SYSTEM BASE  PROGRAM)

### ATTENDANT STATION  ( SUB-MENU 2 )

Only one station can be assigned as an attendant. As a default, the first station(Ex:STN 100) is assigned as a attendant.

PROCEDURE

| SYS PGM,ATTENDENT ASSIGN 100 DIAL STA NO | - To assign attendant.  Enter station number, then assigned attendant station number will be displayed on LCD.  The attendant number cannot be deleted. |
| --- | --- |
| SYS PGM,ATTENDENT ASSIGN 102 DIAL STA NO | - Press [HOLD/SAVE] button for updating database permanently. |

### SET SYSTEM DATE/TIME  ( SUB-MENU 3 )

Date format is Year/Month/Date (YYMMDD), and Time format is Hour/Minute (HHMM) in military form. Whenever System Clock sets, second set to 0.

PROCEDURE

| SYS PGM, SET DATE/TIME 98/04/22 13:21 (10DIGIT) | - To set system clock, enter Date(YYMMDD) and Time(HHMM) serially. |
| --- | --- |
| - SYS PGM, SET DATE/TIME - 98/04/23 18:45 (10DIGIT) | - Press [HOLD/SAVE] button for updating database permanently. |

### LCD DISPLAY MODE ( SUB-MENU 4 )

Two LCD Date Formats are Day/Month/Year (DDMMYY) or Month/Day/Year (MMDDYY) Mode. Two LCD Time Formats are Ordinary (12 hour) / Military (24 hour) Mode. By default LCD Date Format is DDMMYY and LCD Time Format is Ordinary (12 hour).

PROCEDURE

| SYS PGM, LCD DISPLAY MODE 12,DDMMYY , DIAL MODE(1-4) | - Current LCD DISPLAY MODE is displayed on LCD. To change LCD Time Format and LCD Date Format, dial digit. To program, refer to Table A5.5.1. |
| --- | --- |
| - SYS PGM, LCD DISPLAY MODE - 12,MMDDYY , DIAL MODE(1-4) | - Press [HOLD/SAVE] button for updating database permanently. |

| NO | DISPLAY | REMARK |
| --- | --- | --- |
| 1 | 24 HOUR - DD/MM/YY |  |
| 2 | 24 HOUR - MM/DD/YY |  |
| 3 | 12 HOUR - DD/MM/YY | DEFAULT |
| 4 | 12 HOUR - MM/DD/YY |  |

Table 5.7.4-28   Button Configuration for LCD DISPLAY MODE ( Sub-Menu 4 of SYSTEM BASE PROGRAM )

### PBX ACCESS CODES ( SUB-MENU 5 )

Maximum 4 PBX Access Codes are assignable. PBX Access Code is 1 or 2 digits number. By default, PBX Access Codes are not assigned at all.

PROCEDURE

| SYS PGM,PBX CODE PRESS FLX KEY (1-4) | - Press one of the flex buttons 1-4. (Ex: Flex button 1 – 1st PBX Code :). |
| --- | --- |
| SYS PGM,PBX CODE 1 9 (9 .. .. .. ) | - Current assigned PBX code is displayed on LCD.  Dial 2 digits(or 1 digit) for PBX Access code to assign.  Press [SPEED] button to delete PBX Access Code. |
| SYS PGM,PBX CODE 1 8 (8 .. .. .. ) | - Press [HOLD/SAVE] button for updating database permanently. |

### DID/DISA DESTINATION ( SUB-MENU 6 )

A station can be arranged to forward a DID call to the attendant if the station is busy. Vacant or invalid calls are sent to the Main Attendant, or busy tone is presented by Admin. setting. By default, Tone is assigned to DID/DISA Destination.

PROCEDURE

| SYS PGM, DID/DISA DEST PRESS FLEX KEY (1-2) | - Press one of the flex buttons(1-2). To assign DID/DISA Busy destination, press flex button 1. Then selected flex button LED will be lit. |
| --- | --- |
| DID/DISA DEST:BUSY CASE TONE | - To change Busy destination, press flex button 1. Flex button 1 operates in toggle. |
| DID/DISA DEST:BUSY CASE ATTENDANT | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT | RANGE |
| --- | --- | --- | --- |
| 1 | DID/DISA Busy Destination | Tone | Tone / Attendant |
| 2 | DID/DISA Error Destination | Tone | Tone / Attendant |

Table 5.7.6-29 Button Configuration for DID/DISA Destination (Sub-Menu 6 of SYSTEM BASE PROGRAM )

### DISA RETRY COUNTER ( SUB-MENU 7 )

When the DISA user fails to call a station or access CO line, then DISA user can retry the other call within this retry counter. If DISA user cannot access appropriately within this counter, system disconnects the DISA line automatically.

The Default of DISA Retry Counter is 3.

PROCEDURE

| SYS PGM, DISA RETRY COUNT 5 DIAL DIGIT(1-9) | - Dial DISA retry counter. |
| --- | --- |
| SYS PGM, DISA RETRY COUNT 7 DIAL DIGIT(1-9) | - Press [HOLD/SAVE] button for updating database permanently. |

### SMDR ATTRIBUTE ( SUB-MENU 8 )

Station Message Detail Recording (SMDR) will provide details on both incoming and outgoing calls. An assignable data base option, All Call/Long Distance Only is selected, incoming and outgoing local and long distance calls are all provided. If Long Distance Only is selected, then only outgoing calls that meet the toll check status requirements listed below are provided.

PROCEDURE

| SYS PGM,SMDR ATTRIBUTE PRESS FLEX KEY(1-4) | - Press 1st flex button to select ENABLE PRINTING menu of SMDR ATTRIBUTE. To program SMDR ATTRIBUTE, refer to Table A5.8.1. |
| --- | --- |
| SMDR, ENABLE PRINTING YES | (2) To make a change, Press flex button 1 for toggle setting. - YES (LED On), NO (LED Off) |
| SMDR, ENABLE PRINTING NO | (3) Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT(LED) | RANGE | REMARK |
| --- | --- | --- | --- | --- |
| 1 | SMDR Print Enable | NO (OFF) | YES/NO | The system can be set to real time print either all outgoing calls or only limit set by SMDR timer in SYSTEM TIMER PROGRAM (Main-Menu 6). |
| 2 | Print Type | All Call (ON) | All Call / Long Distance | The system can be set to record either all outgoing calls or only long distance calls, exceeding time limit set by SMDR timer. The long distance calls are identified by SMDR long distance code programming. |
| 3 | Print Incoming Call | NO (OFF) | YES/NO | If this option(PIC) is set to YES, all incoming calls are printed with either all outgoing calls or long distance calls. |
| 4 | SMDR Print Baud Rate | 6 | 1 - 8 | The baud rate for the printer can be set to 300, 600, 1200, 2400, 4800, 9600, 19200, or 38400 baud. |

Table 5.7.8-30   Button Configuration for SMDR Attributes ( Sub-Menu 8 of SYSTEM BASE PROGRAM )

### DIAL SPEED RATIO   ( SUB-MENU 9 )

PROCEDURE

| SYS PGM, CO SPEED RATIO 66/33 | - To change Dial Speed Ratio, press the 1st flex button. The 1st flex key acts in toggle. |
| --- | --- |
| - SYS PGM, CO SPEED RATIO - 60/40 | - Press [HOLD/SAVE] button for updating database permanently. |

| SUB MENU | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- |
| PULSE DIAL SPEED RATIO | 60/40% , 66/33% | 66/33 % (ON) | 10PPS Only |

Table 5.7.9-31  Button Configuration for Dial Speed Ratio (Sub-Menu 9 of SYSTEM BASE PROGRAM)

### ALARM ATTRIBUTE  ( SUB-MENU 10 )

PROCEDURE

| SYS PGM,ALARM ATTRIBUTE PRESS FLEX KEY( 1-3) | - Press flex button 1 to select ALARM ATTRIBUTE ENABLE menu of ALARM ATTRIBUTE. |
| --- | --- |
| - ALARM ATTR, ENABLE - YES | - To make a change, Press flex button 1 for toggle setting. ( ALARM ENABLE (LED On), ALARM DISABLE (LED Off) )  To move to another sub menu, press [] or [] button. Note. A SLT must not be assigned to receive signals for either the alarm or Doorbell. |
| ALARM ATTR, ENABLE NO | (3) Press [HOLD/SAVE] button for updating database permanently. |

 LCD Display of Terminal Menu in ALARM ATTRIBUTE

| LCD Display | FLEX KEY |
| --- | --- |
| ALARM ATTR, ENABLE YES | - Alarm Enable |
| - ALARM ATTR,CONTACT TYPE - CLOSE | - Alarm Contact Type |
| - ALARM ATTR, SIGNAL MODE - REPEAT | - Alarm Signal Mode |

| FLEX KEY | TERMINAL MENU | DEFAULT (LED) | RANGE |
| --- | --- | --- | --- |
| 1 | Alarm Enable | No (OFF) | Yes/No |
| 2 | Alarm Contact Type | Close (ON) | Close/Open |
| 3 | Alarm Signal Mode | Repeat (ON) | Repeat/Once |

Table 5.7.10-32   Button Configuration For Alarm Attribute ( Sub-Menu 10 of SYSTEM BASE PROGRAM)

### EXTERNAL CONTROL CONTACT  ( SUB-MENU 11 )

By default, External control contacts are not assigned at all.

System supports 2 external control contacts.

PROCEDURE

| SYS PGM, EXT CONTACT PRESS FLEX KEY(1-2) | - Select EXTERNAL CONTROL CONTACT number with one of the FLEX buttons 1-2. (Ex: Flex button 1 - External Control Contact 1) |
| --- | --- |
| SYS PGM, EXT CONTACT 1 LBC101 TYPE(1-3)+STA # | - If you want to set as LBC, dial 1 and station number. Or If you want to set as DOOR BELL, dial 2 only. (Station number is needed only in LBC setting)  To delete the assignment of External Control Contact, press [SPEED] button. - If you want to set as External Page port, dial 3 and SLT station number  To delete the assignment of External Page Port, press [SPEED] button. |
| SYS PGM, EXT CONTACT 1 LBC102 TYPE(1-3)+STA # | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TYPE | DATA |
| --- | --- | --- |
| 1-2 | 1 : LBC(Loud Bell Control) | Station No |
|  | 2 : Door Bell | - |
|  | 3 : External Page | SLT port |

Table 5.7.11-33 Button Configuration For External Control Contact ( Sub-Menu 11 of SYSTEM BASE PROGRAM)

### BGM(Background Music) TYPE  ( SUB-MENU 12 )

By default, BGM type is set to Internal Music. There are 3 BGM types.

PROCEDURE

| SYS PGM, BGM TYPE INTERNAL DIAL DIGIT(1-3) | - To change the setting, dial BGM type (1-3). |
| --- | --- |
| - SYS PGM, BGM TYPE - EXTERNAL DIAL DIGIT(1-3) | - Press [HOLD/SAVE] button for updating database permanently. |

| DIGIT | BGM TYPE |
| --- | --- |
| 1 | INTERNAL MUSIC |
| 2 | EXTERNAL MUSIC |
| 3 | BOTH MUSIC |

Table 5.7.12-34   Button Configuration For BGM Type ( Sub-Menu 12 of SYSTEM BASE PROGRAM)

### MOH(Music On Hold) TYPE  ( SUB-MENU 13 )

By default, MOH type is set to Internal Music.

There are 3 MOH types.

PROCEDURE

| SYS PGM, MOH TYPE INTERNAL DIAL DIGIT(1-3) | - To change the setting, dial MOH type (1-3). |
| --- | --- |
| - SYS PGM, MOH TYPE - EXTERNAL DIAL DIGIT(1-3) | - Press [HOLD/SAVE] button for updating database permanently. |

| DIGIT | MOH TYPE |
| --- | --- |
| 1 | INTERNAL MUSIC |
| 2 | EXTERNAL MUSIC |
| 3 | DVIB MOH Channel |

Table 5.7.13-35   Button Configuration For MOH Type ( Sub-Menu 13 of SYSTEM BASE PROGRAM)

### SYSTEM GAIN CONTROL ( SUB-MENU 14 )

PROCEDURE

| SYS PGM, SYSTEM GAIN PRESS RX-DEV TYPE:FLX1-6 | - Press one of the flex button(1-6) to select Rx. Device Type. |
| --- | --- |
| - SYSTEM GAIN:DKTU - PRESS TX-DEV TYPE:FLX1-9 | - If you press flex button 1, Rx device is DKTU(Digital Keyset Telephone Unit). Then, dial 1 to select DKTU as Tx device type. |
| - SYSTEM GAIN:DKTU<-DKTU - 10 DIAL GAIN(00-60) | - Dial new gain(00-60). |
| - SYSTEM GAIN:DKTU<-DKTU - 15 DIAL GAIN(00-60) | - Press [HOLD/SAVE] button for updating database permanently. |

| NO | RX DEVICE TYPE | NO | TX DEVICE TYPE |
| --- | --- | --- | --- |
| 1 | DKTU (Digital Keyset Telephone Unit) | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 2 | SLT | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 3 | CO LINE | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 4 | DCO(Digital CO) LINE | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 5 | DVU (Digital Voice Unit) | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | MUSIC2 |
| 6 | DTMF RECEIVER | 1 | SLT |
|  |  | 2 | CO |
|  |  | 3 | DCO |

Table 5.7.14-36 Button Configuration for System Gain Control  (Sub-Menu 14 of SYSTEM BASE PROGRAM)

### AUTO FAX TRANSFER ( SUB-MENU 15 )

PROCEDURE

| - SYS PGM,FAX TRANS CO LIN - 0 DIAL DIGIT (3-6) | - Dial digit for Fax Transfer CO line (1 digit) |
| --- | --- |
| - SYS PGM,FAX TRANS CO LIN - 0 DIAL DIGIT (3-6) | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT (LED) | RANGE |
| --- | --- | --- | --- |
| - | Fax CO Line | None | 1-6 |

### EXTERNAL PAGE PORT ASSIGNMENT( SUB-MENU 16 )

PROCEDURE

| - SYS PGM,EXT PAGE STATION - _ _ _ DIAL STA NO | - Dial external page station port (SLT ) |
| --- | --- |
| - SYS PGM,EXT PAGE STATION - _ _ _ DIAL STA NO | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT (LED) | RANGE |
| --- | --- | --- | --- |
| - | - | None | 100-115 |

### DID/DISA FORWARD TO DVIB ( SUB-MENU 17 )

PROCEDURE

| - SYS , DID/DISA FWD TO DVU - PRESS FLEX KEY (1-2) | - Press one of the flex button(1-2) to select BUSY or NO ANSWER terminal menu. |
| --- | --- |
| - SYS , DID/DISA FWD TO DVU - PRESS FLEX KEY (1-2) | - Set Busy or No Answer forward to DVIB option as in table following. |
| - SYS , DID/DISA FWD TO DVU - PRESS FLEX KEY (1-2) | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT (LED) | RANGE |
| --- | --- | --- | --- |
| 1 | Busy | No | Yes/No |
| 2 | No Answer | No | Yes/No |

## SYSTEM TIMER PROGRAMMING ( MAIN-MENU 6 )

### SYSTEM TIMER 1 (SUB-MENU 1 )

PROCEDURE

| SYS TMR, PRESS FLEX KEY(1-24) | (1) Press flex button appropriating to a Timer to be selected. [] : Move to the next main-menu , TOLL TABLE PROGRAM. [] : Move to the previous main-menu, SYSTEM BASE PROGRAM. |
| --- | --- |
| SYS TMR,EXCL HOLD RECALL 000sec DIAL TIME(0-300) | (2) Enter time with 3digits (Ex.100). LCD shows the current timer. |
| SYS TMR,EXCL HOLD RECALL 300sec DIAL TIME(0-300) | (3) Press [HOLD/SAVE] to update database permanently. LCD will displays the changed time of the timer. |

| FLEX KEY | SUB-MENU | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Exc. Hold Recall Timer | 060 (sec) | 000 - 300 (3 Digits) | Determines the amount of time before a call placed on exclusive hold will recall the station placing the hold. |
| 2 | Sys Hold Recall Timer | 030 (sec) | 000 - 300 (3 Digits) | Determines the amount of time before a call placed on system hold will recall the station placing the hold. |
| 3 | Transfer Recall Timer | 030 (sec) | 000 - 300 (3 Digits) | Determines the amount of time a transferred call will ring at the station receiving the transfer and how long it will recall the station transferring the call. |
| 4 | I-Recall Timer | 030 (sec) | 000 - 300 (3 Digits) | Determines the amount of time before a call recalls the attendant. |
| 5 | Attendant Recall Timer | 01 (min) | 00 - 60 (2 Digits) | Determines the amount of time before system disconnects the call. |
| 6 | CO Ring Detect Timer | 2 (100ms) | 1 - 9 (1 Digit) | The CO Ring Detect Timer controls the time necessary to detect an outside line as ringing into the system. |
| 7 | CO Ring Stop Detect Timer | 6 (sec) | 01-15 (2 Digits) | This timer is to secure time interval between incoming ringing signals so that the active ringing can be lasted in the system until this timer is expired. |
| 8 | Pause Timer | 1 (sec) | 1 - 9 (1 Digit) | Determines the length of the pause for use with automatically sent digits or other speed dialling. |
| 9 | CO Release Guard Timer | 01 (sec) | 01 - 15 (2 Digits) | The CO Release Guard Timer controls the time necessary to guarantee idle loop state when the line is released. |
| 10 | CO Dial Delay Timer | 05 (100ms) | 00 - 99 (2 Digits) | Voice connection to the outside party will be made after this timer. This can be used to prevent illegal dialling in case of slow response from the Central Office Line or PBX. |
| 11 | Preset Forward Timer | 10 (sec) | 00 - 99 (2 Digits) | Determines the amount of time an outside line will ring before being forwarded to a predetermined station. This entry works with Preset Forward Assignments in station attributes. More than one station can be forwarded to the same destination. |
| 12 | Call Forward Busy/No Answer Timer | 10 (sec) | 00 - 99 (2 Digits) | Determines the amount of time to forward a call after a predetermined time. |
| 13 | DID/DISA No Answer Timer | 20 (sec) | 00 - 99 (2 Digits) | A DID call will be forwarded to the attendant if the station is busy or does not answer within this time. |
| 14 | Unsupervised CONF. Timer | 10 (min) | 00 - 99 (2 Digits) | Determines the amount of the time an unsupervised conference can continue after the initiator of the conference has exited the conference. |
| 15 | MSG Wait Reminder Tone Timer | 00 (min) | 00 - 60 (2 Digits) | Determines the amount of time between repeated reminder tones to a key telephone with a message waiting. |
| 16 | Hook Switch Flash Timer | 050 (10ms) | 001 - 250 (3 Digits) | This timer determines how long the user could depress the hook switch in order for it to be considered a FLASH (Timed-Break Recall). (For SLT) |
| 17 | Door Open Timer | 20 (100ms) | 05 - 99 (2 Digits) | This timer determines of the length of time that is needed to activate as a door open relay using external page relay. |
| 18 | Inter Digit Timer | 05 (sec) | 01 - 20 (2 Digits) | The time between digits cannot exceed Inter-digit timer, or error tone is received. |
| 19 | Warm Line Timer | 05 (sec) | 01-20 (2 Digits) | User takes no action after lifting handset or pressing the [MON] button and warm line timer is expired, then idle line selection for warm line is activated. |
| 20 | SMDR Timer | 000 (sec) | 000 - 250 (3 Digits) | When the SMDR is enabled, the calls to be printed is set by this SMDR timer. The calls exceeding this time limit are printed. |
| 21 | Wake-up Fail Ring Timer | 20 (sec) | 00 - 99 (2 Digits) | After a Wake-up fail ring invokes on SYSTEM ATD, the alarm ring exists during this timer. Then if this timer expires, the Alarm ring will disappear. |
| 22 | FAX Tone Detect Timer | 05 (sec) | 01-10 (2 digits) |  |
| 23 | Fax CO Call Timer | 1 (min) | 0-5 |  |
| 24 | DVIB User Record Timer | 020 (sec) | 010-255 (3 digits) |  |

Table 5.8.1-37   Button Configuration for System Timers 1 ( Sub Menu 1 )

### SYSTEM TIMER 2 (SUB-MENU 2 )

PROCEDURE

| SYS TMR 2, PRESS FLEX KEY(1-3) | (1) Press flex button appropriating to a Timer to be selected. |
| --- | --- |
| SYS TMR,EXCL HOLD RECALL 300sec DIAL TIME(0-300) | (3) Press [HOLD/SAVE] to update database permanently. LCD will displays the changed time of the timer. |

| FLEX KEY | SUB-MENU | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DVIB Station Fwd Answer Timer | 4 (sec) | 04-40 (2 digits) |  |
| 2 | Valid User Msg | 04 (sec) | 01-10 (2 digits) |  |
| 3 | DVIB Valid User Message Timer | 4 (sec) | 00-10 (2 digits) |  |

Table 5.8.2-38  Button Configuration for System Timers 2 ( Sub Menu 2 )

## TOLL TABLE ( MAIN-MENU 7 )

Main menu number of TOLL TABLES is 07. Press flex key 7 in Admin. starting mode. Toll tables are used to have access to certain toll free calls as well as being denied certain calls for the stations assigned STATION COS. Exception table A & B allow the station that is programmed in STA COS 2, 3 & 4 to have access to certain toll free calls as well as being denied certain calls.

### TOLL TABLES

The Allow/Deny Tables are organized into 2 sets of tables to support 2 different toll plans at one installed site. Each allow table may contain up to 20 number strings and each deny table 10 number strings. All bins of allow and deny tables have no entries by default. Each number string can contain up to 8 entries including any number 0-9, *, #, "Don't care", "stop".

The following rules should be remembered when setting up the Allow/Deny Tables:

(1)	If the tables have no entries, no restriction is applied.

(2)	If entries are made in the allow table and only there, then only those numbers are allowed.

(3)	If entries are made in the deny table and only there, then only those numbers are denied.

(4)	If there are entries in both tables, the allow table is searched first and if number is found, it is allowed. If not found, the deny table is searched and if number is found, it is denied. If it is not found in either table, it is allowed.

| RULE | ENTRY |  | CONDITIONS & RESULT |  |
| --- | --- | --- | --- | --- |
|  | ALLOW | DENY | ALLOW TABLE | DENY TABLE |
| 1 | NO | NO | NO Restriction | NO Restriction |
| 2 | YES | NO | Found - allowed Not found - denied |  |
| 3 | NO | YES |  | Found - denied Not found - allowed |
| 4 | YES | YES | Found - allowed Not found - check deny table | Found - denied Not Found - allowed |

Table 5.9.1-39   Allow/Deny Rules

(Reference)

| CLASS | REMARK |
| --- | --- |
| STA COS 1 | No restrictions are placed at the station for dialling. |
| STA COS 2 | The assignments in the Exception Table A are monitored for allow and deny numbers. |
| STA COS 3 | The assignments in the Exception Table B are monitored for allow and deny numbers. |
| STA COS 4 | The assignments in the Exception Table A & B are monitored for allow and deny numbers. |
| STA COS 5 | The leading digits can not be a Long Distance code. |
| STA COS 6 | The leading digits can not be a Long Distance code. Only 8 digits maximum can be dialled. |
| STA COS 7 | Not use a CO Line, Only allowed to make an intercom call and to receive any calls. |

PROCEDURE

| TOLL TABLE PRESS FLEX KEY (1-5) | (1) Press one of flex button(1-5) to program a table. (Ex: FLEX BTN 1 - Allow Table A) |
| --- | --- |
| TOLL TABLE: ALLOW A DIAL ENTRY NO (01-20) | (2) Dial entry number. (01-20 for ALLOW TABLE / 01-10 for DENY TABLE). |
| TOLL TABLE: ALLOW A 01:E DIAL NO | (3) LCD displays the selected entry number and current allowed number. Enter new data and each entry can be up to 8 digits. LCD will displays the new data.  To delete current toll number, press [SPEED] button without entering any data. |
| TOLL TABLE: ALLOW A 01:1234E DIAL NO | (4) Press [HOLD/SAVE] button for updating database permanently. |

| BTN | ITEM | DEFAULT | ENTRY | REMARK |
| --- | --- | --- | --- | --- |
| 1 | ALLOW A | - | 01 - 20 |  |
| 2 | DENY A | - | 01 - 10 |  |
| 3 | ALLOW B | - | 01 - 20 |  |
| 4 | DENY B | - | 01 - 10 |  |
| 5 | CANNED TOLL : ALLOW | - | 01 - 10 |  |
|  | : DENY | - | 01 - 10 |  |

Table 5.9.1-40    Button Configuration for Toll Tables

| Valid Data | Function | LCD Display |
| --- | --- | --- |
| 0 - 9, *, # | Numbers | as dialled |
| [DND/FWD] | Don’t Care | ‘D’ |
| [SPEED] | Delete data | - |

Table 5.9.1-41   Data Entry of Toll Table

## AUTHORIZATION CODE TABLE ( MAIN-MENU 8 )

By default, System Authorization Codes are not assigned at all.

PROCEDURE

| AUTHOR CODE TABLE DIAL ENTRY NO(01-30) | (1) Dial 2 digit entry number. |
| --- | --- |
| AUTHOR CODE TABLE 01:E DIAL DIGIT | (2) Dial 5 digit authorization code. The size of authorization code must be 5. |
| AUTHOR CODE TABLE 01:12345E DIAL DIGIT | (3) Press [HOLD/SAVE] button for updating database permanently. |

| Valid Data | Function | LCD Display |
| --- | --- | --- |
| 0 - 9, *, # | Numbers | as dialled |
| [SPEED] | Delete data | - |

Table 5.10-1   Data Entry of Authorization Code Table

## FLEXIBLE NUMBERING PLAN ( MAIN-MENU 9 )

Flexible Numbering Plan Type, Station Number Digit Size, and Flexible Station number can be programmed.

PROCEDURE

| FLEXIBLE NUMBERING PLAN PRESS FLEX KEY(1-3) | (1) Press one of flex button(1-3) to program Flexible Numbering Plan. |
| --- | --- |
| NUMBERING PLAN TYPE BASIC | (2) If you press flex key 1, you can select Numbering Plan Type. To change the value, press the 1st flex key. The 1st flex key acts as toggle setting. |
| STA NUMBER DIGIT SIZE 3 | (3) If you press flex key 2, you can select Station Number Digit Size. To change the value, press the 1st flex key. The 1st flex key acts as toggle setting. |
| FLEXIBLE STA NUMBERING FLX KEY + STA NO | (4) If you press flex key 3, you can change Station Number. Press flex button(1-16) and dial 3 digit(or 2digit) station number to program station number. |
| FLEXIBLE STA NUMBERING 01:100 FLX KEY + STA NO | (5) If you want to program other station number, press other flex button(1-16) and dial 3 digit(or 2digit) station number. |
| FLEXIBLE STA NUMBERING 02:101 FLX KEY + STA NO | (6) Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | SUB MENU | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Numbering Plan Type | Extension / Basic | Basic (OFF) | When the user changed the setting Extension to Basic, the station number is initialized according to station number digit size.  2 Digit Station number : 10 –25  3 Digit Station number : 100 – 115 There is no initialization when the setting is changed Basic to Extension. |
| 2 | Station Number Digit Size | 3 / 2 | 3 (ON) | When the station number digit size is changed, station number is initialized always. |
| 3 | Flexible Station Numbering | Basic : 10–49 (2digit) 100-499 (3digit) Extension:10-99 (2digit) 100-999 (3digit) | - | User can select any station by pressing appropriate flex key. (Ex. 2nd flex key : 2nd Station) |

Table 5.11-1   Button Configuration For Flexible Numbering Plan

## LCR PROGRAM ( MAIN-MENU 10 )

NOTE:  MEMU NEEDED

PROCEDURE

| LCR PROGRAM PRESS FLEX KEY(1-5) | (1) Press flex key 10 to enter LCR PROGRAM menu. |
| --- | --- |
| LCR ACCESS MODE DISABLE | (2) F1: Change LCR access mode Press flex key 1 to select LCD access mode. To change setting, press flex key 1 for 3-way toggle setting. Whenever pressing [F1], the LCD will be changed like “DIS”  “OVR”  “ALL” (toggle) Press [HOLD/SAVE] to save the change |
| DAY ZONE, FLEX KEY(1-3) 1:M/TU/W/TH/F/SA/SU 2: 3: | (3) F2: Set the day-of-week zone (3 zone) Press flex key 2 to select Day Zone menu. To set this day zone, press flex key appropriating to day zone number. |
| DAY ZONE:2, DIAL DAY(1-7) 1:M/TU/W/TH/F/SA/SU 2: 3: | Dial the associated day. For example, if you want to set Saturday as zone2, dial 6(Saturday). Press [HOLD/SAVE] to save the change LCD will be changed like “1:M/TU/W/TH/F/SU 2:SA 3: “ |
| TIME_ZONE OF DAY_ZONE PRESS FLEX KEY(1-3:TOD) | (4) F3: Set the time-of-day zone (3 zone) Press flex key 3 to select Time Zone menu. To enter each time zone, press flex key. (Ex. Flex key 1 : time zone of day zone1) |
| TOD1, TIME_ZON(1DGT)+4DGT 1:00-24 2:..-.. 3:..-.. | To enter value, dial time zone number & time range(5digit) - Time zone + [SPEED] : Delete specified time zone range. If time zone is 1, then time zone range is assigned as 00-24. Time zone 1 cannot be deleted completely. |
| LCR, LDT TABLE(0) DIAL ENTRY NO(00-99) | (5) F4: Program the Leading Digit Table(LDT) Press flex key 4 to select LDT Table menu. Dial entry number (00-99) to specify LDT entry number. The number in the 1st LCD line means filled table number. (LDT table is sorted automatically) |
| 01 BOTH CD:............ DMT: ...... ...... ...... | The user can press flex key(1-3) to select sub-menu. Flex key 1: LCR Type Flex key 2: CD(Code) Flex key 3: DMT Index |
| LDT, LCR TYPE BOTH |  LCR Type Press flex key 1 for 3-way toggle setting. The value will be changed like “BOTH  INT  COL*”. Press [HOLD/SAVE] to save the change. |
| LDT, DIAL CODE(MAX:12DGT) 1234567E |  CD(Code) The max digit no of code is 12. Dial the code and press [HOLD/SAVE] to save the change. |
| LDT, DAY_ZONE(1DGT)+6DGT DMT:000000 000000 000000 |  DMT Index - For Day zone 1 : Dial 1 and 6digits (meaning of 6 digits : each pair(2 digits) means the index to the DMT for the each time zone 1/2/3) - For Day zone 2 : Dial 2 and 6digits - For Day zone 3 : Dial 3 and 6digits (Day zone + [SPEED] : Delete appropriate day zone DMT index) Press [HOLD/SAVE] to save the change. *Note : 1. The modified one may be stored into a different bin, since Aria does ascending sort for the faster lookup. 2. To check another bin, then press [Up]/[Down] button. |
| LCR DMT TABLE DIAL ENTRY NO (00-99) | (6) F5: Program Digit-Modification-Table (DMT) Select the bin number (00-99) |
| 00 A:................ RP01 RN01 AP01 CG1 ALT.. | The current state of selected bin will be displayed. The user have to press flex key(1-6) to enter sub menu. - Flex key 1: Added Digit Stream - Flex key 2: Removal Position - Flex key 3: Number of Digits to be removed - Flex key 4: Add Position - Flex key 5: CO Group - Flex key 6: Alternative DMT Index |
| DMT, ADDED DIGIT (25DGT) 12345678901234567890E |  A (Added digit stream) Dial the Added Digit Stream(25digit). |
| DMT, REMOVE POSITION 01 DIAL DIGIT(01-12) |  RP (Removal Position) Dial Remove Position(01-12) |
| DMT, REMOVE NO OF DGT 01 DIAL DIGIT(00-12) |  RN (Removal number of digits of CODE in each table) Dial Remove No of Digit (00-12) |
| DMT, ADD POSITION 01 DIAL DIGIT(01-13) |  AP (Add Position) Dial Add Position(01-13) |
| DMT, CO GROUP 1 DIAL DIGIT(1-4) |  C (CO group ) Dial CO group number(1-4). TO delete CO group number, press [SPEED] Key. |
| DMT, ALT DMT INDEX 01 DIAL DIGIT(00-99) |  ALT (Alternative DMT index) Dial DMT Index(00-99) |

| FLEX KEY | SUB MENU | Default | Range | Remark |
| --- | --- | --- | --- | --- |
| 1 | LCR Access | Disable | 3-way toggle - DISABLE - OVERRIDE - ALWAYS |  DISABLE : Disable LCR  OVERRIDE: LCR whose attribute is “COL” can be accessed only through Common CO Access Code (‘9’/’0’).  ALWAYS : Dial after seizure of CO line with dialling with Common CO Access Code (‘9’/’0’) and Dial internally without seizure CO line. |
| 2 | Day Zone | Belongs to Zone 1 | - Zone : 3 - Day : 1 - 7 |  |
| 3 | Time Zone | Belongs to Zone 1 | - Zone : 3 - Time : 00 - 24 | Aria accepts it as same value for 00 and 24 and changes to “00”, if input is 24 as starting value and vice versa. *note : The time not belonging to any zone will be considered as zone 1 *note : 10 - 13 means 10:00:00 - 12:59:59 |
| 4 | LDT (Leading Digit Table) | None | 00 – 99 (total:100) | F1: LCR type 3-way toggle (BOTH  INT  COL) F2: CD 12 digits F3: DMT index. 6 digits. (meaning of 6 digits : each pair(2 digits) means the index to the DMT for the each time zone 1/2/3)  F1 – Day zone 1 index to the DMT.  F2 – Day zone 2 index to the DMT.  F3 – Day zone 3 index to the DMT. (For detail, refer to Table A10.1.2) |
| 5 | DMT (Digit Modification Table) | None | 00 – 99 (total:100) | F1: A (Added digit stream). 25digits. F2: RP (Removal Position). 2digit. F3: RN (Removal Number of digits of CODE in each table) F4: AP (Add Position). 2digits. F5: C (CO group). 1digit. F6: ALT (Alternative DMT index). 2digits. (For detail, refer to Table A10.1.3) |

Table 5.12-1   Button Configuration For LCR

| Flex Key | TERMINAL MENU | Default | Range | Remark |
| --- | --- | --- | --- | --- |
| 1 | LCR type | BOTH | 3-way toggle BOTH INT COL |  INT : look up this entry only for internal dialling  COL : look up this entry only after dialling “CO Access Code”  BOTH : look up this entry for both INT and COL |
| 2 | CD | None | 12 digits | To be compared with the dialled digits by a user. |
| 3 | DMT index | 00 | 00-99 |  |

Table 5.12-2   Button Configuration For LDT Table (Main Menu 10 – Sub Menu 4)

| Flex Key | TERMINAL MENU | Default | Range | Remark |
| --- | --- | --- | --- | --- |
| 1 | Added Digit stream | None | 25 digits |  Normal digits (0 .. 9, * , #)  Special characters [CallBk] : Pause [FLASH] : Billing code (Extension number) [SPEED] : Delete Added Digit stream |
| 2 | Removal Position | 01 | 01 – 12 | Index to CD stream in Lead table to be removed |
| 3 | Number of digits to be Removed | 00 | 00-12 | Remove digits in CD stream up to this count |
| 4 | Add Position | 01 | 01-13 | Determine the position of CD stream after removal, where the A stream will be inserted. |
| 5 | CO group | 01 | 1-4 | Determines which CO group is used for LCR dialling |
| 6 | Alternative DMT Index | NONE | 00-99 | Determine alternative DMT index when there is no idle CO line in CO group. |

Table 5.12-3   Button Configuration For DMT Table (Main Menu 10 – Sub Menu 5)

## STATION GROUP PROGRAM ( MAIN-MENU 11 )

PROCEDURE

| HUNT GRP PGM/VM TBL PRESS FLEX KEY (1-2) | - Press flex 1-2 to enter Hunt Group or Voice Mail dial table sub-menu |
| --- | --- |
| HUNT GROUP PROGRAM PRESS FLEX KEY 1-4:GRP NO | - F1: Hunt Group table. Assign as per table following |
| VOICE MAIL DIALING TBL PRESS FLEX KEY (1-7) | - F2: Voice Mail dial table. Assign as per table following |
|  | - Press [HOLD/SAVE] button for updating database permanently. |

| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Hunt Group Program | 1 | Type |  | Cir/Term/Ucd/VM /Ring/None | None |  |
|  |  | 2 | Group Member |  | Station No. | None | Max 6 station |
|  |  | 3 | Attribute |  |  | None |  |
|  |  |  | Cirm/ Term | 1.No Answer Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 2.Ring Timer | 0-99 | 60 | 1 second |
|  |  |  | UCD | 1.First Announcement Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 2.Second Announcement Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 3.Announcement Repeat Timer | 0-99 | 15 | 1 second |
|  |  |  | VM | 1.Ring Timer | 0-99 | 99 | 1 second |
|  |  |  |  | 2.Put Mail Index | 1-4 | 1 |  |
|  |  |  |  | 3.Get Mail Index | 1-4 | 2 |  |
| 2 | VM Dialing Table | 1 | Put Mail |  | 1-2 | Prefix : P# Suffix : - | Flex 1 for Prefix, 2 for Suffix. Max 12 digits. |
|  |  | 2 | Get Mail |  | 1-2 | Prefix : P## Suffix : - |  |
|  |  | 3 | Busy |  | 1-2 | Prefix : P#*3P Suffix : - |  |
|  |  | 4 | No Answer |  | 1-2 | Prefix : P#*4P Suffix : - |  |
|  |  | 5 | Error |  | 1-2 | Prefix : P#*5P Suffix : - |  |
|  |  | 6 | DND |  | 1-2 | Prefix : P#*6P Suffix : - |  |
|  |  | 7 | Disconnect |  | - | ***** |  |

## DATABASE PRINT ( MAIN-MENU 12 )

In order to obtain a hard copy printout of the database, a printer must be connected to the RS-232C connector.

If user entered Database Print menu while printing is going on, the error-tone is heard.

PROCEDURE

| PRINT DATABASE PRESS FLEX KEY (1-10) | (1) To print database, press one of flex buttons 1-10. |
| --- | --- |
| PRINT DB: ALL DB PRESS [HOLD/SAVE] KEY | (2) To print out the selected database, press [HOLD/SAVE] button. |

| FLEX KEY | SUB-MENU | DATA | REMARK |
| --- | --- | --- | --- |
| 1 | Station Database Print | - | All Station(1-16) DB Print |
| 2 | CO Line Database Print | - | All CO(1-6) DB Print |
| 3 | ISDN Database Print | - |  |
| 4 | System Feature Database Print | - |  |
| 5 | System Timer Print | - |  |
| 6 | Toll Tables Print | - |  |
| 7 | Authorization Code Table Print | - |  |
| 8 | Flexible Station Numbering Print | - |  |
| 9 | LCR Database Print | - | Printed only when LCR feature is included. |
| 10 | HUNT Grp/VM Tbl | - |  |
| 11 | All Database Print | - | Print Above 10 menu serially. |

Table 5.9.1-42  Button Configuration for DATABASE Print ( Main-Menu 11 )

( Printing Example  )

This printing example is in case of Aria-16 system.

| Station Database | STATION DATABASE ------------------------------ <STA 100> 1. STATION ATTRIBUTE DND SPDA PAGE FWD PLA SPK ASPK IBOX AHLD DSEC N Y N N Y Y Y Y Y N 2. ISDN ATTRIBUTE SUB_ADDR LONG/SHORT MSN COLR COLP CLIP N N N N Y Y 3. COS : DAY(1)/NIGHT(1) 4. CO GROUP ACCESS : 1234 5. PRESET CALL FORWARD : NOT ASSIGNED 6. FLEX BUTTON ASSIGNMENT 1 2 3 4 5 6 7 8 CO1 CO2 CO3 CO4 STA100 STA101 STA102 STA103 9 10 11 12 13 14 15 16 STA104 STA105 STA106 STA107 STA108 STA109 STA110 STA111 17 18 19 20 21 22 23 24 STA112 STA113 STA114 STA115 USER USER USER USER 7. WARM LINE : NO 8. PAGE ZONE : 1 <STA 101> : : (Printed like above for another station to range end station) |
| --- | --- |
| CO Line Database | CO LINE DATABASE ----------------------------- <CO 1> 1. CO LINE ATTRIBUTE TYPE SIGNAL UNA FLASH DISA_ACT LOOP_SUPERVISN CO DTMF N LOOP N N 2. ISDN CO LINE ATTRIBUTE COLP DID_RN CALL_NO_TYPE DID_USE 0 0 2 N 3. CO GROUP : 1 4. DISA TYPE : NO DISA 5. CO FLASH TIME : 10*10ms 6. CO LOOP SUPERVISION TIME : 5*100ms 7. CO RING ASSIGNMENT 100(A) 101(U) 102(U) 103(U) 104(U) 105(U) 106(U) 107(U) 108(U) 109(U) 110(U) 111(U) 112(U) 113(U) 114(U) 115(U) <CO 2> : : (Printed like above for another CO line) |
| ISDN Database | ISDN PROGRAM ------------------------ 1. TEI TYPE : PORT1(AUTO) PORT2(AUTO) PORT3(AUTO) 2. CALL SUB ADDR : N 3. INCOMING ZERO INSERTION : N 4. OUTGOING ZERO INSERTION : Y 5. OUTGOING CHECK DIGIT : 0 6. MY AREA CODE : NOT ASSIGNED 7. MY NATION CODE : NOT ASSIGNED 8. ISDN COLP TABLE (ENTRY:01-10, MAX:10 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 9. ISDN MSN TABLE (ENTRY:01-24, TEL_NO MAX:20 DIGIT) ENTRY CO TEL_NO STA MSN/SUB_ADDR ---------------------------------------------------------------------------------------- 1 : 1 12345678901234567890E 111 1 2 : 2 1234567890E 112 2 3 : - E ___ 0 4 : - E ___ 0 5 : - E ___ 0 6 : - E ___ 0 7 : - E ___ 0 8 : - E ___ 0 9 : - E ___ 0 10 : - E ___ 0 11 : - E ___ 0 12 : - E ___ 0 13 : - E ___ 0 14 : - E ___ 0 15 : - E ___ 0 16 : - E ___ 0 17 : - E ___ 0 18 : - E ___ 0 19 : - E ___ 0 20 : - E ___ 0 21 : - E ___ 0 22 : - E ___ 0 23 : - E ___ 0 24 : - E ___ 0 10. DID DIGIT CONVERSION RCV DIGIT NO : 3 DIGIT CONVERSION : *** 2nd DIGIT CONVERSION : 1234567890 -> 1234567890 |
| System Feature | SYSTEM FEATURE ---------------------------- 1. SYSTEM ATTRIBUTE HOLD_TYPE AUTO_PRIVACY EXT_NIGHT_RING MULTI_LINE_CONF SYSTEM Y N N CO_LINE_CHOICE OVR_1st_COGRP LAST CHOICE N 2. ATTENDANT STATION : 100 3. DATE & TIME : 98/04/21 14:35 4. LCD DISPLAY MODE : 12H DDMMYY 5. PABX ACCESS CODE : __ __ __ __ 6. DID/DISA DESTINATION BUSY CASE DESTINATION : TONE ERROR CASE DESTINATION : TONE 7. DISA RETRY COUNTER : 3 |
| System Feature (continued) | 8. SMDR ATTRIBUTE ENABLE PRINTING : N PRINT TYPE : LONG DISTANCE PRINT INCOM CALL : N BAUD RATE : 9600 BPS 9. PULSE DIAL SPEED RATIO : 66/33 10. ALARM ATTRIBUTE ENABLE CONTACT_TYPE SIGNAL_MODE N CLOSE REPEAT 11. EXTERNAL CONTROL CONTACT 1:NO 2:NO 12. BGM(Background Music) TYPE : 1(INTERNAL) 13. MOH(Music On Hold) TYPE : 1(INTERNAL) 14. SYSTEM GAIN -------------------------------------- RX DEV ¦ TX DEV ¦ GAIN -------------------------------------- DKTU ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ DTMF ¦ 8 ¦ TONE ¦ 32 ¦ DVU ¦ 29 ¦ MUSIC1 ¦ 29 ¦ MUSIC2 ¦ 29 SLT ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ DTMF ¦ 8 ¦ TONE ¦ 32 ¦ DVU ¦ 29 ¦ MUSIC1 ¦ 29 ¦ MUSIC2 ¦ 29 CO ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ DTMF ¦ 8 ¦ TONE ¦ 32 ¦ DVU ¦ 29 ¦ MUSIC1 ¦ 29 ¦ MUSIC2 ¦ 29 DVU ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ MUSIC2 ¦ 8 DTMF RX ¦ SLT ¦ 26 ¦ CO ¦ 33 ¦ DCO ¦ 33 ------------------------------------- |
| System Timer | SYSTEM TIMER ----------------------- 1. EXCLUSIVE HOLD RECALL TIMER : 60sec 2. SYSTEM HOLD RECALL TIMER : 30sec 3. TRANSFER RECALL TIMER : 30sec 4. I-RECALL TIMER : 30sec 5. ATTENDANT RECALL TIMER : 1min 6. CO RING DETECT TIMER : 2*100msec 7. CO RING STOP DETECTION TIMER : 6sec 8. PAUSE TIMER : 3sec 9. CO RELEASE GUARD TIMER : 1sec 10. CO DIAL DELAY TIMER : 5*100msec 11. PRESET CALL FORWARD TIMER : 10sec 12. CALL FORWARD BUSY/NO ANSWER TIMER : 10sec 13. DID/DISA NO ANSWER TIMER : 20sec 14. UNSUPERVISED CONF TIMER : 10min 15. MSG WAIT REMINDER TONE TIMER : 0min 16. HOOK SWITCH FLASH TIMER : 50*10msec 17. DOOR OPEN TIMER : 20*100msec 18. INTER DIGIT TIMER : 5sec 19. WARM LINE TIMER : 5sec 20. SMDR TIMER : 0sec 21. WAKE-UP FAIL RING TIMER : 20sec |
| Exception Tables | EXCEPTION TABLES ------------------------------ 1. ALLOW TABLE A (ENTRY:01-20, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 11:E 12:E 13:E 14:E 15:E 16:E 17:E 18:E 19:E 20:E 2. DENY TABLE A (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 3. ALLOW TABLE B (ENTRY:01-20, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 11:E 12:E 13:E 14:E 15:E 16:E 17:E 18:E 19:E 20:E 4. DENY TABLE B (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E CANNED TOLL TABLES ---------------------------------- 1. ALLOW TABLE (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 2. DENY TABLE (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E |
| Authorization Code Table | AUTHORIZATION CODE TABLE --------------------------------------------- (ENTRY:01-30, 5 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 11:E 12:E 13:E 14:E 15:E 16:E 17:E 18:E 19:E 20:E 21:E 22:E 23:E 24:E 25:E 26:E 27:E 28:E 29:E 30:E |
| Flexible Numbering Plan | FLEXIBLE NUMBERING PLAN ------------------------------------------ 1.NUMBERING PLAN TYPE : BASIC 2.STA NUMBER DIGIT SIZE : 3 3.FLEXIBLE STA NUMBERING 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 |
| LCR Database | LCR DATABASE ----------------------- 1. LCR ACCESS MODE : DISABLE 2. DAY ZONE : 1:M/TU/W/TH/F/SA/SU 2: 3: 3. TIME ZONE FOR DAY ZONE 1 : 1:00-24 2:00-24 3:00-24 FOR DAY ZONE 2 : 1:00-24 2:00-24 3:00-24 FOR DAY ZONE 3 : 1:00-24 2:00-24 3:00-24 4. LDT TABLE ENTRY LCR TYPE CODE DMT_INDEX_OF_DAY_ZONE ----------------------------------------------------------------------------------------------- 0 BOTH 123456789012E 111213 212223 313233 1 BOTH E . . . . . . . . . . . . . . . . . . 2 BOTH E . . . . . . . . . . . . . . . . . . : : 98 BOTH E . . . . . . . . . . . . . . . . . . 99 BOTH E . . . . . . . . . . . . . . . . . . 5. DMT TABLE ENTRY ADDED_DIGIT R_POS R_NO A_POS CGRP ALT_DMT -------------------------------------------------------------------------------------------------------- 0 E 1 0 01 1 . . 1 12345678901234567890E 1 0 01 1 . . : : 98 3467992457E 2 0 01 1 . . 99 E 1 0 01 1 . . |
| All Database | (Print above all) |