CONTENTS

7.  ADMIN PROGRAMMING	161

7.1	GENERAL DESCRIPTION	161

7.1.1	INTRODUCTION	161

7.1.2	TO ENTER THE PROGRAMMING MODE	161

7.1.3	HOW TO PROGRAM	163

7.1.4	BUTTON DEFINITION	164

7.1.5	ADMIN PROGRAMMING INDEX	165

7.1.6	DEFAULT  VALUES	168

7.2	PROGRAMMING	183

7.2.1	DATABASE INITIALIZATION (MAIN MENU 1)	183

7.2.2	STATION BASE PROGRAM (MAIN-MENU 2)	184

7.2.3	CO LINE PROGRAMMING (MAIN-MENU 3)	194

7.2.4	ISDN PROGRAM (MAIN-MENU 4)	200

7.2.5	SYSTEM BASE PROGRAMMING (MAIN-MENU 5)	206

7.2.6	SYSTEM TIMER PROGRAMMING (MAIN-MENU 6)	223

7.2.7	TOLL TABLE (MAIN-MENU 7)	226

7.2.8	AUTHORIZATION CODE TABLE (MAIN-MENU 8)	228

7.2.9	FLEXIBLE NUMBERING PLAN (MAIN-MENU 9)	228

7.2.10	LCR PROGRAM (MAIN-MENU 10)	231

7.2.11	HUNT GROUP PROGRAM / VM TABLE (MAIN-MENU 11)	235

7.2.12	DATABASE PRINT (MAIN-MENU 12)	239

# 7.  ADMIN PROGRAMMING

## 7.1	GENERAL DESCRIPTION

### 7.1.1	INTRODUCTION

The GDK-16/20 Key Telephone System can be programmed to meet each customer's individual need. All programming is done at station 100 (DKT 1) using KD-24D, KD-36D or LKD-30D digital key telephone.

Upon entering the program mode, the key telephone at station 100 cannot operate as a normal telephone but as a programming instrument with all of the buttons redefined. The keys of the dial pad are used to enter the various data fields and to enter numerical information. The 24 buttons located at the top of the phone (Flex Buttons) are used to indicate the specific data field and to enter information. Sometimes [SPEED] button is used to delete the data.

See TABLE 7.1.6.1 – 7.1.6.10 for default data. If this pre-programming suits the customer, additional Admin program is not necessary. To change admin data, the user enter the admin programming mode and select program code. During Admin programming, other keysets in the system operate normally.

When Admin programming, LCD and LED’s indicate current programmed data and status. If the programmer enters correct data, then LCD and LED’s show the entered data and the data is stored in the temporary buffer area. Real system database is not changed and has no effect on telephone operation unless permanent updating procedure is executed. Pressing [HOLD/SAVE] button, all data in the temporary buffer (same as LCD and LED’s show their status) is saved into permanent memory. Tones are provided to let the programmer know whether data entry is correct (confirmation tone) or not (error tone).

### 7.1.2	TO ENTER THE PROGRAMMING MODE

- Lift handset or press the [MON] button on the admin station, and hear ICM dial tone (optional),

- Press [TRANS/PGM] button and dial * # (confirmation tone heard),

- This procedure places the station into the Admin. Programming mode (confirmation tone heard). And LCD will display the first step of Admin. Programming (Refer to the below LCD).

ADMIN PROGRAM START PRESS FLEX KEY(1-11)

### 7.1.3	HOW TO PROGRAM

- There are 12 main-menus in the Admin. Programming mode.

- Each main program has several sub-menus and each sub-menu can have several terminal menus (sub-menus of a sub-menu), but some sub-menus don’t have any terminal-menus (It means that sub-menus are terminal-menus.). Refer to the followings;

| MAIN-MENU |  A main-menu has several sub-menus.  To select a sub-menu, press a flex-key responding to the sub-menu number. |
| --- | --- |



| SUB-MENU |  A sub-menu may have several terminal-menus, but some sub-menus may not.  To select a terminal-menu, press a flex-key responding to the terminal-menu number. |
| --- | --- |



| TERMINAL-MENU |  Enter data at the terminal-menu with dialing digits or flex-keys. |
| --- | --- |

#### 7.1.3.1	Example of Admin. Program

PROCEDURE

| ADMIN PROGRAM START PRESS FLEX KEY(1-12) | - When the user enters Admin. Programming mode, the first message will be displayed on the LCD. Press a flex-key to select a main-menu among 12 main-menus. |
| --- | --- |
| STA PGM ENTER STA RANGE | - Select the 2nd main-menu, “STATION BASE PROGRAM”, by pressing the 2nd flex-key in step 1. - Dial station range. (The second station number must be more than or equal to the first station number) |
| STA PGM,100-100 PRESS FLEX KEY(1-9) | - And then, press a flex key (1-9) or /. -  Flex key (1-9): Select a sub-menu of STA BASE PROGRAM. -  : Move to the next main-menu, CO LINE BASE PROGRAM. - : Move to the previous main-menu, INITIALIZATION. |
| STA PGM,100-115,ATTRIBUT PRESS FLEX KEY(1-13) | - Pressing a flex key 1, the LCD displays as the left side. |
| STA PGM,100-115, DND YES | - Pressing a flex key 1, the LCD displays as the left side. And then press a flex key 1 to change the current value. Press [HOLD/SAVE] button to update the database permanently. - /: Go to the other same level menu. - [TRANS/PGM]: Go to the Admin. starting menu. - [REDIAL]: Go to the 1 level upper menu. |

### 7.1.4	BUTTON DEFINITION

Buttons are used for the following purposes in the Admin. Programming mode.

- [HOLD/SAVE]: Update the system database permanently.

- [TRANS/PGM]: Move to the start of admin program menu.

- [REDIAL]: Move to the upper level menu.

- (SUB-MENU  MAIN-MENU or TERMINAL-MENU  SUB-MENU)

- : Move to the next menu in the same level. (Round-robin)

- (Ex: DND (1st terminal-menu of STATION ATTRIBUTE)  SPEED DIAL ACCESS (2nd terminal- menu of STATION ATTRIBUTE)

- : Move to the previous menu in the same level. (Round-robin)

- (Ex 1: INITIALIZATION (1st main-menu)  PRINT DATABSE (last main-menu)

- Ex 2: STATION CLASS OF SERVICE (3rd sub-menu of 2nd main-menu) 

- ISDN STATION ATTRIBUTE (2nd sub-menu of 2nd main-menu))

- [SPEED]: Clear database of the current terminal menu.

- FLEX-KEY:  1. Used for selecting a menu (main/sub/terminal) responding to the number.

- 2. Also used as toggle key for entering data, if entered value has only 2 cases number  (i.e. Yes/No, Enable/Disable)

- DIGIT: Used for entering data by dialing.

### 7.1.5	ADMIN PROGRAMMING INDEX

| FLEX KEY | MAIN MENU | FLEX KEY | SUB MENU |
| --- | --- | --- | --- |
| 1 | Initialization | 1 | Station Database Initialization |
|  | (DB Init) | 2 | CO Line Database Initialization |
|  |  | 3 | Database Initialization (ISDN Table) |
|  |  | 4 | System Feature Database Initialization |
|  |  | 5 | Database Initialization (System Timer) |
|  |  | 6 | Database Initialization (Toll Table) |
|  |  | 7 | Authorization Code Table Database Initialization |
|  |  | 8 | Flexible Station Number Initialization |
|  |  | 9 | Flexible Button Program Initialization |
|  |  | 10 | LCR Database Initialization (LCR package only-MENU) |
|  |  | 11 | Hunt Group/Voice Mail Table initialization |
|  |  | 12 | All Database Initialization |
| 2 | Station Base Program | 1 | Station Attribute |
|  | (Sta pgm) | 2 | ISDN Station Attribute |
|  |  | 3 | Station Class-Of-Service |
|  |  | 4 | CO Line Group Access |
|  |  | 5 | Preset Call Forward |
|  |  | 6 | Flex Buttons Assignment |
|  |  | 7 | Warm Line Selection |
|  |  | 8 | Page Zone |
|  |  | 9 | Language Code |
| 3 | CO Line Base Program | 1 | CO Line Attribute |
|  | (CO PGM) | 2 | ISDN CO Line Attribute |
|  |  | 3 | Group Assignment |
|  |  | 4 | DISA Type |
|  |  | 5 | CO Flash Timer |
|  |  | 6 | CO Loop Supervision Timer |
|  |  | 7 | CO Line Ring Assignment |
| 4 | ISDN Program | 1 | TEI Type |
|  | (ISDN PGM) | 2 | Calling Sub-Address |
|  |  | 3 | Incoming Zero Insertion |
|  |  | 4 | Outgoing Zero Insertion |
|  |  | 5 | Outgoing Check Digit |
|  |  | 6 | My Area Code |
|  |  | 7 | My Nation Code |
|  |  | 8 | International Code |
|  |  | 9 | COLP Table (Entry no:01-10) |
|  |  | 10 | MSN Table (Entry no:01-24) |
|  |  | 11 | DID Digit Conversion Table |
|  |  | 12 | ISDN Transfer Code |
|  |  | 13 | ISDN Supplementary Service |
|  |  | 14 | Loopback |

| FLEX KEY | MAIN MENU | FLEX KEY |  | SUB MENU |
| --- | --- | --- | --- | --- |
| 5 | System Base Program | 1 |  | System Attribute |
|  | (SYS PGM) | 2 |  | Attendant Assignment |
|  |  | 3 |  | Setting System Date/Time |
|  |  | 4 |  | LCD Date/Time Display Mode |
|  |  | 5 |  | PBX Access Code |
|  |  | 6 |  | DID/DISA Destination |
|  |  | 7 |  | DISA Retry Counter |
|  |  | 8 |  | SMDR Attributes |
|  |  | 9 |  | Pulse Dial / Speed Ratio |
|  |  | 10 |  | Alarm Attributes |
|  |  | 11 |  | External Control Contact |
|  |  | 12 |  | BGM(Background Music) Type |
|  |  | 13 |  | MOH(Music On Hold) Type |
|  |  | 14 |  | System Gain Control |
|  |  | 15 |  | Fax Transfer CO Line |
|  |  | 16 |  | External Page Station |
|  |  | 17 |  | DID/DISA Forward to DVU |
|  |  | 18 |  | CCR Table |
|  |  | 19 |  | Admin Password |
|  |  | 20 |  | DVU Setting |
|  |  | 21 |  | Weekly Time Table |
|  |  | 22 |  | ACNR Tone Candence |
| 6 | System Timers | Timer – 1 | 1 | Exclusive Hold Recall Timer |
|  | (SYS TMR) |  | 2 | System Hold Recall Timer |
|  |  |  | 3 | Transfer Hold Recall Timer |
|  |  |  | 4 | I-Recall Timer |
|  |  |  | 5 | Attendant Recall Timer |
|  |  |  | 6 | CO Ring Detect Timer |
|  |  |  | 7 | CO Ring Stop Detect Timer |
|  |  |  | 8 | Pause Timer |
|  |  |  | 9 | CO Release Guard Timer |
|  |  |  | 10 | CO Dial Delay Timer |
|  |  |  | 11 | Preset Forward Timer |
|  |  |  | 12 | Call Forward Busy/No Answer Timer |
|  |  |  | 13 | DID/DISA No Answer Timer |
|  |  |  | 14 | Unsupervised Conference Timer |
|  |  |  | 15 | MSG Wait Reminder Tone Timer |
|  |  |  | 16 | Hook Switch Flash Timer |
|  |  |  | 17 | Door Open Timer(Enhanced) |
|  |  |  | 18 | Inter Digit Timer |
|  |  |  | 19 | Warm Line Timer |
|  |  |  | 20 | SMDR Timer |
|  |  |  | 21 | Wake-up Fail Ring Timer |
|  |  |  | 22 | Fax Tone Detect Timer |
|  |  |  | 23 | Fax CO Call Timer |
|  |  |  | 24 | DVU User Record Timer |
| 6 | System Timers | Timer - 2 | 1 | DVU Forward Answer Timer |
|  | (SYS TMR) |  | 2 | Valid User Message Timer |
|  |  |  | 3 | Hunt Disconnect Timer |
|  |  |  | 4 | Call Park Recall Timer |
|  |  |  | 5 | Normal Ring To DVIB Timer |
|  |  |  | 6 | CCR Inter Digit Timer |
|  |  |  | 7 | DISA Delay Timer |
|  |  |  | 8 | - ACNR Pause Timer |
|  |  |  | 9 | - ACNR Delay Timer |
|  |  |  | 10 | - ACNR Tone Detect Timer |
|  |  |  | 11 | - ACNR No Answer Timer |
|  |  |  | 12 | - ACNR Retry Counter |
|  |  |  | 13 | - ACNR Retry No Tone |
| 7 | Toll Table | 1 |  | Toll Exception Table – Allow A (Entry no:01-20) |
|  |  | 2 |  | Toll Exception Table – Deny A (Entry no:01-10) |
|  |  | 3 |  | Toll Exception Table – Allow B (Entry no:01-20) |
|  |  | 4 |  | Toll Exception Table – Deny B (Entry no:01-10) |
|  |  | 5 |  | 1. Canned Toll Table –Allow (Entry no:01-10) |
|  |  |  |  | 2. Canned Toll Table –Deny (Entry no:01-10) |
| 8 | Authorization Code Table (Author Code Table) | - |  | Authorization Code Table (Entry no:01-30) |
| 9 | Flexible Numbering Plan | 1 |  | Numbering Plan Type(Extension / Basic) |
|  |  | 2 |  | Station Number Digit Size (3 / 2) |
|  |  | 3 |  | Flexible Station Number Assignment |
| 10 | LCR Program - MEMU needed | 1 2 3 4 5 |  | LCR Access Mode Day Zone Time Zone of Day Zone LDT Table DMT Table |
| 11 | Hunt Group/Voice Mail Table | Hunt Group | 1 | Group Type |
|  |  |  | 2 | Group Member |
|  |  |  | 3 | Group Attribute |
|  |  | VM Table | 1 | Put Mail |
|  |  |  | 2 | Get Mail |
|  |  |  | 3 | Busy Table |
|  |  |  | 4 | No Answer Table |
|  |  |  | 5 | Error Table |
|  |  |  | 6 | DND Table |
|  |  |  | 7 | Disconnect Table |
| 12 | Print Database | 1 |  | Station Database Print |
|  |  | 2 |  | CO Line Database Print |
|  |  | 3 |  | ISDN Database Print |
|  |  | 4 |  | System Feature Database Print |
|  |  | 5 |  | System Timer Database Print |
|  |  | 6 |  | Toll Table Print |
|  |  | 7 |  | Authorization Code Table Print |
|  |  | 8 |  | Flexible Station Number Print |
|  |  | 9 |  | LCR Database Print |
|  |  | 10 |  | Hunt Group/Voice Mail Table Print |
|  |  | 11 |  | All Database Print |

### 7.1.6	DEFAULT  VALUES

TABLE 7.1.6.1    Station Base Program  (Main Menu 2)

| NO | SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Station Attribute | 1 | DND (Do-Not-Disturb) | YES/NO | YES |  |  |
|  |  | 2 | Speed Dial Access | YES/NO | YES |  |  |
|  |  | 3 | Page Access | YES/NO | YES |  |  |
|  |  | 4 | Call Forward Access | YES/NO | YES |  |  |
|  |  | 5 | PLA (Preferred Line Answer) | YES/NO | YES |  |  |
|  |  | 6 | Speaker-Phone/Headset | SP/HEAD | Speaker-Phone |  |  |
|  |  | 7 | Auto Speaker Selection | YES/NO | YES |  |  |
|  |  | 8 | ICM Box Signaling | YES/NO | YES |  |  |
|  |  | 9 | Automatic Hold | YES/NO | Station1:YES Station2-16:NO |  |  |
|  |  | 10 | Data Line Security | YES/NO | NO |  |  |
|  |  | 11 | DVIB Access | YES/NO | YES |  |  |
|  |  | 12 | Two-way Recording | YES/NO | NO |  |  |
|  |  | 13 | DID Ring | YES/NO | NO |  | DID Ring field (Main menu 2 – Sub menu 1 – Terminal menu 13) is used to determine the destination of an incoming DID call. If this field is set as “Yes”, the original destination of DID call receives the ring. But, if it is set as “No”, the original destination is ignored and the ring is distributed according to the ring assignment |
| 2 | ISDN Station Attribute | 1 | Sub Address | YES/NO | NO |  | If this field is set to YES, then GDK sends the called extension number in called party sub- address information of setup message to ISDN extension. |
|  |  | 2 | Long/Short | Long/Short | Short |  |  |
|  |  | 3 | MSN | YES/NO | NO |  | If this field is set to YES, then GDK sends the called extension number in called party number information of setup message to ISDN extension. |
| NO | SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT |  | REMARK |
| 2 | ISDN Station Attribute | 4 | COLR/CLIR | YES/NO | NO |  |  |
|  |  | 5 | COLP | YES/NO | NO |  |  |
|  |  | 6 | CLIP | YES/NO | NO |  |  |
|  |  | 7 | CLIS | YES/NO | NO |  |  |
|  |  | 8 | CLI NAME Display | YES/NO | NO |  | - If this field is ON, the system check whether the received CLI is matched with the speed dial data or not. If they are matched, the speed dial name is displayed. |
|  |  | 9 | - KEYPAD FACILITY | KEYPAD /DTMF | DTMF |  | - This field determines that ISDN station sends digit in DTMF or keypad facility after connected. |
| 3 | Station COS | - | Day COS/Night COS | 11 - 77 | 11 |  | 2 Digit 1st Digit-Day COS 2nd Digit-Night COS |
| 4 | CO Group Access | 1- 4 | CO Line Group | 1-4 | 1-4 |  | 1st Flex : CO GRP 1 : 4th Flex : CO GRP 4 |
| 5 | Preset Call Forward | - |  | Station No | Not Assigned | The ISDN station or ICM box cannot be assigned as preset call forward station. |  |
| 6 | Flex Buttons |  |  | 1-6 |  | Type |  |
|  | Assignment |  |  | -- |  | 1: User Button |  |
|  |  |  |  | 1-6 |  | 2: CO x |  |
|  |  |  |  | 1-4 | Not Available | 3: CO Group x |  |
|  |  |  |  | - |  | 4: LOOP |  |
|  |  |  |  | STA No. |  | 5: STA xxx |  |
|  |  |  |  | SPD Bin No. (01-99) |  | 6: SPD xx |  |
| 7 | Warm Line Selection | - |  | 1-4 | Not Assigned | 1:Flex Button 1) 2:CO Line 3:CO Group 4:Station |  |
| 8 | Page Zone | - |  | 1-2 | 1 | 1:Page Zone 1 2:Page Zone 2 |  |
| 9 | Language Code | - |  | 1-2 | Each Nation’s Language |  |  |

- Under following conditions, flex. button cannot be assigned as warm line.

-  If any station among entered range is SLT or 2B-SLT, you cannot save input value.

-  If you entered the bigger flex. button number than stations have, you cannot save input value.

TABLE  7.1.6.2  CO LINE BASE PROGRAM  (MAIN MENU 3)

| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE | DEFAULT |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CO Line Attribute | 1 | CO Line Type |  | PBX / CO | CO |  |  |
|  |  | 2 | CO Line Signal Type |  | DTMF / Pulse | DTMF |  |  |
|  |  | 3 | Universal Night Answer (UNA) |  | Yes / No | No |  |  |
|  |  | 4 | Flash Type |  | Ground / Loop | Loop |  | Only “Loop” type is supported |
|  |  | 5 | DISA Account |  | Yes/No | No |  |  |
|  |  | 6 | Loop Supervision |  | Yes/No | No |  |  |
|  |  | 7 | DID/DISA Announcement |  | Yes/No | Yes |  |  |
|  |  | 8 | CPT Tone Detect |  | YES/NO | NO |  |  |
| 2 | ISDN CO Line Attribute | 1 | COLP |  | 00-10 | 00 |  | 00:Not Assigned 01-10:indicate an entry of Outgoing COLP table |
|  |  | 2 | DID_RN |  | 00-99 | 00 |  | 00:Do not ignore 01-99:indicate an ignored called party number (DID_RN : DID removes number from called party information) |
|  |  | 3 | Type of Calling Number |  | 0-8 | 2 |  | 0:Unknown number 1:International type 2:National number 3:Not used 4:Subscriber number 5:Not used 6:Not used 7:Not used 8:No send COLP |
|  |  | 4 | DID Line Use |  | Yes/No | No |  | Yes for ISDN |
| 3 | CO Line Group Assign | - |  |  | 1 - 4 | 1 |  |  |
| 4 | DISA Type | - |  |  | U / Day / Night/Both | U |  | 0/1/2/3 |
| 5 | CO Flash Timer | - |  |  | 000 - 300 | 050 |  | 10msec base |
| 6 | CO Loop Supervision Timer | - |  |  | 0 - 20 | 0 |  | 100msec base |
| 7 | CO Line Ring Assignment (For AU70P -X.x version | - |  |  | No / Day / Night/Both | Station 1:All ring Station 2-16:No ring |  | - Flex key 1-16: ring assignment for station - Flex key 17-20: ring assignment for station group1) |
|  | CO Line Ring Assignment (Only AU75P -X.x version) | 1 | Ring to station |  | No / Day / Night/Both | Station 1:All ring Station 2-22:No ring |  |  |
|  |  | 2 | 1 | Ring to Hunt group 1 |  | No ring |  |  |
|  |  |  | 2 | Ring to Hunt group 2 |  |  |  |  |
|  |  |  | 3 | Ring to Hunt group 3 |  |  |  |  |
|  |  |  | 4 | Ring to Hunt group 4 |  |  |  |  |

1) Station  group  ring  assignment (Flex key 17-20): Only one station group must be assigned for day mode.

The  rule is applied to night mode, as well. But station group for day and night can be different.

TABLE 7.1.6.3    ISDN PROGRAM  (MAIN MENU 4)

| FLEX KEY | SUB MENU | TERMINAL MENU |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TEI Type | - |  | Auto/Fixed | Auto (3 Ports) | After changing TEI, press RESET button on MBU to operate with the new TEI type. |
| 2 | Calling Sub-Address | - |  | Yes/No | No | If this field is set to YES, then GDK sends the calling extension number in calling party sub address information of setup message. |
| 3 | Incoming Zero Insertion | - |  | Yes/No | No | This feature will be implemented in the next upgrade version. |
| 4 | Outgoing Zero Insertion | - |  | Yes/No | Yes |  |
| 5 | Outgoing Check Digit | - |  | 0 – 9 | 0 |  |
| 6 | My Area Code | - |  | - | Not Assigned |  |
| 7 | My Nation Code | - |  | - | Not Assigned |  |
| 9 | COLP Table | - |  | Entry No (01-10) | Not Assigned | Max : 10 digits |
| 10 | Msn Table | - |  | Entry No (01-24) | Not Assigned | Flex key 2: Day destination STA #(2digit/3digit)/HUNT #(61-64) /DVU Flex key 4: Night destination STA #(2digit/3digit)/HUNT #(61-64) /DVU |
| 11 | Did Digits Conversion | 1. DID Received Digit from PX |  | 2 – 4 | 3 |  |
|  |  | 2. DID First Two Digits Conversion |  | DID Received Digit No | *** | 0 - 9 # : Ignore digit * : using received digit |
|  |  | 3. DID Second Digit Conversion |  | 10 Digits | 1234567890 |  |
| 12 | ISDN Transfer Code | - |  | - | * # |  |
| 13 | ISDN Supplemen-tary Service | - Service Type | 1. Hold/Retrieve | Func/ Keypad | Keypad |  |
|  |  |  | 2. 3-party Conf. |  |  |  |
|  |  | 2.Hold Code |  | 10 digits | Not Assigned |  |
|  |  | 3.Retrieve Code |  | 10 digits | Not Assigned |  |
|  |  | 4.3-Party Conference Code |  | 10 digits | Not Assigned |  |
| 14 | Loop Back | - |  | On/Off | Off | Note 2) |

Note 1) In Basic II version, MSN destination can be either a station or a station group. So, it is possible to program MSN destination as a group number (61-64).

Note 2) The switch SW7-2 must be turned on to enable Loop-Back feature. Otherwise, it is no use setting Loop-Back ON in Admin. Program.

TABLE 7.1.6.4  SYSTEM BASE PROGRAM  (MAIN MENU 5)

| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE |  | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | System | 1 | Hold Preference |  | SYS / EXC |  | SYS |  |
|  | Attributes | 2 | Privacy |  | YES / NO |  | YES |  |
|  |  | 3 | External Night Ring |  | YES / NO |  | NO |  |
|  |  | 4 | Multi-line Conference |  | YES / NO |  | NO |  |
|  |  | 5 | CO Line Choice |  | Last Choice / Round-robin |  | Last Choice |  |
|  |  | 6 | Override 1st CO Group |  | YES / NO |  | NO |  |
|  |  | 7 | Night DVU |  | YES / NO |  | NO |  |
|  |  | 8 | Music on Camp-on |  | YES / NO |  | NO |  |
| 2 | Attendant Assignment | - |  |  | Station No. |  | 100 (1st Station) | ICM box cannot be assigned as attendant station. |
| 3 | Set System Date/Time | - |  |  |  |  |  |  |
| 4 | Lcd Display Mode | - |  |  | 1-4 |  | 3 (12-DD/MM/YY) | 1:24- DD/MM/YY 2:24- MM/DD/YY 3:12- DD/MM/YY 4:12- MM/DD/YY |
| 5 | Pbx Access Code | - |  |  | Max 2 Digits |  | Not Assigned | Max 4 PBX Access Codes |
| 6 | Did/Disa | 1 | Busy Destination |  | Tone / ATD |  | Tone |  |
|  | Destination | 2 | Error Destination |  | Tone / ATD |  | Tone |  |
| 7 | Disa Retry Counter |  | DISA Retry Counter |  | 1-9 |  | 3 |  |
| 8 | Smdr | 1 | SMDR Print Enable |  | YES / NO |  | NO |  |
|  | Attributes | 2 | SMDR Recording Call Type |  | All Call / Long Distance |  | All Call |  |
|  |  | 3 | Print Incoming Call |  | YES / NO |  | NO |  |
|  |  | 4 | SMDR Print Baud Rate (RS-232C) |  | 1-8 |  | 6 | 1:300Bps 2:600Bps 3:1200Bps 4:2400Bps 5:4800Bps 6:9600Bps 7:19200Bps 8:38400Bps |
|  |  | 5 | SMDR Save |  | NO |  | YES/NO | The system can be set to record either all outgoing calls (ALL) or only limit set by timer in Btn12 (SMDR Start Timer) |
| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE |  | DEFAULT | REMARK |
| 8 | Smdr Attributes | 6 | Records In Detail |  | YES |  | YES/NO | If this field is set as ‘NO’, the customer can see not the detailed call information but information of total call( total metering count and total cost for individual station) for SMDR Save Record. Otherwise, the customer can see |
| 9 | Attributes | - |  |  | 60/40 or 66/33 |  | 66/33 (10 PPS Only) |  |
| 10 | Alarm | 1 | Alarm Enable |  | YES / NO |  | NO |  |
|  | Attribute | 2 | Alarm Contact Type |  | Close / Open |  | Close |  |
|  |  | 3 | Alarm Signal Mode |  | Repeat / Once |  | Repeat |  |
| 11 | External Control Contact | 1 | First Contact |  | 1-3 (Loud Bell /Door/ Ext page) |  | None |  |
|  |  | 2 | Second Contact |  |  |  | None |  |
| 12 | BGM Type | - | - |  | 1-3 |  | 1 | 1:Inetrnal 2:External 3:Both |
| 13 | MOH Type | - | - |  | 1-3 |  | 1 | 1:Inetrnal 2:External 3:DVIB MOH |
| 14 | System Gain Control | 1 | Digital Keyset Rx Gain |  | 00-60 |  |  | 1.DKTU |
|  |  |  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 2 | SLT Rx Gain |  | 00-60 |  |  | 1.DKTU |
|  |  |  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 3 | CO Line Rx Gain |  | 00-60 |  |  | 1.DKTU |
|  |  |  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  |  |  | 9.MUSIC2 |
| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE |  | DEFAULT | REMARK |
| 14 | System Gain Control | 4 | Digital CO Line (DCO) Rx Gain |  | 00-60 |  |  | 1.DKTU |
|  |  |  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  |  |  | 5.DTMF RX |
|  |  |  |  |  |  |  |  | 6.TONE |
|  |  |  |  |  |  |  |  | 7.DVU |
|  |  |  |  |  |  |  |  | 8.MUSIC1 |
|  |  |  |  |  |  |  |  | 9.MUSIC2 |
|  |  | 5 | DVU Rx Gain |  | 00-60 |  |  | 1.DKTU |
|  |  |  |  |  |  |  |  | 2.SLT |
|  |  |  |  |  |  |  |  | 3.CO |
|  |  |  |  |  |  |  |  | 4.DCO |
|  |  |  |  |  |  |  |  | 5.MUSIC2 |
|  |  | 6 | DTMF Rx Gain |  | 00-60 |  |  | 1.SLT |
|  |  |  |  |  |  |  |  | 2.CO |
|  |  |  |  |  |  |  |  | 3.DCO |
| 15 | Fax Transfer CO Line | - |  |  | 3-6 |  | None |  |
| 16 | External Page Port Assignment | - | - |  | Station No. |  | None | Only SLT port |
| 17 | DID/DISA Forward To DVIB | 1 | Fwd to DVU : Busy |  | YES/NO |  | NO |  |
|  |  | 2 | Fwd to DVU : No Answer |  | YES/NO |  | NO |  |
|  |  | 3 | Fwd to DVU : Attendant No Answer |  | YES/NO |  | NO |  |
| 18 | CCR Table | 1 |  | Day/Night CCR Table |  |  | 1-2 | Input Type 1. Station 2. Hunt Group 3. DVU 4. DVU DROP 5. System Speed 6. Internal Page 7. External Page 8. All Call Page |
|  |  | 2 |  | Other CCR Table |  |  | 1-9 |  |
| 19 | Admin Password | - |  | - |  | Not Assigned | 4 Digits | - Admin Password can be assigned to enter Admin Programming mode for only Administrator who knows the Admin Password. |
| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE |  | DEFAULT | REMARK |
| 20 | DVU Setting | 1 |  | DATE/TIME Prompt Order |  | DATE/TIME | DATE/TIME TIME/DATE | - The system attendant can change the date & time prompt order so that the date & time is played as date & time or time & date order when a DVIB message is retrieved with the recorded date & time. |
|  |  | 2 |  | DVU Message Retrieve Order |  | FIFO | FIFO/LIFO | - DVIB message retrieve order is settable so that station users are able to retrieve latest message or oldest message first when multiple messages are stored in their stations. |
|  |  | 3 |  | Auto Delete of Retrieved Saved Messages |  | Not Assigned | 000-300 days | The recorded DVIB messages in the all stations may be deleted after assigned term with admin programming. The retrieved and saved messages will be deleted after assigned term. When it is set to 000, the DVIB message will not be deleted automatically. |
| 21 | Weekly Time Table | - |  | - |  | - | 1-7 | Day of Week 1. Day Start Time : 0000-2359 (Default: 0900) 2. Night Start Time: 0000-2359 (Default:1800) |
| 22 | ACNR Tone Candence | 1 |  | Ring-Back Tone |  | ON:050 OFF:100 | 000-255 (20 msec) |  |
|  |  | 2 |  | Busy Tone |  | ON:025 OFF:025 | 000-255 (20 msec) |  |
|  |  | 3 |  | Error Tone |  | ON:012 OFF:012 | 000-255 (20 msec) |  |
|  |  | 4 |  | Second Dial Tone |  | ON:070 OFF:000 | 000-255 (20 msec) |  |

TABLE 7.1.6.5  SYSTEM TIMER PROGRAM  (MAI	N MENU 6)

| NO | SUB MENU |  | FLEX KEY |  | SUB MENU |  |  | RANGE |  | DEFAULT | REMARK |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TIMER 1 |  | 1 |  | Exclusive Hold Recall Timer |  |  | 000 - 300 |  | 060 | 1 sec base |  |  |  |
|  |  |  | 2 |  | System Hold Recall Timer |  |  | 000 - 300 |  | 030 | 1 sec base |  |  |  |
|  |  |  | 3 |  | Transfer Recall Timer |  |  | 000 - 300 |  | 030 | 1 sec base |  |  |  |
|  |  |  | 4 |  | I-Hold Recall Timer |  |  | 000 - 300 |  | 030 | 1 sec base |  |  |  |
|  |  |  | 5 |  | Attendant Recall Timer |  |  | 00 - 60 |  | 01 | 1 min base |  |  |  |
|  |  |  | 6 |  | CO Ring Detect Timer |  |  | 1 - 9 |  | 2 | 100 msec base |  |  |  |
|  |  |  | 7 |  | CO Ring Stop Detect Timer |  |  | 1 - 15 |  | 6 | 1 sec base |  |  |  |
|  |  |  | 8 |  | Pause Timer |  |  | 1 - 9 |  | 1 | 1 sec base |  |  |  |
|  |  |  | 9 |  | CO Release Guard Timer |  |  | 01 - 15 |  | 01 | 1 sec base |  |  |  |
|  |  |  | 10 |  | CO Dial Delay Timer |  |  | 00 - 99 |  | 05 | 100 msec base |  |  |  |
|  |  |  | 11 |  | Preset Call Forward Timer |  |  | 00 - 99 |  | 10 | 1 sec base |  |  |  |
|  |  |  | 12 |  | Call Forward Busy/No Answer Timer |  |  | 00 - 99 |  | 10 | 1 sec base |  |  |  |
|  |  |  | 13 |  | DID/DISA No answer Timer |  |  | 00 - 99 |  | 20 | 1 sec base |  |  |  |
|  |  |  | 14 |  | Unsupervised Conference Timer |  |  | 00 - 99 |  | 10 | 1 min base |  |  |  |
|  |  |  | 15 |  | Message Reminder Tone Timer |  |  | 00 - 60 |  | 00 | 1 min base |  |  |  |
|  |  |  | 16 |  | Hook Switch Flash Timer |  |  | 001 - 250 |  | 050 | 10 msec base |  |  |  |
|  |  |  | 17 |  | Door Open Timer |  |  | 05 - 99 |  | 20 | 100 msec base |  |  |  |
|  |  |  | 18 |  | Inter-Digit Timer |  |  | 01 - 20 |  | 05 | 1 sec base |  |  |  |
|  |  |  | 19 |  | Warm Line Timer |  |  | 01 - 20 |  | 05 | 1 sec base |  |  |  |
|  |  |  | 20 |  | SMDR Start Timer |  |  | 000 - 250 |  | 000 | 1 sec base |  |  |  |
|  |  |  | 21 |  | Wake-up Fail Ring Timer |  |  | 00 - 99 |  | 20 | 1 sec base |  |  |  |
|  |  |  | 22 |  | FAX Tone Detect Timer |  |  | 01 - 10 |  | 05 | 1 sec base |  |  |  |
|  |  |  | 23 |  | FAX CO Call Timer |  |  | 1 - 5 |  | 1 | 1 min base |  |  |  |
|  |  |  | 24 |  | DVU User Record Timer |  |  | 010 - 255 |  | 20 | 1 sec base |  |  |  |
| 2 | TIMER 2 |  | 1 |  | DVU Station Forward Answer Timer |  |  | 04 - 40 |  | 04 | 1 sec base (NW: 10) |  |  |  |
|  |  |  | 2 |  | DVU Valid User Message Timer |  |  | 0 - 10 |  | 04 | 1 sec base |  |  |  |
|  |  |  | 3 |  | Hunt Disconnect Timer |  |  | 01 – 10 |  | 03 | 1 min base |  |  |  |
|  |  |  |  | 4 |  | Call Park Recall Timer | 0-600 (3 Digits) |  | 120 (sec) |  |  | - Determines the amount of time before a call placed in a call park location will recall the station placing the park. |  |  |
|  |  |  |  | 5 |  | Normal Ring To DVIB Timer | 0-300 (3 Digits) |  | 4 (sec) |  |  | When Normal CO ring comes, all ring assigned station will ring during “Normal Ring to DVIB Timer” and the call will be routed to DVIB. |  |  |
| NO | SUB MENU |  | FLEX KEY |  | SUB MENU |  |  | RANGE |  | DEFAULT | REMARK |  |  |  |
| 2 |  | TIMER 2 |  | 6 |  | CCR Inter Digit Timer | - 000 – 300 - (3 Digits) |  | - 030 |  |  | - This field is used for the CCR inter-digit timer in the DISA/DID CO line. (100ms base). |  |  |
|  |  |  |  | - 7 |  | DISA Delay Timer | 00-99 (3 Digits) |  | 00 |  |  | When this timer is expired, DISA is activated (1 sec base) |  |  |
|  |  |  |  | - 8 |  | - ACNR Pause Timer | - 005 - 300 - (3 Digits) |  | - 030 |  |  | - When expired, ACNR is activated. - (1 sec base) |  |  |
|  |  |  |  | - 9 |  | - ACNR Delay Timer | - 000 – 300 - (3 Digits) |  | - 030 |  |  | - When ACNR Pause Timer expires and there is no available CO Line in the group, this timer is invoked. |  |  |
|  |  |  |  | - 10 |  | - ACNR Tone Detect Timer | 000 –300 (3 Digits) |  | 030 |  |  | - This timer is invoked upon completion of dialing and system considers the CO party as busy in the case that CPTU cannot detect the valid tone type until this timer expires. (1 sec base) |  |  |
|  |  |  |  | - 11 |  | - ACNR No Answer Timer | - 10 – 50 - (2 Digits) |  | - 30 |  |  | - This Timer is invoked after system detects CO ring back tone or voice from CO party. After this timer, system retries ACNR. (1 sec base) |  |  |
|  |  |  |  | - 12 |  | - ACNR Retry Counter | 1 – 30 - (1 Digit) |  | - 03 |  |  | - This is decreased every time station retries ACNR, ACNR is canceled if set to 0. |  |  |

| NO | SUB MENU |  | FLEX KEY |  | SUB MENU |  |  | RANGE |  | DEFAULT | REMARK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | - 13 |  | - ACNR Retry No Tone | - 1 – 9 - (5sec) |  | - 1 |  |  | - 1 means 5 seconds, GDK-16 will wait this value to decide NO TONE. - 3 means 15 seconds. (Only for CIS) |

TABLE 7.1.6.6  TOLL TABLE (MAIN MENU 7)

| FLEX KEY | SUB MENU | TERMINAL MENU | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Allowed Table A | Entry No (01 – 20) | Not Assigned | Max : 8 digits |
| 2 | Denied Table A | Entry No (01 – 10) |  | Max : 8 digits |
| 3 | Allowed Table B | Entry No (01 – 20) |  | Max : 8 digits |
| 4 | Denied Table B | Entry No (01 – 10) |  | Max : 8 digits |
| 5 | Canned Toll Table : Allow | Entry No (01 – 10) |  | Max : 8 digits |
|  | Canned Toll Table : Deny | Entry No (01 – 10) |  | Max : 8 digits |

TABLE 7.1.6.7  AUTHORIZATION CODE TABLE (MAIN MENU 8)

| SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| Authorization Code Table | - | Entry No. (01-30) | 5 Digits | Not Assigned |  |

TABLE 7.1.6.8  FLEXIBLE STATION NUMBERING PLAN (MAIN MENU 9)

| FLEX KEY | SUB MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Numbering Plan Type | Extension / Basic | Basic |  |
| 2 | Sta Number Digit Size | 3 digit / 2 digit | 3 digit |  |
| 3 | Flexible Station Number | For AU70P-X.x version FLEX KEY 1-16 | For AU70P-X.x version 100-115 | Each flex key denotes corresponding station. |
|  |  | For AU75P-X.x version Flex Key 1-22 | For AU75P-X.x version 100 - 121 |  |

TABLE 7.1.6.9  LCR PROGRAM  (MAIN MENU 10)

| - FLEX KEY | - SUB MENU | - RANGE | - DEFAULT |  | - REMARK |
| --- | --- | --- | --- | --- | --- |
| - 1 | - LCR Access | - 3-way toggle - - DISABLE - - OVERRIDE - - ALWAYS | - Disable |  |  |
| - 2 | - Day Zone | - F1: Zone 1. Day : 1 - 7 | - 1-7 |  | - Each day belongs to only one-day zone. |
|  |  | - F2: Zone 2. Day : 1 - 7 | - - |  |  |
|  |  | - F3: Zone 3. Day : 1 - 7 | - - |  |  |
| - 3 | - Time Zone | - F1: Zone 1. Time : 00 – 24 | - 00-24 |  | - Each hour belongs to only one time zone. |
|  |  | - F2: Zone 2. Time : 00 – 24 | - - |  |  |
|  |  | - F3: Zone 3. Time : 00 – 24 | - - |  |  |
| - 4 | - LDT - (Leading Digit Table) | - 00 – 99 - (total:100) | - None | - F1: LCR type, BOTH / INT / COL. - F2: CD 12 digits - F3: DMT index. 6 digits. Total three DMT indices for each time zone1/2/3 -  F1 – Day zone 1 index to the DMT. -  F2 – Day zone 2 index to the DMT. -  F3 – Day zone 3 index to the DMT. - (For detail, refer to Table 7.2.37) |  |
| - 5 | - DMT - (Digit Modification Table) | - 00 – 99 - (total:100) | - None | - F1: A (Added digit stream). 25digits - F2: RP (Removal Position). 2digits - F3: RN (Removal Number of digits of - CODE in each table) 2 digits - F4: AP (Add Position). 2digits - F5: CG (CO group). 1digit - F6: ALT (Alternative DMT index). 2digits (For detail, refer to Table 7.2.38) |  |

TABLE 7.1.6.10  HUNT GROUP/VOICE MAIL PROGRAM  (MAIN MENU 11)

| NO | SUB MENU | FLEX KEY | TERMINAL MENU |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Hunt Group Program | 1 | Type |  | Cir/Term/UCD /VM/Ring/None | None |  |
|  |  | 2 | Group Member |  | Station No. | None | Max 6 stations |
|  |  | 3 | Attribute |  |  | None |  |
|  |  |  | Cir/ Term | 1.No Answer Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 2.Ring Timer | 0-99 | 60 | 1 second |
|  |  |  |  | 3.Overflow Destination | STA#,HUNT#, DVU, SYS SPD# |  | Destination Type - STA - HUNT - DVU - SYS SPD |
|  |  |  |  | 4.Overflow Timer | 000-600 | 180 | 1 second |
|  |  |  | UCD | 1.First Announcement Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 2.Second Announcement Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 3.Announcement Repeat Timer | 0-99 | 15 | 1 second |
|  |  |  |  | 4.Overflow Destination | STA#,HUNT#, DVU, SYS SPD# |  | Destination Type 1.STA - HUNT - DVU - SYS SPD |
|  |  |  |  | 5.Overflow Timer | 000-600 | 180 | 1 second |
|  |  |  | VM | 1.Ring Timer | 0-99 | 99 | 1 second |
|  |  |  |  | 2.Put Mail Index | 1-4 | 1 |  |
|  |  |  |  | 3.Get Mail Index | 1-4 | 2 |  |
|  |  |  |  | 4.Overflow Destination | STA#,HUNT#, DVU, SYS SPD# |  | Destination Type - STA - HUNT - DVU - SYS SPD |
|  |  |  |  | 5.Overflow Timer | 000-600 | 180 | 1 second |
|  |  |  | Ring | - Ring Announce Timer | 0-99 | 99 | 1 second |
|  |  |  |  | 2.Overflow Destination | STA#,HUNT#, DVU, SYS SPD# |  | Destination Type - STA - HUNT - DVU - SYS SPD |
|  |  |  |  | 3.Overflow Timer | 000-600 | 180 | 1 second |

| NO | SUB MENU | FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | VM Dialing Table | 1 | Put Mail | 1-2 | Prefix : P# Suffix : - | Flex 1 for Prefix, 2 for Suffix. Max 12 digits |
|  |  | 2 | Get Mail | 1-2 | Prefix : P## Suffix : - |  |
|  |  | 3 | Busy | 1-2 | Prefix : P#*3P Suffix : - |  |
|  |  | 4 | No Answer | 1-2 | Prefix : P#*4P Suffix : - |  |
|  |  | 5 | Error | 1-2 | Prefix : P#*5P Suffix : - |  |
|  |  | 6 | DND | 1-2 | Prefix : P#*6P Suffix : - |  |
|  |  | 7 | Disconnect | - | ***** |  |

## 7.2	PROGRAMMING

### 7.2.1	DATABASE INITIALIZATION (MAIN MENU 1)

The system has been pre-programmed with certain features that are called default data (Table 7.1.6.1 – 7.1.6.10). These features are loaded into memory when the system is initialized. The system should be always initialized when installed or at any time the database has been corrupted. To initialize the system to default values, proceed as follows.

PROCEDURE

| DB INIT PRESS FLEX KEY (1-12) | - Press a flex key (1-12) or []/[] key. -  Press one of flex keys (1-10) as described at Table 7.2.1. -  []: Change to the next main menu, STATION BASE PROGRAM. -  []: Change to the previous main menu, PRINT DATABASE. |
| --- | --- |
| DB INIT,STATION PRESS [HOLD/SAVE] KEY | - Press [HOLD/SAVE] button or []/[] key. When pressing [HOLD/SAVE] button, the selected database is initialized with default data.  []: Change to the next sub menu, CO LINE DATABASE INITIALIZATION.  []: Change to the previous sub menu, ALL DATABASE INITIALIZATION. |

| FLEX KEY | SUB MENU | REMARK |
| --- | --- | --- |
| 1 | Station Database Initialization |  |
| 2 | CO Line Database Initialization |  |
| 3 | ISDN Database Initialization |  |
| 4 | System Feature Database Initialization |  |
| 5 | System Timer Database Initialization |  |
| 6 | Toll Table Database Initialization |  |
| 7 | Authorization Code Table Initialization |  |
| 8 | Flexible Station Number Initialization |  |
| 9 | Flexible Button Program Initialization |  |
| 10 | LCR Database Initialization | Exist only when LCR feature is included. |
| 11 | Hunt Group/Voice Mail Table Initialization |  |
| 12 | All Database Initialization |  |

TABLE 7.2.1  Button Configuration of Initialization (MAIN MENU 1)

### 7.2.2	STATION BASE PROGRAM (MAIN-MENU 2)

The user can enter STATION BASE PROGRAM, one of main menus, by pressing 2nd FLEX BTN or []/[] at the beginning of the Admin. Programming.

#### (1)	STATION ATTRIBUTES (MAIN MENU 2 - SUB-MENU 1)

PROCEDURE

| ADMIN PROGRAM START PRESS FLEX KEY(1-12) | - Select a main-menu 2 by pressing 2nd flex key. |
| --- | --- |
| STA PGM ENTER STA RANGE | - Enter 6 digits for station range (Ex:100-112). Or enter 4 digits if station number is 2 digits. The second station number must be more than or equal to the first station number |
| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Press a flex key 1. The user can press a flex key (1-9) or []/[]. |
| STA PGM,100-112,ATTRIBUTE PRESS FLEX KEY(1-14) | - Press a flex key 1. The user can press a flex key (1-13) or []/[]. -  Press one of flex keys (1-13) to enter a sub-program of STATION ATTRIBUTES.(next LCD) -  []: Move to the next sub-menu, ISDN STATION ATTRIBUTES. -  []: Move to the previous main menu, LANGUAGE CODE. |
| STA PGM,100-112 DND YES | - To change the value, press the 1st flex key as Table 7.2.2 The 1st flex key acts as toggle setting. |
| STA PGM,100-112 DND YES | - Press [HOLD/SAVE] button or []/[] key.  [HOLD/SAVE]: Confirm and update the DND status of the station in the range, then LCD displays the current DND status.  []/[]: Change the current menu to the next/previous terminal menu.  [TRANS/PGM]: Go to the start of the Admin. program.  [REDIAL]: Go to the 1 level upper menu. |

 LCD Display of Terminal Menu in STATION ATTRIBUTES

| LCD Display | FLEX KEY |
| --- | --- |
| STA ATTR,100-112 DND YES | 1. Do Not Disturb (DND) |
| STA ATTR,100-112 SPEED YES | 2. Speed Dial Access |
| STA ATTR,100-112 PAGE YES | 3. Page Access |
| STA ATTR,100-112 FWD YES | 4. Call Forward Access |
| STA ATTR,100-112 PLA YES | 5. Preferred Line Answer (PLA) |
| STA ATTR,100-112 SPEAKER YES | 6. Speaker-Phone/Headset |
| STA ATTR,100-112 AUTO SP YES | 7. Auto Speaker Selection |
| STA ATTR,100-112 ICM BOX YES | 8. ICM Box Signaling |
| STA ATTR,100-112 A HOLD NO | 9. Automatic Hold |
| STA ATTR,100-112 SEC/3.1 NO | 10. Data Line Security, 3.1KHz |
| STA ATTR, 100-112 DVU ACCS YES | 11. DVU Access |
| STA ATTR, 100-112 TWO WAY NO | 12. Two-way Recording |
| STA ATTR, 100-112 DID RIN NO | 13. DID Ring |
| STA ATTR, 100-112 3.1KHZ NO | 14. bearer Capability 3.1KHz |

| FLEX KEY | TERMINAL MENU | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- |
| 1 | DND | YES (ON) | Enables DND to be activated by the station. |
| 2 | Speed Dial Access | YES (ON) | Allows access to system speed dial by the station. |
| 3 | Page Access | YES (ON) | Allows access to paging by the station. |
| 4 | Call Forward | YES (ON) | Enables call forward to be activated by the station. |
| 5 | PLA | YES (ON) | The allowance to answer calls by simply lifting handset or pressing [MON] button with the answering priority. |
| 6 | Speaker Phone | YES (ON) | Operate with speakerphone. |
| 7 | Auto Speaker Selection | YES (ON) | Allows accessing a CO line or place a DSS call by pressing appropriate {CO} or {DSS} button without lifting handset or pressing [MON] button. |
| 8 | ICM Box Signaling | YES (ON) | Allows receiving ICM box signal. |
| 9 | Automatic Hold | STA 1:YES STA 2-16:NO | While on a CO line, the station user seizes another CO line by depressing the {CO} button. The first CO line goes on hold automatically. |
| 10 | Data Line Security/3.1KHz | NO (OFF) | The allowance to protect from override and camp-on, while busy state. |
| 11 | DVU Access | YES (ON) | Enables a station to forward calls to DVIB |
| 12 | Two-way Recording | NO (OFF) | Enables a station to use two way recording feature |
| 13 | DID ring | NO (OFF) | Enables a station to use DID ring feature |
| 14 | Bearer Capability | NO (OFF) | YES (Only for Station 12,13 with Italy version) |

TABLE 7.2.2   Station Attributes (Sub-Menu 1 of Station Base Program)

#### (2)	STATION ISDN ATTRIBUTES (MAIN MENU 2 - SUB-MENU 2)

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - To select STATION ISDN ATTRIBUTE, press flex key 2. |
| --- | --- |
| STA PGM,100-112, ISDN STA PRESS FLEX KEY(1-9) | - To select SUB ADDR menu, press flex key 1. |
| STA ISDN,SUB ADDR NO | - LCD will show the current status. To program SUB ADDRESS, press the 1st flex key for toggle setting. (YES(LED On), NO(LED Off)) |
| STA ISDN,SUB ADDR YES | - LCD will show again the changed status. Press [HOLD/SAVE] button for updating database permanently.  [] : Move to the next terminal menu, LONG/SHORT.  [] : Move to the previous terminal menu, CLIP. |

| FLEX KEY | FIELD | DEFAULT | REMARK |
| --- | --- | --- | --- |
| 1 | Sub Address | NO | This field determines that ISDN station will use Sub Address or not. ON:YES, OFF:NO |
| 2 | Long/Short | SHORT | This field determines that ISDN station acts in Short passive mode or not. ON:LONG, OFF:SHORT |
| 3 | MSN | NO | This field determines that ISDN station will use MSN or not. ON:YES, OFF:NO |
| 4 | COLR/CLIR | NO | Restricts sending Calling Line Identification or Connected Line Number to PX. |
| 5 | COLP | NO | Allow printing Connected Line Number. |
| 6 | CLIP | NO | This field determines whether the caller’s ID is printed or not in PRIB/BRIB/STIB (T-mode). |
| 7 | CLIS | NO | This field determines whether the call’s ID is saved or not when the user does not answer. |
| 8 | CLI NAME Display | NO | This field determines that CLI name will be displayed or not . ON:YES, OFF:NO |
| 9 | - KEYPAD FACILITY | DTMF | - This field determines that ISDN station sends digit in DTMF or keypad facility after connected. |

Table 7.2.3  ISDN Station Attributes (Sub-Menu 2 of Station Base Program)

#### (3)	STATION COS  (MAIN MENU 2 - SUB-MENU 3)

Default gives all stations COS 1 for day and night operation. Each station must be assigned a class of service which governs that station's toll restriction for the day and night operation. (Table7.2.4)

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select COS menu by pressing flex key 3. |
| --- | --- |
| STA PGM,100-112, COS 11 DIAL 2 DIGITS(D+N) | - Dial 2 digits (11-77) for Day/Night COS. The current COS is displayed on the LCD. -  [] : Move to the next terminal menu, CO Line Group Access. -  [] : Move to the previous terminal menu, ISDN Station Attribute. |
| STA PGM,100-112, COS 22 DIAL 2 DIGITS(D+N) | - Press [HOLD/SAVE] button or []/[] key. The database is updated permanently by pressing [HOLD/SAVE] button. LCD displays the current COS. |

| CLASS | REMARK |
| --- | --- |
| STA COS 1 | No restrictions are placed at the station for dialing. |
| STA COS 2 | The assignments in the Exception Table A are monitored for allow and deny numbers. |
| STA COS 3 | The assignments in the Exception Table B are monitored for allow and deny numbers. |
| STA COS 4 | The assignments in the Exception Table A & B are monitored for allow and deny numbers. |
| STA COS 5 | The leading digits can not be a Long Distance code. |
| STA COS 6 | The leading digits can not be a Long Distance code. Only 8 digits maximum can be dialed. |
| STA COS 7 | A CO line cannot be used, only allowed to make an intercom call and to receive any calls. |

TABLE 7.2.4  Station Class-Of-Service (Sub-Menu 3 of Station Base Program)

#### (4)	CO LINE GROUP ACCESS   (MAIN MENU 2 - SUB-MENU 4)

If you want to change CO line group access of some stations, program this feature. Default gives to access CO Line Group 1-4 for all stations.

PROCUEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) |  | - Select CO LINE GROUP ACCESS menu by pressing a flex key 4. |
| --- | --- | --- |
| STA,100-112,CO GROUP ACC 12_4 FLEX KEY(1-4) |  | - Press flex keys (1-4) for toggle setting. LED's of flex key (1-4) displays the current status of the first station in the range. |
| STA,100-112,CO GROUP ACC 1234 FLEX KEY(1-4) |  | - Press [HOLD/SAVE] button for updating database permanently. |

| BTN | TERMINAL MENU | REMARK |
| --- | --- | --- |
| 1-4 | - | Access Authority of CO Group 1-4 (Yes: LED-On, No: LED-Off) |

TABLE 7.2.5   Button Configuration for CO Group Access

(Sub-Menu 4 of Station Base Program)

#### (5)	PRESET CALL FORWARD (MAIN MENU 2 - SUB-MENU 5)

A station can be programmed so that incoming CO lines can be forwarded to a preset station if the first station does not answer for Call Forward No Answer Timer. Default assigns no stations.

The maximum allowed number of preset call forward chain is 5. The ISDN station or ICM Box cannot be assigned as preset call forward station.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select PRESET CALL FORWARD menu by pressing a flex key 5. |
| --- | --- |
| STA,100-112, PRESET FWD ___ DIAL STA NO | - Enter the station number want to forward. (Ex.101). LCD shows current status of the station. To clear the Preset Call Forward station, press [SPEED] button. |
| STA,100-112, PRESET FWD 101 DIAL STA NO | - Press [HOLD/SAVE] button for updating database permanently. If current preset call forward chain number is 5, the user cannot save his input. |

#### (6)	FLEX BUTTON ASSIGNMENT (MAIN MENU 2 - SUB-MENU 6)

Flex. button programming is supported up to 24.

Each Flex. button of key station can be assigned as one of the followings;

When a flexible button is programmed as a function, the old one is cleared if there is a flexible button as the same function. In case of LOOP key, multiple LOOP keys are available.

| DIGIT (to dial) | TYPE | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | User Button | - | User can program by button programming procedure. |
| 2 | {CO x} Button | 1 - 6 | CO line number |
| 3 | Reserved |  |  |
| 4 | {LOOP} | - |  |
| 5 | {STA xxx} | 100 - 115 | Station No.(Station number range can be changed) |
| 6 | {SPD xx} | 01 -99 | Speed Bin |

TABLE 7.2.6   Button Type for Flexible Button Assignment

(Sub-Menu 6 of STATION BASE PROGRAM)

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select FLEX BUTTON ASSIGNMENT menu by pressing a flex key 6. |
| --- | --- |
| STA,100-112, BUTTON PGM PRESS FLEX KEY(1-24) | - Press a flex key (01-24) to be programmed. |
| STA,100-112, BUTTON PGM 01:CO1 DIAL TYPE(1-6) | - The LCD will display the current status of the pressed flex key. - Dial a digit (1-6), then LCD will display the changed type. |
| STA,100-112, BUTTON PGM 01:CO DIAL NO(1-6) | - If the pressed type in step-3 needs additional data (i.e. Digit 2,3,5,6), dial additional data. (See Table 7.2.6) |
| STA,100-112, BUTTON PGM 01:CO2 DIAL TYPE(1-6) | - Press [HOLD/SAVE] button for updating database permanently. |

| BTN | 2 Button | 8 Button | 12 Button | 24 Button | 30 Button | 44 Button |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | {LOOP} | {CO 1} | {CO 1} | {CO 1} | {CO 1} | {CO 1} |
| 2 | {LOOP} | {CO 2} | {CO 2} | {CO 2} | {CO 2} | {CO 2} |
| 3 |  | {CO 3} | {CO 3} | {CO 3} | {CO 3} | {CO 3} |
| 4 |  | {LOOP} | {CO 4} | {CO 4} | {CO 4} | {CO 4} |
| 5 |  | {STA 100} | {STA 100} | {STA 100} | {STA 100} | {STA 100} |
| 6 |  | {STA 101} | {STA 101} | {STA 101} | {STA 101} | {STA 101} |
| 7 |  | {STA 102} | {STA 102} | {STA 102} | {STA 102} | {STA 102} |
| 8 |  | {STA 103} | {STA 103} | {STA 103} | {STA 103} | {STA 103} |
| 9 |  |  | {STA 104} | {STA 104} | {STA 104} | {STA 104} |
| 10 |  |  | {STA 105} | {STA 105} | {STA 105} | {STA 105} |
| 11 |  |  | {STA 106} | {STA 106} | {STA 106} | {STA 106} |
| 12 |  |  | {STA 107} | {STA 107} | {STA 107} | {STA 107} |
| 13 |  |  |  | {STA 108} | {STA 108} | {STA 108} |
| 14 |  |  |  | {STA 109} | {STA 109} | {STA 109} |
| 15 |  |  |  | {STA 110} | {STA 110} | {STA 110} |
| 16 |  |  |  | {STA 111} | {STA 111} | {STA 111} |
| 17 |  |  |  | {STA 112} | {STA 112} | {STA 112} |
| 18 |  |  |  | {STA 113} | {STA 113} | {STA 113} |
| 19 |  |  |  | {STA 114} | {STA 114} | {STA 114} |
| 20 |  |  |  | {STA 115} | {STA 115} | {STA 115} |
| 21 |  |  |  | Empty | Empty | Empty |
| 22 |  |  |  | Empty | Empty | Empty |
| 23 |  |  |  | Empty | Empty | Empty |
| 24 |  |  |  | Empty | Empty | Empty |
| 25 - 44 | - | - | - | - | Empty | Empty |

TABLE 7.2.7  Initial Button Configurations (Sub-Menu 6 of Station Base Program)

| FLEX KEY | TYPE | DATA |
| --- | --- | --- |
| 1-24 | 1 : User Button | - |
|  | 2 : CO | CO Line 1-6 |
|  | 3: Reserved | - |
|  | 4 : LOOP | - |
|  | 5 : STA xxx | Station No(2 digits or 3 digits) |
|  | 6 : SPD xx | Speed Bin 01-99 |

TABLE 7.2.8  Button Configuration for Flexible Button Assignments

(Sub-Menu 6 of Station Base Program)

#### (7)	WARM LINE SELECTION  (MAIN MENU 2 - SUB-MENU 7)

Idle Line Selection for a station which is assigned to warm line, is activated when user takes no action for Warm Line Timer after lifting handset or pressing the [MON] button. Warm line is programmable at Sub-Menu 7 of STATION BASE PROGRAM. All stations are not assigned any Idle Line Selection by default.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select WARM LINE SELECTION menu by pressing a flex key 7. |
| --- | --- |
| STA PGM,100-112, WARM LIN CO1 DIAL TYPE(1-4) | - LCD will show the current status. Dial type and number serially. (Refer Table 7.2.9) The changed status will be displayed on the LCD. -  Press [SPEED] button to delete Warm Line Selection. -  SLT can’t select key assign for Warm Line Selection. |
| STA PGM,100-112, WARM LIN COGRP3 DIAL TYPE(1-4) | - Press [HOLD/SAVE] button for updating database permanently. |

| DIGIT | ITEM | RANGE | REMARK |
| --- | --- | --- | --- |
| 1 | Flex Button | 01-44 | To activate a feature on a flex button as if pressed. |
| 2 | CO Line | 1-6 | To seize a CO Line |
| 3 | CO Group | 1-4 | To seize a CO Line Group |
| 4 | Station | STA NUM | To call other station |

TABLE 7.2.9 Button Configuration for Warm Line Selection 
(Sub-menu 7 of STATION BASE PROGRAM)

#### (8)	PAGE ZONE  (MAIN MENU 2 - SUB-MENU 8)

Each station is assigned to paging zones. A station can be in any or no zone at all. International all call is defined as the sum of all zones. If station is not in any internal zone, it will not receive any page announcement. In GDK-16, system supports 2 internal paging zones. Default assigns all stations to internal page zone 1.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select PAGE ZONE menu by pressing flex key 8. |
| --- | --- |
| STA PGM,100-112,PAGE ZON 1_ PRESS FLEX KEY(1-2) | - LCD will show the current status. Press flex key (1-2) to change page zone. The flex key acts in toggle. |
| STA PGM,100-112,PAGE ZON 12 PRESS FLEX KEY(1-2) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (9)	LANGUAGE CODE  (MAIN MENU 2 - SUB-MENU 9)

A country can use several languages and this menu is for such case. User can select a language for your own use.

Normal LCD display during call process follows this language setting. But Admin LCD display is exceptional. It is only displayed in English.

PROCEDURE

| STA PGM,100-112 PRESS FLEX KEY(1-9) | - Select LANGUAGE CODE menu by pressing a flex key 9. |
| --- | --- |
| STA PGM, 100-112 1:ENGLISH DIAL DIGIT: 1-2 | - LCD will show the current status. Dial digit (1-2) to change language code. |
| STA PGM, LANGUAGE CODE 2:NORWAY DIAL DIGIT: 1-2 | - Press [HOLD/SAVE] button for updating database permanently. |

### 7.2.3	CO LINE PROGRAMMING (MAIN-MENU 3)

When programming, LCD and LEDs indicate current programmed data and programming status. If the programmer enters correct data, then LCD and LED's show the entered data and the data is stored in the temporary buffer area. To save data permanently, press the [HOLD/SAVE] button, then all data in the temporary buffer (same as LCD and LED’s show their status) are stored into system memory.

#### (1)	CO LINE ATTRIBUTE (MAIN MENU 3 - SUB-MENU 1)

PROCEDURE

| CO PGM ENTER CO RANGE (1-6) |  |  | - Enter CO line range (Ex.1-6) |
| --- | --- | --- | --- |
| CO PGM,1-6 PRESS FLEX KEY (1-7) |  |  | - Press flex button 1 to enter CO LINE ATTRIBUTE menu.  []: Move to the next main-menu, ISDN PROGRAM. []: Move to the previous main-menu, STATION BASE PROGRAM. |
| CO PGM,1-6, ATTRIBUTE PRESS FLEX KEY (1-8) |  |  | - Press a flex button 1 to enter LINE TYPE of CO LINE ATTRIBUTE menu. |
| CO ATTR,1-6 LINE TYPE CO |  |  | - To change CO LINE TYPE, press flex key 1 for toggle setting. LCD displays the current status. |
| CO ATTR,1-6 LINE TYPE PBX |  |  | - Press [HOLD/SAVE] button for updating database permanently. |

 LCD Display of Terminal Menu in CO LINE ATTRIBUTE

| LCD Display | FLEX KEY |
| --- | --- |
| CO ATTR,1-6 LINE TYPE CO | - CO Line Type |
| CO ATTR,1-6 SIGNAL TYPE DTMF | - Signal Type |
| CO ATTR,1-6 UNA NO | - Universal Night Answer (UNA) |
| CO ATTR,1-6 FLASH TYPE LOOP | - CO Flash Type |
| CO ATTR,1-6 DISA ACCOUNT NO | - DISA Account |
| CO ATTR,1-6 LOOP SUPERVS NO | - Loop Supervision |
| CO ATTR,1-6 DVU ANNC NO | - DVU Announcement |
| CO ATTR,1-6 CTP TN DETEC NO | - CTP Tone Detect |

| FLEX KEY | TERMINAL MENU | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- | --- |
| 1 | CO Line Type | PBX/CO | CO(OFF) | When marked PBX, a 1 or 2 digit dial code may be entered after which toll restriction is applied. |
| 2 | CO Dial Type | DTMF/Pulse | DTMF(ON) | Each CO line can be programmed to be either DTMF (tone) or dial pulse. |
| 3 | UNA | YES/NO | NO(OFF) | The allowance of Universal Night Answer service. |
| 4 | CO Flash Type | Ground/ Loop | LOOP(OFF) | Flash types of CO line are Loop Flash (timed-break) and Ground Flash. (Only Loop Flash is supported in GDK-16.) |
| 5 | DISA Account | YES/NO | NO(OFF) | When accessed another CO line in the system by DISA line, you should enter authorization code if this flag is set. |
| 6 | Loop Supervision | YES/NO | NO(OFF) |  |
| 7 | DVU Announcement | YES/NO | NO(OFF) | This field determines whether DVU announcement for DID/DISA and Hunt group announcement are provided or not to CO line. |
| 8 | CPT Tone Detect | YES/NO | NO(OFF) |  |

TABLE 7.2.10  Button Configuration for CO Line Attribute

(Sub-Menu 1 of CO Line Base Program)

#### (2)	ISDN CO LINE ATTRIBUTE (MAIN MENU 3 - SUB-MENU 2)

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | - Enter CO line range. (ex. 1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY (1-7) | - Press flex button 2, to select CO LINE ISDN ATTRIBUTE. |
| CO PGM,1-4,ISDN ATTRIBUT PRESS FLEX KEY(1-4) | - Press flex button 1, to select ISDN COLP menu of ISDN ATTRIBUTE. |
| CO PGM,ISDN ATTR,COLP 0 DIAL COLP NO(0-10) | - LCD will show the current status. Dial COLP number, then entered data will be displayed on LCD. |
| CO PGM,ISDN ATTR,COLP 3 DIAL COLP NO(0-10) | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | ITEM | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | COLP 1) | 00-10 | 00 | 00: Not assigned 01-10: Indicate an entry of outgoing COLP table (PGM 4-8). |
| 2 | DID_RN | 00-99 | 00 | 00: Do not ignore, 01-99: Indicate an ignored called party number. (DID_RN: DID Remove number from called party information) |
| 3 | TYPE OF CALLING NUMBER | 0-8 | 2 | 0 : Unknown number 1 : International type 2 : National number 3 : Not used 4 : Subscriber number type 5 : Not used 6 : Not used 7 : Not used 8 : No send COLP |
| 4 | DID Use | YES/NO | NO | NOTE: If LCO line is set as DID, it is not guaranteed that incoming LCO call will be correctly handled by the system. It is strongly recommended that LCO should not be set as DID. |

TABLE 7.2.11  ISDN CO Line Attribute (SUB-MENU 2 of CO LINE BASE PROGRAM)

1) COLP : COnnected Line identification Presentation

#### (3)	CO LINE GROUP ASSIGNMENT (MAIN MENU 3 - SUB-MENU 3)

CO groups should be assigned according to CO Type, etc. Default gives CO Group 1 for all stations. There can be 4 CO groups.

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | - Enter CO line range. (Ex.1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY(1-7) | - Press flex button 3 to enter CO LINE GROUP ASSIGNMENT menu of CO LINE BASE PROGRAM. |
| CO PGM,1-4, GROUP ASSIGN GRP1 DIAL GRP NO(1-4) | - To program, dial a digit appropriating to CO group number. |
| CO PGM,1-4, GROUP ASSIGN GRP2 DIAL GRP NO(1-4) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (4)	DISA TYPE (MAIN MENU 3 - SUB-MENU 4)

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | - Enter CO line range. (Ex.1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY(1-7) | - Press a flex button 4 to enter DISA TYPE menu of CO LINE BASE PROGRAM. |
| CO PGM,1-4, DISA TYPE NO DISA DIAL TYPE(0-3) | (3) To program, dial a digit (0-3). 0: No DISA, 1: Day DISA 2: Night DISA, 3: Day/Night DISA |
| CO PGM,1-4, DISA TYPE DAY DIAL TYPE(0-3) | - Press [HOLD/SAVE] button for updating database permanently |

| SUB MENU | DEFAULT | REMARK |
| --- | --- | --- |
| DISA Type | No DISA | Each CO line in the system can be programmed as DISA (Direct Inward System Access) line and DISA types are as followings; - 0(No DISA:U) / 1(Day Only: D) / 2(Night Only: N) / 3(Day-Night: B) |

TABLE 7.2.12   CO DISA Type (Sub-Menu 4 of Co Line Base Program)

#### (5)	CO FLASH TIMER (MAIN MENU 3 - SUB-MENU 5)

User can initiate an open loop or ground flash across a CO line for receiving CO dial tone or transferring a PABX. In this case, flash maintains for CO Flash Timer. Default of CO Flash Timer is 50.

PROCEDURE

| CO PGM, ENTER CO RANGE(1-6) | - Enter CO line range. (Ex.1-4) |
| --- | --- |
| CO PGM, 1-4 PRESS FLEX KEY(1-7) | - Press flex button 5 to enter CO FLASH TIMER menu of CO LINE BASE PROGRAM. LCD shows the current flash timer. |
| CO PGM,1-4, FLASH TIME 010*10ms DIAL TIME:0-300 | - Enter CO Flash Timer value (000-300), then entered data will be displayed on the LCD. |
| CO PGM,1-4, FLASH TIME 300*10ms DIAL TIME:0-300 | (4) Press [HOLD/SAVE] button for updating database permanently. |

#### (6)	CO LOOP SUPERVISION TIMER (MAIN MENU 3 - SUB-MENU 6)

The system can recognize the status of a CO line (normally open or closed) by open loop detect timer.

Default of CO Loop Supervision Timer is 0.

PROCEDURE

| CO PGM, ENTER CO RANGE(1-6) | - Enter CO line range. (Ex.1-4) |
| --- | --- |
| CO PGM, 1-4 PRESS FLEX KEY(1-7) | - Press flex button 6 to enter CO OPEN LOOP DETECT TIMER menu. LCD shows the current timer value. |
| CO PGM,1-4, LOOP SUPR TMR 00*100ms DIAL TIME(0-20) | - Enter Loop Supervision Timer value (00-20), then entered data will be displayed on the LCD. |
| CO PGM,1-4, LOOP SUPR TMR 20*100ms DIAL TIME(0-20) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (7)	CO RING ASSIGNMENT (MAIN MENU 3 - SUB-MENU 7)

The system can assign the status of CO rings.

PROCEDURE

| CO PGM ENTER CO RANGE(1-6) | - Enter CO line range. (Ex.1-4) |
| --- | --- |
| CO PGM,1-4 PRESS FLEX KEY (1-7) | - Press flex button 7 to enter CO RING ASSIGNMENT menu of CO LINE BASE PROGRAM. |
| CO PGM,1-4, RING ASSIGN 101:NO FLEX KEY(1-20) | - Each flex button (1-16) means each station in order. Flex button 17-20 means hunt group ring assignment. If you want to know the current CO ring assignment of any station, press the appropriate flex button. Then LCD will display current station number and ring assignment of station. If you want to change the CO ring assignment, press the flex button again. The flex button acts in 4-way toggle. (No ring, Day ring, Night ring, All ring) |
| CO PGM,1-4, RING ASSIGN 101:DAY FLEX KEY(1-20) | - Press [HOLD/SAVE] button for updating database permanently. |

|  | Termin. Menu | FLEX KEY | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| CO line ring assignment (For AU70 - X.x version |  | 1 - 16 | Station 1 – 16 | Station 1: ALL RING Station 2 – 16 : NO RING | 0 : NO RING 1 : DAY RING 2 : NIGHT RING 3 : ALL RING |
|  |  | 17 - 20 | Hunt group 1-4 | Hunt grp1-4:NO RING | 1) |
| CO line ring assignment (For AU75 - X.x version | 1 | 1 - 22 | Ring to Station 1-22 | Station 1: ALL RING Station 2 – 22 : NO RING | 0 : NO RING 1 : DAY RING 2 : NIGHT RING 3 : ALL RING |
|  | 2 | 1 2 3 4 | Ring to hunt group 1 Ring to hunt group 2 Ring to hunt group 3 Ring to hunt group 4 | NO RING |  |

1)  Hunt group ring assignment (Flex key 17-20): Only 1 hunt group must be assigned for day mode. The rule is applied to night mode, too. But hunt group for day and night can be different.

TABLE7.2.13 CO Line Ring Assignment (SUB-MENU 7 of CO LINE BASE PROGRAM)

### 7.2.4	ISDN PROGRAM (MAIN-MENU 4)

| FLEX KEY | SUB MENU | TERMINAL MENU |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TEI Type | - |  | AUTO/ FIXED | AUTO | After changing TEI, press RESET button on the MBU to operate with the new TEI type. |
| 2 | Calling SUB- Address | - |  | YES/NO | NO | If this field is set ON, then GDK sends the calling extension number in calling party sub address information of setup message. |
| 3 | Incoming Zero Insertion | - |  | YES/NO | NO |  |
| 4 | Outgoing Zero Insertion | - |  | YES/NO | YES |  |
| 5 | Outgoing Check Digit | - |  | 0 – 9 | 0 |  |
| 6 | My Area Code | - |  | Max 5 digits | Not Assigned | To delete MY AREA CODE, press [SPEED] button. |
| 7 | My Nation Code | - |  | Max 5 digits | Not Assigned | To delete MY NATION CODE, press [SPEED] button. |
| 8 | International Code | - |  | - | Not Assigned |  |
| 9 | COLP Table | - |  | 01 – 10 (Entry) | Not Assigned | Max : 10 Digits |
| 10 | MSN Table | - |  | 01 – 24 (Entry) | Not Assigned | Flex key 1: CO line No. (1-6) Flex key 2: Station No. (2 digit or 3 digit)/Hunt Grp. No.(61-64) Flex key 3: MSN No. (1-8) Flex key 4: Night destination Station No. (2 digit / 3 digit) or Station Group No. (61-64) Flex key 5: Night MSN (1-8) Flex key 6: Telephone No. (max: 20 digit) |
| 11 | DID Digit Conversion | 1.DID Received Digit from PX |  | 2 - 4 | 3 |  |
|  |  | 2.DID First Two Digits Conversion |  | DID Received Digit No | *** | 0 – 9 # : Ignore digit * : using received digit |
|  |  | 3.DID Second Digit Conversion |  | 10 Digits | 1234567890 |  |
| 12 | ISDN Transfer Code | - |  | - | * # |  |
| 13 | ISDN Supplementary Service | 1. Service Type | - Hold/ Retrieve | Keypad / Functional | Keypad |  |
|  |  |  | 2. 3 Party Conf. | Keypad / Functional | Keypad |  |
|  |  | 2. Hold Code |  | 10 Digits | None |  |
|  |  | 3. Retrieve Code |  | 10 Digits | None |  |
|  |  | 4. 3 Party Conference Code |  | 10 Digits | None |  |
| 14 | Loop Back | - |  | YES/NO | NO | Note 2) |

TABLE 7.2.14 ISDN Program (MAIN MENU 4)

Note 1) In Basic II version, MSN destination can be either a station or a station group. So, it is possible to program MSN destination as a group number (61-64).

Note 2) The switch SW7-2 must be turned ON to enable Loop-Back feature. Otherwise, it is no use setting Loop-Back ON in Admin. Programming.

#### (1)	TEI TYPE (MAIN MENU 4 - SUB-MENU 1)

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY (1-10) | - Press flex key 1 to select TEI Type menu of ISDN PROGRAM. |
| --- | --- |
| ISDN PGM, TEI TYPE AAA PRESS FLEX KEY(1-3) | - LCD will show the current status. To program TEI Type, press the flex key 1 - 3 for toggle setting. Each flex key means each ISDN port - FIXED (LED On), AUTO (LED Off) |
| ISDN PGM, TEI TYPE AFA PRESS FLEX KEY(1-3) | (3) LCD will show again the changed status.7 Press [HOLD/SAVE] button for updating database permanently. []: Move to the next sub-menu, CALLING SUB-ADDRESS. []: Move to the previous sub-menu, DID DIGIT CONVERSION. |

Note: After changing TEI type, press RESET button on the MBU to operate with the new TEI type.

(2)	ISDN COLP TABLE (MAIN MENU 4 - SUB-MENU 9)

If COLR is not activated, the GDK will send COLP information to PX when a called party the incoming call according to the type of calling party number.

PROCEDURE

| ISDN PGM, COLP TABLE DIAL ENTRY NO(01-10) | - Dial entry number (01-10) to program a table. |
| --- | --- |
| ISDN PGM, COLP TABLE 01:1234E DIAL NO | - Dial COLP number (max. 10 digits). -  To delete installed COLP number, press [SPEED] button without entering any data. |
| ISDN PGM, COLP TABLE 01:1234E DIAL NO | - Press [HOLD/SAVE] button for updating database permanently. |

#### (3)	ISDN MSN TABLE (MAIN MENU 4 - SUB-MENU 10)

By default, ISDN MSN Tables are not assigned at all.

PROCEDURE

| ISDN PGM, MSN TABLE DIAL ENTRY NO(01-24) | - Dial 2-digit entry number. |
| --- | --- |
| MSN 1:C1 D-112 0 N-111 3 TEL:543665E | - Press one of the flex keys (1-6) to select entered data type.  Flex key 1: CO line number (1-6) Never Touch.  Flex key 2: Day Destination STA # - (2digit or 3digit)/ HUNT #(61-64)/DVU -  Flex key 3: Day Destination MSN No. (1-8)  Flex key 4: Night Destination STA # - (2digit or 3digit)/ HUNT #(61-64)/DVU -  Flex key 5: Night Destination MSN No. (1-8) -  Flex key 6: Telephone number (max. 20 digits) |
| MSN TBL 1 CO1 DIAL CO NO(1-6) | - If you press flex key 1 in step 2, you select CO line number. Dial CO line number (1-6). |
| MSN TBL 1: DAY STA112 DIAL STA/GRP/DVU | - If you press flex key 2 in step 2, you select station number for Day Destination. Dial station number (2 digits or 3 digits). Or, you can dial hunt group number (61-64). or Dial ‘#’ to select the ‘DVU’. |
| MSN TBL 1: DAY MSN 1 DIAL MSN NO(1-8) | - If you press flex key 3 in step 2, you select MSN number for Day Destination. Dial MSN number (1-8). |
| MSN TBL 1: NIGHT STA112 DIAL STA/GRP /DVU | - If you press flex key 4 in step 2, you select station number for Night Destination. Dial station number (2 digits or 3 digits). Or, you can dial hunt group number (61-64). or Dial ‘#’ to select the ‘DVU’. |
| MSN TBL 1: NIGHT MSN 1 DIAL MSN NO(1-8) | - If you press flex key 5 in step 2, you select MSN number for Night Destination. Dial MSN number (1-8). |
| MSN TBL 1:TEL NO(20DGT) 5675543665E | - If you press flex key 6 in step 2, you select telephone number. Dial telephone number (max. 20 digits). |
| MSN TBL 1 CO1 DIAL CO NO(1-6) | - To delete installed number, press [SPEED] button without entering any data. |
| MSN 1:C1 D-112 0 N-111 3 TEL:543665E | - Press [HOLD/SAVE] button for updating database permanently. |

#### (4)	DID DIGITS CONVERSION (MAIN MENU 4 - SUB-MENU 11)

With Direct Inward Dialing (DID) a user dials through a public telephone network. To complete the connection, the DID trunk receives two, three or four digits from the central office, and routes the call to the proper corresponding station. The DID dialed digits may be modified by stripping up to two leading digits, by substitution of two leading digits (after stripping).

If station number size is changed, the 3 terminal menu setting of DID DIGITS CONVERSION is re-initialized according to the station number size automatically.

PROCEDURE

| ISDN PGM, DID DIGIT TABLE PRESS FLEX KEY (1-3) | - To program DID DIGITS CONVERSION TABLE, press one of the flex buttons (1-3) as Table 7.2.15. LCD shows current status of DID Digit Conversion. |
| --- | --- |
| DID TBL: RECEIVE DIGIT 3 DIAL NO (2-4) | - To assign received digit number from PBX, enter digit number data, then entered data will be displayed on LCD. |
| DID TBL: CONVERT DIGITS *** DIAL DIGITS | - To assign converted DID digits, press digits (combination of 0-9, *, #), then entered digits will be displayed on the LCD. The number of digits to be dialed depends on the receiving digit number. |
| 1234567890 :2nd DGT CONV 1234567890 DIAL 10DIGIT | - To assign DID 2nd digit conversion table, enter 10 digits - (Ex: 1 2 3 4 5 6 7 8 9 0). |
| 1234567890 :2nd DGT CONV 1224567890 DIAL 10DIGIT | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- |
| 1 | DID Received Digit No from PX | 3 | 2 - 4 |  |
| 2 | DID Digit Conversion | *** | d, *, # | d : digit (0 - 9) # : ignore digits * : any kind of digit |
| 3 | DID 2nd Digit Conversion | (1 2 3 4 5 6 7 8 9 0) |  | 10 Digits |

TABLE 7.2.15 Button Configuration for DID Digit Conversion

(SUB-MENU 10 of ISDN PROGRAM)

#### (5)	ISDN TRANSFER CODE (MAIN MENU 4 - SUB-MENU 12)

Max digit no. of ISDN Transfer Code is 2. By default, ISDN Transfer Code is assigned as *#.

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY(1-14) | - Press flex button 12 of the 1-14. |
| --- | --- |
| ISDN TRANSFER CODE (STA) *#, DIAL CODE(2DIGIT) | - Current assigned ISDN Transfer Code is displayed on LCD. - Dial 2 digits for ISDN Transfer Code to assign. - Press [SPEED] button to delete ISDN Transfer Code. |
| ISDN TRANSFER CODE (STA) *#, DIAL CODE(2DIGIT) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (6)	ISDN SUPPLEMENTARY SERVICE (MAIN MENU 4 - SUB-MENU 13)

- ISDN Supplementary Service Type

User can assign supplementary service type for 2 cases. One is Hold/Retrieve, the other is 3-party conference. By default, 2 case service types are all keypad.

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY(1-14) | - Press flex button 13 of the 1-14. |
| --- | --- |
| ISDN PGM, SUPPLEMENTRAY PRESS FLEX KEY(1-4) | - Press flex button 1 to select ISDN Supplementary Service. |
| ISDN SUPPLEMENT SVC TYPE PRESS FLEX KEY (1-2) | - User has to select which case service type he wants to assign. |
| ISDN SVC TYPE HOLD/RETRV KEY_PAD | - Current assigned Service Type is displayed on LCD. - Press flex key to change. |
| ISDN SVC TYPE HOLD/RETRV FUNCTION | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Hold/Retrieve SVC Type | Functional / Keypad | Keypad (On) |  |
| 2 | 3-Party Conference SVC Type | Functional / Keypad | Keypad (On) |  |

TABLE 7.2.16 Button Configuration for ISDN Supplementary SVC Type

(SUB-MENU 13 of ISDN PROGRAM)

- ISDN 3-Party Conference Code

Max digit no. of ISDN Hold Code is 10. By default, ISDN Hold Code is not assigned at all. The same is for ISDN Retrieve Code, ISDN 3-Party Conference Code.

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY(1-14) | - Press flex button 13 of the 1-14. |
| --- | --- |
| ISDN SUPPLEMENTARY PRESS FLEX KEY (1-4) | - Press flex button 2 to select ISDN Hold Code. |
| ISDN HOLD CODE-MAX:10DGT 9988 | - Current assigned ISDN Hold Code is displayed on the LCD. - Dial digits to enter new ISDN Hold Code. |
| ISDN HOLD CODE-MAX:10DGT 5577 | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 2 | ISDN Hold Code | Max 10 Digits | Not Assigned |  |
| 3 | ISDN Retrieve Code | Max 10 Digits | Not Assigned |  |
| 4 | ISDN 3-Party Conference Code | Max 10 Digits | Not Assigned |  |

TABLE 7.2.17  Button Configuration for ISDN Supplementary Code

(SUB-MENU 13 of ISDN PROGRAM)

#### (7)	LOOP BACK (MAIN MENU 4 - SUB-MENU 14)

Loop Back is used only for test. Default value of Loop Back is No.

PROCEDURE

| ISDN PROGRAM PRESS FLEX KEY(1-14) | - Press flex button 14 of the 1-14. |
| --- | --- |
| ISDN PGM, LOOP BACK NO | - Current assigned Loop Back is displayed on the LCD. - Press flex button to change. |
| ISDN PGM, LOOP BACK YES | - Press [HOLD/SAVE] button for updating database permanently. |

### 7.2.5	SYSTEM BASE PROGRAMMING (MAIN-MENU 5)

To change system parameters, select the main-menu 5 by pressing the 5th flex-key. When programming, LCD and LEDs indicate current programmed data and programming status. If the programmer enters correct data, then LCD and LED’s show the entered data and the data is stored in the temporary buffer area. After pressing the [HOLD/SAVE] button, all data in the temporary buffer (same as LCD/LEDs show their status) are stored into the permanent system memory.

PROCEDURE

| ADMIN PROGRAM START PRESS FLEX KEY(1-12) | - If you select SYSTEM BASE PROGRAM, following message will be displayed on the LCD. |
| --- | --- |
| SYS PGM PRESS FLEX KEY(1-22) | - Press one of the Flex key (1-22) which you want. Each flex key number means sub menu number. If you press [] or [] button, same level menu message will be displayed on the LCD one after another. |

 LCD Display of Sub Menu in SYSTEM BASE PROGRAM

| LCD DISPLAY | FLEX KEY |
| --- | --- |
| SYS PGM, ATTRIBUTE PRESS FLEX KEY(1-8) | - System Attribute |
| SYS PGM, ATTENDANT ASSIGN 100 DIAL STA NO | - Attendant Assignment |
| SYS PGM, SET DATE/TIME 01/08/22 11:06 (10DIGIT) | - Set System Date / Time |
| SYS PGM,LCD DISPLAY Mode 12/DDMMYY Dial Mode(1-4) | - LCD Display Mode |
| SYS PGM,PBX CODE PRESS FLEX KEY (1-4) | - PBX Access Code |
| SYS PGM,DID/DISA DEST PRESS FLEX KEY(1-2) | - DID/DISA Destination |
| SYS PGM, DISA RETRY COUNT 3 DIAL DIGIT(1-9) | - DISA Retry Counter |
| SYS PGM,SMDR ATTRIBUTE PRESS FLEXKEY(1-6) | - SMDR Attribute |
| SYS PGM, CO SPEED RATIO 66/33 | - Pulse Dial / Speed Ratio |
| SYS PGM, ALARM ATTRIBUTE PRESS FLEX KEY (1-3) | - Alarm Attribute |
| SYS PGM, EXT CONTACT PRESS FLEX KEY (1-2) | - External Control Contact |
| SYS PGM, BGM TYPE INTERNAL DIAL DIGIT(1-3) | - BGM (Background Music) Type |
| SYS PGM, MOH TYPE INTERNAL DIAL DIGIT(1-3) | - MOH (Music-On-Hold) Type |
| LCD DISPLAY | FLEX KEY |
| SYS PGM,SYSTEM GAIN PRESS RX-DEV TYPE:FLX1-6 | - System Gain |
| SYS PGM,FAX TRANS CO LINE 3 DIAL DIGIT(3-6) | - Fax Transfer CO Line |
| SYS PGM, EXT PAGE STATION 103 DIAL STA NO. | - External Page Station |
| SYS, DID/DISA FWD TO DVU PRESS FLEX KEY (1-3) | - DID/DISA Forward to DVU |
| SYS PGM, CCR TABLE PRESS FLEX KEY(1-2) | - CCR Table |
| SYS PGM, ADMIN PASSWORD ____ DIAL DIGIT(MAX:4) | - Admin Password |
| SYS PGM, DVU SETTING PRESS FLEX KEY(1-3) | - DVU Setting |
| SYS PGM, WEEKLY TIME TBL DIAL ENTRY NO(1-7) | - Weekly Time Table |
| SYS PGM,ACNR TONE CADENC PRESS FLEX KEY(1-4) | - ACNR Tone Candence |

#### (1)	SYSTEM ATTRIBUTE (MAIN MENU 5 – SUB MENU 1)

PROCEDURE

| SYS PGM,ATTRIBUTE PRESS FLEX KEY (1-8) | - Press one of the flex buttons (1-8) to select a sub menu of SYSTEM ATTRIBUTE. |
| --- | --- |
| SYS ATTR, HOLD TYPE SYSTEM | (2) When you press flex button 1 in SYSTEM ATTRIBUTE menu, the current status will be displayed on the LCD. To program, use the 1st flex key for toggle setting (Refer to Table 7.2.18). The changed status will be displayed on the LCD. (SYSTEM (LED On), EXCLUSIVE (LED Off)). If you want to go to another menu in SYSTEM ATTRIBUTE, press [] or [] button. |
| SYS ATTR,HOLD TYPE EXCLUSIVE | - Press [HOLD/SAVE] button for updating database permanently. |

 LCD Display of Terminal Menu in SYSTEM ATTRIBUTES

| LCD DISPLAY | FLEX KEY |
| --- | --- |
| SYS ATTR, HOLD TYPE SYSTEM | - Hold Preference |
| SYS ATTR, AUTO PRIVACY YES | - Auto Privacy |
| SYS ATTR, EXT NIGHT RING NO | - External Night Ring |
| SYS ATTR, MULTI-LINE CONF YES | - Multi-Line Conference |
| SYS ATTR, CO LINE CHOICE LAST CHOICE | - CO Line Choice |
| SYS ATTR, OVR 1ST CO GRP NO | - Override 1st CO Group |
| SYS ATTR, NIGHT DVU NO | - Night DVU |
| SYS ATTR, MUSIC ON CAMPON NO | - Music/Camp-on |

| FLEX KEY | TERMINAL MENU | DEFAULT | RANGE |  | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | Hold Preference | System (ON) | System/ Exclusive |  | If system hold is preferred, press [HOLD/SAVE] button and twice for exclusive hold. |
| 2 | Auto Privacy | YES(ON) | YES/NO |  | The system can be programmed to override CO line call to gain access to the conversation. If privacy is disabled, a station privileged to override in PGM 11 to join an existing call in progress. |
| 3 | External Night Ring | NO(OFF) | YES/NO | When CO lines are marked UNA, ring will be sent to LBC1 when an incoming call occurs on those lines during night service. |  |
| 4 | Multi-Line Conference | NO(OFF) | YES/NO | The system allows a conference with multiple CO lines. |  |
| 5 | CO Line Choice | Last Choice (ON) | Last Choice /Round-robin | The method of a CO line seizing on a CO line group access. |  |
| 6 | Override 1st Co Group | NO(OFF) | YES/NO |  |  |
| 7 | Night DVU | YES/NO | NO (OFF) | If Night DVU is enabled, DISA CO lines are disconnected in night mode after night system greeting is played. |  |
| 8 | Music on Camp on | YES/NO | NO (OFF) | A station can hear music instead of ring back tone when making camp-on to another station. |  |

TABLE 7.2.18 Button Configuration for System Attribute

(SUB-MENU 1 of SYSTEM BASE PROGRAM)

#### (2)	ATTENDANT STATION (MAIN MENU 5 – SUB MENU 2)

Only one station can be assigned as an attendant. As a default, the first station (Ex: STA 100) is assigned as the attendant.

PROCEDURE

| SYS PGM,ATTENDENT ASSIGN 100 DIAL STA NO | - To assign attendant, - Enter station number, then assigned attendant station number will be displayed on the LCD. - The attendant number cannot be deleted. |
| --- | --- |
| SYS PGM,ATTENDENT ASSIGN 102 DIAL STA NO | - Press [HOLD/SAVE] button for updating database permanently. |

#### (3)	SET SYSTEM DATE/TIME (MAIN MENU 5 – SUB MENU 3)

Date format is Year/Month/Date (YYMMDD), and Time format is Hour/Minute (HHMM) in military format. Whenever System Clock is set, second set to 0.

PROCEDURE

| SYS PGM, SET DATE/TIME 00/04/22 13:21 (10DIGIT) | - To set system clock, enter Date(YYMMDD) and Time(HHMM) serially. |
| --- | --- |
| SYS PGM, SET DATE/TIME 00/04/23 18:45 (10DIGIT) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (4)	LCD DISPLAY MODE (MAIN MENU 5 – SUB MENU 4)

Two LCD date formats are Day/Month/Year (DDMMYY) or Month/Day/Year (MMDDYY) Mode. Two LCD time formats are Ordinary (12 hour) / Military (24 hour) mode. By default LCD date format is DDMMYY and LCD time format is Ordinary (12 hour).

PROCEDURE

| SYS PGM, LCD DISPLAY MODE 12,DDMMYY , DIAL MODE(1-4) | - Current LCD DISPLAY MODE is displayed on the LCD. To change LCD Time Format and LCD Date Format, dial digit. To program, refer to Table7.2.19. |
| --- | --- |
| SYS PGM, LCD DISPLAY MODE 12,MMDDYY , DIAL MODE(1-4) | - Press [HOLD/SAVE] button for updating database permanently. |

| NO | DISPLAY | REMARK |
| --- | --- | --- |
| 1 | 24 HOUR - DD/MM/YY |  |
| 2 | 24 HOUR - MM/DD/YY |  |
| 3 | 12 HOUR - DD/MM/YY | DEFAULT |
| 4 | 12 HOUR - MM/DD/YY |  |

TABLE 7.2.19 Button Configuration for LCD Display Mode

(SUB-MENU 4 of SYSTEM BASE PROGRAM)

#### (5)	PBX ACCESS CODES (MAIN MENU 5 – SUB MENU 5)

Maximum 4 PBX Access Codes are assignable. PBX access code is 1 or 2 digit number. By default, PBX Access Codes are not assigned at all.

PROCEDURE

| SYS PGM,PBX CODE PRESS FLX KEY (1-4) | - Press one of the flex buttons 1-4. (Ex : Flex button 1 – 1st PBX Code) |
| --- | --- |
| SYS PGM,PBX CODE 1 9 (9 .. .. .. ) | - Current assigned PBX code is displayed on the LCD. - Dial 2 digits (or 1 digit) for PBX Access code to assign. - Press [SPEED] button to delete PBX Access Code. |
| SYS PGM,PBX CODE 1 8 (8 .. .. .. ) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (6)	DID/DISA DESTINATION (MAIN MENU 5 – SUB MENU 6)

A station can be arranged to forward a DID call to the attendant if the station is busy. Vacant or invalid calls are sent to the stations with ring assignment for those lines, or busy tone is presented by Admin. programming. By default Tone is assigned to DID/DISA Destination.

PROCEDURE

| SYS PGM, DID/DISA DEST PRESS FLEX KEY (1-2) | - Press one of the flex buttons (1-2). To assign DID/DISA busy destination, press flex button 1. Then selected flex button LED will be lit. |
| --- | --- |
| DID/DISA DEST:BUSY CASE TONE | - To change Busy-Destination, press flex button 1. Flex button 1 operates in toggle. Refer to TABLE 7.2.20. |
| DID/DISA DEST:BUSY CASE ATTENDANT | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT | RANGE |
| --- | --- | --- | --- |
| 1 | DID/DISA Busy Destination | Tone | Tone / Attendant (Ring) |
| 2 | DID/DISA Error Destination | Tone | Tone / Attendant (Ring) |

TABLE 7.2.20 Button Configuration for DID/DISA Destination

(SUB-MENU 6 of SYSTEM BASE PROGRAM)

#### (7)	DISA RETRY COUNTER (MAIN MENU 5 – SUB MENU 7)

When the DISA user fails to call a station or access a CO line, then DISA user can retry the other call within this retry counter. If DISA user cannot access appropriately within this counter, system disconnects the DISA line automatically.

The default value of DISA Retry Counter is 3.

PROCEDURE

| SYS PGM, DISA RETRY COUNT 5 DIAL DIGIT(1-9) | - Dial DISA retry counter. |
| --- | --- |
| SYS PGM, DISA RETRY COUNT 7 DIAL DIGIT(1-9) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (8)	SMDR ATTRIBUTE (MAIN MENU 5 – SUB MENU 8)

Station Message Detail Recording (SMDR) will provide details on both incoming and outgoing calls. An assignable database option, All Call/Long Distance Only is selected, incoming and outgoing local and long distance calls are all provided. If Long Distance Only is selected, then only outgoing calls that meet the toll check status requirements listed below are provided.

PROCEDURE

| SYS PGM,SMDR ATTRIBUTE PRESS FLEX KEY(1-6) | - Press the 1st flex button to select ENABLE PRINTING menu of SMDR ATTRIBUTE. To program SMDR ATTRIBUTE, refer to Table 7.2.21. |
| --- | --- |
| SMDR, ENABLE PRINTING YES | (2) To make a change, press flex. button 1 for toggle setting. - YES (LED On), NO (LED Off) |
| SMDR, ENABLE PRINTING NO | (3) Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | DEFAULT (LED) | RANGE | REMARK |
| --- | --- | --- | --- | --- |
| 1 | SMDR Print Enable | NO (OFF) | YES/NO | The system can be set to real time print either all outgoing calls or only limit set by SMDR timer in SYSTEM TIMER PROGRAM (Main Menu 6). |
| 2 | Print Type | All Call (ON) | All Call / Long Distance | The system can be set to record either all outgoing calls or only long distance calls, exceeding time limit set by SMDR timer. |
| 3 | Print Incoming Call | NO (OFF) | YES/NO | If this option (PIC) is set to YES, all incoming calls are printed with either all outgoing calls or long distance calls. Note: SMDR output for incoming calls does not include CLI information. |
| 4 | SMDR Print Baud Rate | 6 | 1 - 8 | The baud rate for the printer can be set to 300, 600, 1200, 2400, 4800, 9600, 19200, or 38400 baud. |
| 5 | SMDR Save | NO (OFF) | YES/NO | The system can be set to record either all outgoing calls (ALL) or only limit set by SMDR timer in SYSTEM TIMER PROGRAM(Main-Menu 6) |
| 6 | Record In Detail | YES(ON) | YES/NO | If this field is set as ‘NO’, the customer can see not the detailed call information but information of total call( total metering count and total cost for individual station) for SMDR Save Record. Otherwise, the customer can see |

TABLE 7.2.21 Button Configuration for SMDR Attributes

(SUB-MENU 8 of SYSTEM BASE PROGRAM)

#### (9)	DIAL SPEED RATIO (MAIN MENU 5 – SUB MENU 9)

PROCEDURE

| SYS PGM, CO SPEED RATIO 66/33 | - To change Dial Speed Ratio, press the 1st flex button. The 1st flex key acts in toggle. |
| --- | --- |
| SYS PGM, CO SPEED RATIO 60/40 | - Press [HOLD/SAVE] button for updating database permanently. |

| SUB MENU | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- |
| Pulse Dial Speed Ratio | 60/40% , 66/33% | 66/33%(ON) | 10PPS Only |

TABLE 7.2.22 Button Configuration for Dial Speed Ratio

(SUB-MENU 9 of SYSTEM BASE PROGRAM)

#### (10)	ALARM ATTRIBUTE (MAIN MENU 5 – SUB MENU 10)

PROCEDURE

| SYS PGM,ALARM ATTRIBUTE PRESS FLEX KEY( 1-3) | - Press flex button 1 to select ALARM ATTRIBUTE ENABLE menu of ALARM ATTRIBUTE. |
| --- | --- |
| ALARM ATTR, ENABLE YES | - To make a change, press flex button 1 for toggle setting. (ALARM ENABLE (LED On), ALARM DISABLE (LED Off)) - To move to another sub menu, press [] or [] button. Note: A SLT must not be assigned to receive signals for either the alarm or doorbell. |
| ALARM ATTR, ENABLE NO | (3) Press [HOLD/SAVE] button for updating database permanently. |

 LCD Display of Terminal Menu in ALARM ATTRIBUTE

| LCD DISPLAY | FLEX KEY |
| --- | --- |
| ALARM ATTR, ENABLE YES | - Alarm Enable |
| ALARMATTR, CONTACT TYPE CLOSE | - Alarm Contact Type |
| ALARM ATTR, SIGNAL MODE REPEAT | - Alarm Signal Mode |

| FLEX KEY | TERMINAL MENU | DEFAULT (LED) | RANGE |
| --- | --- | --- | --- |
| 1 | Alarm Enable | No (OFF) | Yes/No |
| 2 | Alarm Contact Type | Close (ON) | Close/Open |
| 3 | Alarm Signal Mode | Repeat (ON) | Repeat/Once |

TABLE 7.2.23 Button Configuration for Alarm Attribute

(SUB-MENU 10 of SYSTEM BASE PROGRAM)

#### (11)	EXTERNAL CONTROL CONTACT (MAIN MENU 5 – SUB MENU 11)

By default, External control contacts are not assigned at all.

System supports 2 external control contacts.

PROCEDURE

| SYS PGM, EXT CONTACT PRESS FLEX KEY(1-2) | - Select EXTERNAL CONTROL CONTACT number with one of the Flex buttons 1-2. (Ex: Flex button 1 - External Control Contact 1) |
| --- | --- |
| SYS PGM, EXT CONTACT 1 LBC101 TYPE(1-2)+STA NO | - If you want to set as LBC, dial 1 and station number. Or if you want to set as DOOR BELL, dial 2 only. (Station number is needed only in LBC setting) - To delete the assignment of External Control Contact, press [SPEED] button. |
| SYS PGM, EXT CONTACT 1 LBC102 TYPE(1-2)+STA NO | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TYPE | DATA |
| --- | --- | --- |
| 1-2 | 1 : LBC (Loud Bell Control) | Station No. |
|  | 2 : Door Bell | - |

TABLE 7.2.24 Button Configuration for External Control Contact

(SUB-MENU 11 of SYSTEM BASE PROGRAM)

#### (12)	BGM (Background Music) TYPE (MAIN MENU 5 – SUB MENU 12)

By default, BGM type is set to Internal Music. There can be 3 BGM types.

PROCEDURE

| SYS PGM, BGM TYPE INTERNAL DIAL DIGIT(1-3) | - To change the setting, dial BGM type (1-3). |
| --- | --- |
| SYS PGM, BGM TYPE EXTERNAL DIAL DIGIT(1-3) | - Press [HOLD/SAVE] button for updating database permanently. |

| DIGIT | BGM TYPE |
| --- | --- |
| 1 | Internal Music |
| 2 | External Music |
| 3 | Both Music |

TABLE 7.2.25 Button Configuration for BGM Type(SUB-MENU 12 of SYSTEM BASE PROGRAM)

#### (13)	MOH (Music On Hold) TYPE (MAIN MENU 5 – SUB MENU 13)

By default, MOH type is set to Internal Music. There can be 3 MOH types.

PROCEDURE

| SYS PGM, MOH TYPE INTERNAL DIAL DIGIT(1-3) | - To change the setting, dial MOH type (1-3). |
| --- | --- |
| SYS PGM, MOH TYPE EXTERNAL DIAL DIGIT(1-3) | - Press [HOLD/SAVE] button for updating database permanently. |

| DIGIT | MOH TYPE |
| --- | --- |
| 1 | Internal Music |
| 2 | External Music |
| 3 | DVU MOH |

TABLE 7.2.26 Button Configuration for MOH Type

(SUB-MENU 13 of SYSTEM BASE PROGRAM)

#### (14)	SYSTEM GAIN CONTROL (MAIN MENU 5 – SUB MENU 14)

PROCEDURE

| SYS PGM, SYSTEM GAIN PRESS RX-DEV TYPE:FLX1-6 | - Press one of the flex button (1-6) to select Rx. Device Type. Refer to TABLE 7.2.27 |
| --- | --- |
| SYSTEM GAIN:DKTU PRESS TX-DEV TYPE:FLX1-9 | - If you press flex button 1, RX device is DKTU (Digital Keyset Telephone Unit). Then, dial 1 to select DKTU as TX device type. |
| SYSTEM GAIN:DKTU<-DKTU 10 DIAL GAIN(00-60) | - Dial new gain (00-60). |
| SYSTEM GAIN:DKTU<-DKTU 15 DIAL GAIN(00-60) | - Press [HOLD/SAVE] button for updating database permanently. |

| NO | RX DEVICE TYPE | NO | TX DEVICE TYPE |
| --- | --- | --- | --- |
| 1 | DKTU (Digital Keyset Telephone Unit) | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 2 | SLT | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 3 | CO Line | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 4 | DCO(Digital CO) Line | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | DTMF |
|  |  | 6 | TONE |
|  |  | 7 | DVU |
|  |  | 8 | MUSIC1 |
|  |  | 9 | MUSIC2 |
| 5 | DVU (Digital Voice Unit) | 1 | DKTU |
|  |  | 2 | SLT |
|  |  | 3 | CO |
|  |  | 4 | DCO |
|  |  | 5 | MUSIC2 |
| 6 | DTMF Receiver | 1 | SLT |
|  |  | 2 | CO |
|  |  | 3 | DCO |

TABLE 7.2.27 Button Configuration for System Gain Control

(SUB-MENU 14 of SYSTEM BASE PROGRAM)

#### FAX TRANSFER CO LINE (MAIN MENU 5 - SUB-MENU 15)

Station 102 will be used as the fax extension. This is not programmable.

PROCEDURE

| SYS PGM, FAX TRANS CO LIN 3 DIAL DIGIT(3-6) | - Press flex button 15 to select Fax Transfer CO Line menu. |
| --- | --- |
| SYS PGM, FAX TRANS CO LIN 4 DIAL DIGIT(3-6) | - Dial new CO line (3-6). |
| SYS PGM, FAX TRANS CO LIN 4 DIAL DIGIT(3-6) | - Press [HOLD/SAVE] button for updating database permanently. |

#### (16)	EXT PAGE STATION (MAIN MENU 5 - SUB-MENU 16)

PROCEDURE

| SYS PGM, EXT PAGE STATION 102 DIAL STA NO | - Press flex button 16 to select External Page Station menu. |
| --- | --- |
| SYS PGM, EXT PAGE STATION 103 DIAL STA NO | - Dial new station no. |
| SYS PGM, EXT PAGE STATION 103 DIAL STA NO | - Press [HOLD/SAVE] button for updating database permanently. |

#### (17)	DID/DISA FWD TO DVU  (MAIN MENU 5 - SUB-MENU 17)

PROCEDURE

| SYS, DID/DIAS FWD TO DVU PRES FLEX KEY (1-3) | - Press one of the flex button (1-3) to select terminal menu of DID/DISA Fwd to DVU. |
| --- | --- |
| DID FWD TO DVU: BUSY CASE YES | - Current assigned setting is displayed on the LCD. - Press flex button to change setting. |
| DID FWD TO DVU: BUSY CASE YES | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | RANGE | DEFAULT (LED) |
| --- | --- | --- | --- |
| 1 | Fwd to DVU : Busy Case | Yes/No | No (OFF) |
| 2 | Fwd to DVU : No Answer Case | Yes/No | No (OFF) |
| 3 | Fwd to DVU : Attendant No Answer Case | Yes/No | No (OFF) |

TABLE 7.2.28 Button Configuration for DID/DISA Fwd to DVU

(SUB-MENU 17 of SYSTEM BASE PROGRAM)

#### (18)	CCR TABLE( SUB-MENU 18 )

PROCEDURE

| SYS PGM, CCR TABLE PRESS FLEX KEY (1-2) |  |  | - To select DAY/NIGHT or OTHER CCR Table, press Flex BTN 1~2 Flex 1- DAY/NIGHT CCR Flex 2- OTHER CCR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OTHER CCR TABLE DIAL ENTRY (1-9) |  |  | - To select CCR Table number, dial 1~9. |  |  |  |
| CCR TABLE 1 – ANNC 11 PRESS FLEX KEY (1-10) |  |  | - To select each entry number of CCR table, press Flex. BTN 1~10, then the entered data will be displayed on the LCD. |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1: NOT ASSIGNED |  |  | - To program call routing destination, dial destination type as follow. 1 – STA 2 – HUNT 3 – DVU 4 – DVU DROP 5 – SYSTEM SPEED DIAL 6 – INTERNAL PAGE 7 – EXTERNAL PAGE 8 – ALL CALL PAGE |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : STA 100 |  |  | - To program station number, dial the station number. (Ex. Dial station number '100') |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : HUNT GRP 62 |  |  | - To program Hunt Group, dial a hunt group number. (Ex. Dial Hunt group 62.) |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : DVU 11 |  |  | - To program system announcement, dial a DVU system announcement number. (Ex. Dial sys announcement 11) |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : DVU DROP (#) |  |  | - To program system drop the CO line after providing the announcement, dial a DVU system announcement number. |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : SYS SPD 99 |  |  | - To program System Speed Dial, dial a system speed bin number. (Ex. Dial Speed bin 99.) |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : INT PAGE 2 |  |  | - To program Internal Page , dial a Internal page number. (Ex. Dial 2.) |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : EXT PAGE 2 |  |  | - To program External Page , dial a External page number. (Ex. Dial 2.) |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : ALL CALL PAGE |  |  | - To program All Call Page , dial a All Call Page number. ( Dial 8.) |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : NOT ASSIGNED |  |  | - To delete the content of entry, press the [SPEED] button. |  |  |  |
| CCR TABLE 1 – ANNC 11 INDEX 1 : |  |  | - Press the [HOLD/SAVE] button for updating database permanently. |  |  |  |
| TYPE(DIGIT) | TYPE |  |  | RANGE | DEFAULT | REMARK |
| 1 | Station |  |  | STA # | - | #: Station Number |
| 2 | Hunt Group |  |  | HUNT # | - | #: Hunt number |
| 3 | DVU |  |  | 11-19 | - |  |
| 4 | DVU DROP |  |  | Annonuce # |  | #: DVU number |
| 5 | System Speed |  |  | 21-99 | - |  |
| 6 | Internal Page |  |  | 1-2 | - |  |
| 7 | External Page |  |  | 1 | - |  |
| 8 | All Call Page |  |  |  | - |  |

TABLE 3.2   Custom Call Routing Table

- Admin Password ( SUB-MENU 19 )

Admin Password can be assigned to enter Admin Programming mode for only Administrator who knows the Admin Password.  Admin Password is not assigned by default. 147* will override all passwords.

PROCEDURE

| SYS PGM, ADMIN PASSWORD .... DIAL DIGIT(MAX:4) | - After you select SYSTEM BASE PROGAM, Press flexible button 17 - The current admin password is displayed on the LCD |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SYS PGM, ADMIN PASSWORD 1234 DIAL DIGIT(MAX:4) | - To assign Admin Password, enter 4 digits number. Otherwise to delete the admin password, press [SPEED] button. (ex 1234) Then, Press [HOLD/SAVE] button for updating database permanently. - And, the entered admin password will be displayed on LCD. |  |  |  |  |

(20)	  DVU SETTING ( SUB-MENU 20 )

PROCEDURE

| SYS PGM,DVU SETTING PRESS FLEX KEY (1-3) | - Press one of the flex buttons(1-3) to select a sub menu of DVU SETTING |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DVU SET, DATE/TIME ORDER DATE/TIME | (2) When you press flex button 1 in DVU SETTING, the current status will be displayed on the LCD. To program, use the 1st flex key for toggle setting (Refer to Table 3.4.1).The changed status will be displayed on the LCD. ( DATE/TIME(LED On) , TIME/DATE (LED Off) ). |  |  |  |  |
|  | - If you want to go to another menu in DVU SETTING, press [] or [] Button. |  |  |  |  |
| DVU SET, DATE/TIME ORDER DATE/TIME | - Press [HOLD/SAVE] button for updating database permanently. |  |  |  |  |

 LCD Display of Sub Menu in DVU SETTING

| LCD Display | FLEX KEY |
| --- | --- |
| DVU SET, DATE/TIME ORDER DATE/TIME | - DATE/TIME Prompt Order |
| DVU SET, DVU MSG RETRIEV FIFO | - DVU Message Retrieve Order |
| DVU SET, MSG AUTO DELETE 000 DAYS DIAL DAY: 0-300 | - Auto Delete Day of Saved Messages. |

(21)	  WEEKLY TIME TABLE ( SUB-MENU 21 )

PROCEDURE

| SYS PGM, WEEKLY TIME TBL DIAL ENTRY NO(1-7) |  |  |  | - Dial one of the entry no(1-7) to select a sub menu of Weekly Time Table. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MON TIME TBL(1DGT+4DGT) DAY:0900 NIGHT:1800 |  |  |  | - The dialed digit is matched the day of week.(Ex. DIAL 1: Monday). |  |  |
| MON TIME TBL(1DGT+4DGT) DAY:0900 NIGHT:1800 |  |  |  | - To select the each mode(Day or Night), dial the digit for mode selection(Day: 1, Night: 2/e.g. Dial 1) and dial 4 digits continuously(e.g. 0930 for 09:30). |  |  |
|  | MON TIME TBL(1DGT+4DGT) DAY:0930 NIGHT:1800 |  |  |  | - Press [HOLD/SAVE] button for updating database permanently. |  |
| NO |  | ITEM | REMARK |  |  |  |
| 1 |  | Monday | Refer Table 3.5.2 |  |  |  |
| 2 |  | Tuesday |  |  |  |  |
| 3 |  | Wednesday |  |  |  |  |
| 4 |  | Thursday |  |  |  |  |
| 5 |  | Friday |  |  |  |  |
| 6 |  | Saturday |  |  |  |  |
| 7 |  | Sunday |  |  |  |  |

TABLE 3.5.1 WEEKLY TIME TABLE

| NO | ITEM | DEFAULT | REMARK |
| --- | --- | --- | --- |
| 1 | Day |  | Day ring mode start time (HH:MM) |
| 2 | Night |  | Night ring mode start time (HH:MM) |

TABLE 3.5.2 WEEKLY TIME TABLE

(22)	  ACNR TONE CADENCE( SUB-MENU 22 )

PROCEDURE

| SYS PGM,ACNR TONE CADENC PRESS FLEX KEY(1-4) |  | - Press one of the flex buttons(1-4) to select a sub menu of ACNR Tone Cadence |  |
| --- | --- | --- | --- |
| ACNR TONE,RBT CADENCE ON:050 OFF:050 (20msec) |  | - If you press the flex button 1, you can program ACNR RBT cadence. |  |
| ACNR TONE,RBT CADENCE ON:050 OFF:050 (20msec) |  | - To select the each mode(On or Off), dial the digit for mode selection(On: 1, Off: 2/ e.g. dial 1) and dial 3 digits continuously(e.g. 100). |  |
|  | ACNR TONE,RBT CADENCE ON:100 OFF:050 (20msec) |  | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | - LCD DISPLAY |
| --- | --- |
|  | - ACNR TONE,RBT CADENCE ON:050 OFF:050 (20msec) |
| 2 | - ACNR TONE,BUSY CADENCE - ON:050 OFF:050 (20msec) |
| 3 | - ACNR TONE,ERROR CADENCE - ON:050 OFF:050 (20msec) |
| 4 | - ACNR TONE,S-DIAL CADENCE - ON:050 OFF:050 (20msec) |

TABLE 3.6.1 ACNR TONE CADENCE

### 7.2.6	SYSTEM TIMER PROGRAMMING (MAIN-MENU 6)

PROCEUDURE

| SYS TMR, PRESS FLEX KEY(1-2) | - Press flex button to select Timer group. - []: Move to the next main-menu, TOLL TABLE PROGRAM. - []: Move to the previous main-menu, SYSTEM BASE PROGRAM. |
| --- | --- |
| SYM TMR 1, PRESS FLEX KEY(1-24) | - Press one of the flex buttons to select appropriate timer. |
| TIMER1,EXCL HOLD RECALL 300sec DIAL TIME(0-300) | - Current timer value is displayed on the LCD. If you want to change the value, enter the new timer in digit. |
| TIMER1,EXCL HOLD RECALL 250sec DIAL TIME(0-300) | - Press [HOLD/SAVE] button to update database permanently. LCD will display the changed time of the timer. |

| NO | SUB MENU | FLEX KEY | SUB-MENU | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | System Timer -1 | 1 | Exc. Hold Recall Timer | 060 (sec) | 000 – 300 (3 Digits) | Determines the amount of time before a call placed on exclusive hold will recall the station placing the hold. |
|  |  | 2 | Sys. Hold Recall Timer | 030 (sec) | 000 – 300 (3 Digits) | Determines the amount of time before a call placed on system hold will recall the station placing the hold. |
|  |  | 3 | Transfer Recall Timer | 030 (sec) | 000 – 300 (3 Digits) | Determines the amount of time a transferred call will ring at the station receiving the transfer and how long it will recall the station transferring the call. |
|  |  | 4 | I-Recall Timer | 030 (sec) | 000 – 300 (3 Digits) | Determines the amount of time before a call is recalled to the attendant. |
|  |  | 5 | Attendant Recall Timer | 01 (min) | 00 – 60 (2 Digits) | Determines the amount of time before system disconnects the call. |
|  |  | 6 | CO Ring Detect Timer | 2 (100ms) | 1 – 9 (1 Digit) | The CO Ring Detect Timer controls the time necessary to detect an outside line as ringing into the system. |
|  |  | 7 | CO Ring Stop Detect Timer | 6 (sec) | 01-15 (2 Digits) | This timer is to secure time interval between incoming ringing signals so that the active ringing can be lasted in the system until this timer is expired. |
|  |  | 8 | Pause Timer | 1 (sec) | 1 - 9 (1 Digit) | Determines the length of the pause for use with automatically sent digits or other speed dialing. |
|  |  | 9 | CO Release Guard Timer | 01 (sec) | 01 – 15 (2 Digits) | The CO Release Guard Timer controls the time necessary to guarantee idle loop state when the line is released. |
|  |  | 10 | CO Dial Delay Timer | 05 (100ms) | 00 - 99 (2 Digits) | Voice connection to the outside party will be made after this timer. This can be used to prevent illegal dialing in case of slow response from the Central Office Line or PBX. |

| NO | SUB MENU | FLEX KEY |  | SUB-MENU | DEFAULT | RANGE | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | System Timer -1 |  | 11 | Preset Forward Timer | 10 (sec) | 00 - 99 (2 Digits) | Determines the amount of time an outside line will ring before being forwarded to a predetermined station. This entry works with Preset Forward Assignments in station attributes. More than one station can be forwarded to the same destination. |
|  |  |  | 12 | Call Forward Busy/No Answer Timer | 10 (sec) | 00 - 99 (2 Digits) | Determines the amount of time to forward a call after a predetermined time. |
|  |  |  | 13 | DID/DISA No Answer Timer | 20 (sec) | 00 - 99 (2 Digits) | A DID call will be forwarded to the attendant if the station is busy or does not answer within this time. |
|  |  |  | 14 | Unsupervised CONF. Timer | 10 (min) | 00 - 99 (2 Digits) | Determines the amount of the time an unsupervised conference can continue after the initiator of the conference has exited the conference. |
|  |  |  | 15 | MSG Wait Reminder Tone Timer | 00 (min) | 00 - 60 (2 Digits) | Determines the amount of time between repeated reminder tones to a key telephone with a message waiting. |
|  |  |  | 16 | Hook Switch Flash Timer | 050 (10ms) 005 | 001 – 250 (3 Digits) | This timer determines how long the user could depress the hook switch in order for it to be considered a FLASH (Timed-Break Recall). (For SLT) |
|  |  |  | 17 | Door Open Timer | 20 (100ms) | 05 - 99 (2 Digits) | This timer determines of the length of time that is needed to activate as a door open relay using external page relay. |
|  |  |  | 18 | Inter Digit Timer | 05 (sec) | 01 - 20 (2 Digits) | The time between digits cannot exceed Inter-digit timer, or error tone is received. |
|  |  |  | 19 | Warm Line Timer | 05 (sec) | 01-20 (2 Digit) | User takes no action after lifting handset or pressing the [MON] button and warm line timer is expired, then idle line selection for warm line is activated. |
|  |  |  | 20 | SMDR Timer | 000 (sec) | 000 - 250 (3 Digits) | When the SMDR is enabled, the calls to be printed are set by this SMDR timer. The calls exceeding this time limit are printed. |
|  |  |  | 21 | Wake-up Fail Ring Timer | 20 (sec) | 00 - 99 (2 Digits) | After a Wake-up fail ring invokes on system attendant, the alarm ring exists during this timer. Then if this timer expires, the Alarm ring will disappear. |
|  |  |  | 22 | Fax Tone Detect Timer | 05 (sec) | 01-10 (2 Digits) | Increase to 8sec if fax doesn’t answer. |
|  |  |  | 23 | Fax CO Call Timer | 1 (min) | 1-5 (1 Digit) | Increase if fax’s get cut off half way through. |
|  |  |  | 24 | DVU User Record Timer | 20 (sec) | 010-255 (3 Digits) | The time duration of DVU user greeting |
| NO | SUB MENU | FLEX KEY |  | SUB-MENU | DEFAULT | RANGE | REMARK |
| 2 | System Timer – 2 |  | 1 | DVU Forward Answer Timer | 4 (sec) | 04-40 (2 Digits) |  |
|  |  |  | 2 | DVU Valid User MSG Timer | 4 (sec) | 00-10 (2 Digits) |  |
|  |  |  | 3 | Hunt Disconnect Timer | 3 (min) | 01-10 (2 Digits) |  |
|  |  |  | 4 | Call Park Recall Timer | 120 (sec) | 0-600 (3 Digits) | - Determines the amount of time before a call placed in a call park location will recall the station placing the park. |
|  |  |  | 5 | Normal Ring To DVIB Timer | 4 (sec) | 0-300 (3 Digits) | When Normal CO ring comes, all ring assigned station will ring during “Normal Ring to DVIB Timer” and the call will be routed to DVIB. |
|  |  |  | 6 | CCR Inter Digit Timer | - 030 | - 000 – 300 - (3 Digits) | - This field is used for the CCR inter-digit timer in the DISA/DID CO line. (100ms base). |
|  |  |  | - 7 | DISA Delay Timer | 00 | 00-99 (3 Digits) | When this timer is expired, DISA is activated (1 sec base) |
|  |  |  | - 8 | - ACNR Pause Timer | - 030 | - 005 - 300 - (3 Digits) | - When expired, ACNR is activated. - (1 sec base) |
|  |  |  | - 9 | - ACNR Delay Timer | - 030 | - 000 – 300 - (3 Digits) | - When ACNR Pause Timer expires and there is no available CO Line in the group, this timer is invoked. - When ACNR Delay Timer expired, - - Invoke ACNR Pause Timer if is no available CO line Still, ACNR is activated. (1 sec base) |
|  |  |  | - 10 | - ACNR Tone - Detect Timer | 030 | 000 –300 (3 Digits) | - This timer is invoked upon completion of dialing and system considers the CO party as busy in the case that CPTU cannot detect the valid tone type until this timer expires. (1 sec base) |
|  |  |  | - 11 | - ACNR No Answer Timer | - 30 | - 10 – 50 - (2 Digits) | - This Timer is invoked after system detects CO ring back tone or voice from CO party. After this timer, system retries ACNR. (1 sec base) |
|  |  |  | - 12 | - ACNR Retry Counter | - 03 | 1 – 30 - (1 Digit) | - This is decreased every time station retries ACNR, ACNR is canceled if set to 0. |
|  |  |  | - 13 | - ACNR Retry No Tone | - 1 | - 1 – 9 - (5sec) | - 1 means 5 seconds, GDK-16 will wait this value to decide NO TONE. - 3 means 15 seconds. (Only for CIS) |

TABLE 7.2.29 Button Configuration for System Timers (MAIN MENU 6)

### 7.2.7	TOLL TABLE (MAIN-MENU 7)

Main menu number of TOLL TABLES is 7. Press flex key 7 in Admin. starting mode. Toll tables are used to have access to certain toll free calls as well as being denied certain calls for the stations assigned STATION COS. Exception table A & B allow the station that is programmed in STA COS 2, 3 & 4 to have access to certain toll free calls as well as being denied certain calls.

#### (1)	TOLL TABLES

The Allow/Deny Tables are organized into 2 sets of tables to support 2 different toll plans at one installed site. Each allow table may contain up to 20 number strings and each deny table 10 number strings. All bins of allow and deny tables have no entries by default. Each number string can contain up to 8 entries including any number 0-9, *, #, "Don't care", "stop".

The following rules should be remembered when setting up the Allow/Deny Tables:

- If the tables have no entries, no restriction is applied.

- If entries are made in the allow table and only there, then only those numbers are allowed.

- If entries are made in the deny table and only there, then only those numbers are denied.

- If there are entries in both tables, the allow table is searched first and if number is found, it is allowed. If not found, the deny table is searched and if number is found, it is denied. If it is not found in either table, it is allowed.

| RULE | ENTRY |  | CONDITIONS & RESULT |  |
| --- | --- | --- | --- | --- |
|  | ALLOW | DENY | ALLOW TABLE | DENY TABLE |
| 1 | NO | NO | NO Restriction | NO Restriction |
| 2 | YES | NO | Found - allowed Not found - denied |  |
| 3 | NO | YES |  | Found - denied Not found - allowed |
| 4 | YES | YES | Found - allowed Not found - check deny table | Found - denied Not Found - allowed |

TABLE 7.2.30 Allow/Deny Rules

| CLASS | REMARK |
| --- | --- |
| STA COS 1 | No restrictions are placed at the station for dialing. |
| STA COS 2 | The assignments in the Exception Table A are monitored for allow and deny numbers. |
| STA COS 3 | The assignments in the Exception Table B are monitored for allow and deny numbers. |
| STA COS 4 | The assignments in the Exception Table A & B are monitored for allow and deny numbers. |
| STA COS 5 | The leading digits can not be a Long Distance code. |
| STA COS 6 | The leading digits can not be a Long Distance code. Only 8 digits maximum can be dialed. |
| STA COS 7 | Not use a CO Line, Only allowed to make an intercom call and to receive any calls. |

TABLE 7.2.31 Station Class-of-Service (SUB-MENU 3 of STATION BASE PROGRAM)

PROCEDURE

| TOLL TABLE PRESS FLEX KEY (1-5) |  | - Press one of flex buttons (1-5) to program a table. Refer to TABLE 7.2.32 (Ex: FLEX BTN 1 - Allow Table A) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TOLL TABLE: ALLOW A DIAL ENTRY NO (01-20) |  | - Dial entry number. (01-20 for Allow Table/01-10 for Deny Table). |  |  |  |
| TOLL TABLE: ALLOW A 01:E DIAL NO |  | (3) LCD displays the selected entry number and current allowed number. Enter new data and each entry can be up to 8 digits. LCD will display the new data. - To delete current toll number, press [SPEED] button without entering any data. |  |  |  |
| TOLL TABLE: ALLOW A 01:1234E DIAL NO |  | - Press [HOLD/SAVE] button for updating database permanently. |  |  |  |
| BTN | ITEM |  | DEFAULT | ENTRY | REMARK |
| 1 | Allow A |  | - | 01 - 20 | Max. 8 Digits |
| 2 | Deny A |  | - | 01 - 10 |  |
| 3 | Allow B |  | - | 01 - 20 |  |
| 4 | Deny B |  | - | 01 - 10 |  |
| 5 | Canned Toll : Allow |  | - | 01 - 10 |  |
|  | Canned Toll : Deny |  | - | 01 - 10 |  |

TABLE 7.2.32 Button Configuration for Toll Tables

| VALID DATA | FUNCTION | LCD DISPLAY |
| --- | --- | --- |
| 0 - 9, *, # | Numbers | as dialed |
| [DND/FWD] | Don’t care | ‘D’ |
| [SPEED] | Delete data | - |

TABLE 7.2.33 Data Entry of Toll Table

### 7.2.8	AUTHORIZATION CODE TABLE (MAIN-MENU 8)

By default, System Authorization Codes are not assigned at all.

PROCEDURE

| AUTHOR CODE TABLE DIAL ENTRY NO(01-30) | - Dial 2-digit entry number. |
| --- | --- |
| AUTHOR CODE TABLE 01:E DIAL DIGIT | - Dial 5-digit authorization code. The size of authorization code must be 5. |
| AUTHOR CODE TABLE 01:12345E DIAL DIGIT | - Press [HOLD/SAVE] button for updating database permanently. |

| VALID DATA | FUNCTION | LCD DISPLAY |
| --- | --- | --- |
| 0 - 9, *, # | Numbers | as dialed |
| [SPEED] | Delete data | - |

TABLE 7.2.34 Data Entry of Authorization Code Table

### 7.2.9	FLEXIBLE NUMBERING PLAN (MAIN-MENU 9)

Flexible Numbering Plan Type, Station Number Digit Size, and Flexible Station Number can be programmed.

PROCEDURE

| FLEXIBLE NUMBERING PLAN PRESS FLEX KEY(1-3) | - Press one of flex buttons (1-3) to program Flexible Numbering Plan. Refer to TABLE 7.2.35. |
| --- | --- |
| NUMBERING PLAN TYPE BASIC | - If you press flex button 1, you can select Numbering Plan Type. - To change the value, press the 1st flex key. The 1st flex key acts as toggle setting. |
| STA NUMBER DIGIT SIZE 3 | - If you press flex button 2, you can select Station Number Digit Size. - To change the value, press the 1st flex key. The 1st flex key acts as toggle setting. |
| FLEXIBLE STA NUMBERING FLX KEY + STA NO | - If you press flex button 3, you can change Station Number. Press flex button (1-16) and dial 3-digit (or 2-digit) station number to program station number. Then, press the [SPEED] button to delete current assigned button data. |
| FLEXIBLE STA NUMBERING 01:100 FLX KEY + STA NO | - If you want to program other station number, press other flex button (1-16) and dial 3-digit (or 2-digit) station number. |
| FLEXIBLE STA NUMBERING 02:101 FLX KEY + STA NO | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | SUB MENU | RANGE | DEFAULT (LED) | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Numbering Plan Type | Extension / Basic | Basic (OFF) | When the user changed the setting Extension to Basic, the station number is initialized according to station number digit size.  2 Digit Station number :10 –25  3 Digit for AU70P-X.x ver. Station number :100 – 115 - 3 Digit for AU75P-X.x version Station Number 100 - 121 There is no initialization when the setting is changed Basic to Extension. |
| 2 | STA Number Digit Size | 3 / 2 | 3 (ON) | When the station number digit size is changed, station number is always initialized. |
| 3 | Flexible STA Numbering | Basic : 10–49 (2 digit) 100-499 (3 digit) Extension:10-99(2 digit) 100-999 (3 digit) | - | User can select any station by pressing appropriate flex key. (Ex. 2nd flex key : 2nd Station) |

TABLE 7.2.35   Button Configuration for Flexible Numbering Plan

### 7.2.10	LCR PROGRAM (MAIN-MENU 10)

PROCEDURE

| LCR TABLE PRESS FLEX KEY(1-5) | - Press flex button 10 to enter LCR PROGRAM menu. |
| --- | --- |
| LCR ACCESS MODE DISABLE | - F1 : Change LCR access mode Press flex button 1 to select LCD access mode. To change setting, press flex button 1 for 3-way toggle setting. Whenever pressing [F1], the LCD will be changed as, “DIS”  “OVR”  “ALL” (toggle) Press [HOLD/SAVE] button to save the change. (Refer TABLE 7.2.36) |
| DAY ZONE, FLEX KEY(1-3) 1:M/TU/W/TH/F/SA/SU 2: 3: | - F2 : Set the day-of-week zone (3 zones) Press flex button 2 to select Day Zone menu. To set this day zone, press flex button appropriating to day zone number. |
| DAY ZONE:2, DIAL DAY(1-7) 1:M/TU/W/TH/F/SA/SU 2: 3: | - Dial the associated day. For example, if you want to set Saturday as zone 2, dial 6 (Saturday). Press [HOLD/SAVE] button to save the change LCD will be changed as “1:M/TU/W/TH/F/SU 2:SA 3:“ (Refer TABLE 7.2.36) |
| TIME_ZONE OF DAY_ZONE PRESS FLEX KEY(1-3:TOD) | - F3 : Set the time-of-day zone (3 zones) Press flex button 3 to select Time Zone menu. To enter each time zone, press the flex button. (Ex. Flex button 1: time zone of day zone 1) |
| TOD1, TIME_ZON(1DGT)+4DGT 1:00-24 2:..-.. 3:..-.. | - To enter value, dial time zone number & time range (5-digit). (Refer TABLE 7.2.36) - Time zone + [SPEED]: Delete specified time zone range. If time zone is 1, then time zone range is assigned as 00-24. Time zone 1 cannot be deleted completely. |
| LCR, LDT TABLE(0) DIAL ENTRY NO(00-99) | (5) F4 : Program the Leading Digit Table (LDT) Press flex key 4 to select LDT TABLE menu. Dial entry number (00-99) to specify LDT entry number. The number in the 1st. LCD line means filled table number. (LDT table is sorted automatically) |
| 01 BOTH CD:............ DMT: …… …… …… | - The user can press flex button (1-3) to select sub-menu. - Flex key 1 : LCR Type - Flex key 2 : CD (Code) - Flex key 3 : DMT Index |
| LDT, LCR TYPE BOTH | - ■ LCR Type - Press flex button 1 for 3-way toggle setting. - The value will be changed as “BOTH  INT  COL…”. - Press [HOLD/SAVE] button to save the change. |
| LDT, DIAL CODE(MAX:12DGT) 1234567E | - ■ CD (Code) - The max digit no. of code is 12. - Dial the code and press [HOLD/SAVE] button to save the change. |
| LDT, DAY_ZONE(1DGT)+6DGT DMT:000000 000000 000000 | ■ DMT Index - For Day zone 1 : Dial 1 and 6 digits(meaning of 6 digits: each pair (2 digits) means the index to the DMT for the each time zone 1/2/3) - For Day zone 2 : Dial 2 and 6digits - For Day zone 3 : Dial 3 and 6digits (Day zone + [SPEED] : Delete appropriate day zone DMT index) Press [HOLD/SAVE] button to save the data. * Note : - The modified one may be stored into a different bin, since GDK does ascending sort for the faster lookup. - To check another bin, then pressing [▲]/[▼] button. - About the index of DMT (Refer TABLE 7.2.37) |
| LCR DMT TABLE DIAL ENTRY NO (00-99) | (6) F5 : Program Digit-Modification-Table (DMT) Select the entry number (00-99) |
| 00 A:................ RP01 RN01 AP01 CG1 ALT.. | - The current state of selected bin will be displayed. The user should press flex button (1-6) to enter sub menu. - Flex BTN 1: Added Digit Stream - Flex BTN 2: Removal Position - Flex BTN 3: Number of Digits to be removed - Flex BTN 4: Add Position - Flex BTN 5: CO Group - Flex BTN 6: Alternative DMT Index |
| DMT, ADDED DIGIT (25DGT) 12345678901234567890E | - ■ A (Added digit stream) - Dial the Added Digit Stream (25-digit). |
| DMT, REMOVE POSITION 01 DIAL DIGIT(01-12) | - ■ RP (Removal Position) - Dial Remove Position (01-12) |
| DMT, REMOVE NO OF DGT 01 DIAL DIGIT(00-12) | - ■ RN (Removal number of digits of CODE in each table) - Dial Remove No of digit (00-12) |
| DMT, ADD POSITION 01 DIAL DIGIT(01-13) | - ■ AP (Add Position) - Dial Add Position (01-13) |
| DMT, CO GROUP 1 DIAL CO GRP NO(1-4) | - ■ C (CO group) - Dial CO group number (1-4). To delete CO group number, press [SPEED] button. |
| DMT, ALT DMT INDEX 01 DIAL DIGIT(00-99) | - ■ ALT (Alternative DMT index) - Dial DMT Index. (00-99) (Refer TABLE 7.2.37) |

| - FLEX KEY |  | - SUB MENU |  | - Default |  | - Range |  | - Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - 1 |  | - LCR Access |  | - Disable |  | - 3-way toggle - - DISABLE - - OVERRIDE - - ALWAYS |  | - DISABLE: Disable LCR - OVERRIDE: LCR whose attribute is “COL” can be accessed only through Common CO Access Code (‘9’/’0’). - ALWAYS: Dial after seizing CO line with dialing with Common CO Access Code (‘9’/’0’) and dial internally without seizing CO line. |
| - 2 |  | - Day Zone |  | - Belongs to Zone 1 |  | - - Zone : 3 - - Day : 1 - 7 |  |  |
| - 3 |  | - Time Zone |  | - Belongs to Zone 1 |  | - - Zone : 3 - - Time : 00 - 24 |  | - GDK accepts it as same value for 00 and 24 and changes to “00”, if input is 24 as starting value and vice versa. - *Note : The time not belonging to any zone will be considered as zone 1 - *Note : 10 - 13 means 10:00:00 - 12:59:59 |
| - 4 |  | - LDT - (Leading Digit Table) |  | - None |  | - 00 – 99 - (total:100) |  | - F1: LCR type - 3-way toggle (BOTH  INT  COL) - F2: CD (12 digits) - F3: DMT index. 6 digits. - (meaning of 6 digits : each pair(2 digits) means the - index to the DMT for the each time zone 1/2/3) -  F1 – Day zone 1 index to the DMT. -  F2 – Day zone 2 index to the DMT. -  F3 – Day zone 3 index to the DMT. - (For detail, refer to Table 7.2.37) |
| - 5 | - DMT - (Digit Modification Table) |  | - None |  | - 00 – 99 - (total:100) |  | - F1: A (Added digit stream). 25 digits - F2: RP (Removal Position). 2 digits - F3: RN (Removal Number of digits of CODE in each table) 2 digits - F4: AP (Add Position). 2digits - F5: CG (CO Group). 1digit - F6: ALT (Alternative DMT index). 2 digits - (For detail, refer to Table 7.2.38) |  |

TABLE 7.2.36   Button Configuration for LCR (Main Menu 10)

| - Flex Key | - TERMINAL MENU | - Default | - Range | - Remark |
| --- | --- | --- | --- | --- |
| - 1 | - LCR type | - BOTH | - 3-way toggle - BOTH INT COL | - INT : look up this entry only for internal dialing - COL : look up this entry only after dialing “CO Access Code” - BOTH : look up this entry for both INT and COL |
| - 2 | - CD | - None | - 12 digits | - To be compared with the dialed digits by a user. |
| - 3 | - DMT index | - 00 | - 00-99 |  |

TABLE 7.2.37   Button Configuration for LDT Table (Main Menu 10 – Sub Menu 4)

| - Flex Key | - TERMINAL MENU | - Default | - Range | - Remark |
| --- | --- | --- | --- | --- |
| - 1 | - Added Digit Stream | - None | - 25 digits | - Normal digits (0 .. 9, * , #) - Special characters [CALLBK]: Pause [FLASH]: Billing code (Extension number) [SPEED]: Delete Added Digit stream |
| 2 | Removal Position | 01 | 01 – 12 | Index to CD stream in Lead table to be removed |
| 3 | Number of digits to be Removed | 00 | 00 - 12 | Remove digits in CD stream up to this count |
| 4 | Add Position | 01 | 01 - 13 | Determine the position of CD stream after removal, where the A stream will be inserted. |
| 5 | CO Group | 01 | 1 - 4 | Determines which CO group is used for LCR dialing |
| 6 | Alternative DMT Index | None | 00 - 99 | Determine alternative DMT index when there is no idle CO line in CO group. |

TABLE 7.2.38   Button Configuration for DMT Table (Main Menu 10 – Sub Menu 5)

### 7.2.11	HUNT GROUP PROGRAM / VM TABLE (MAIN-MENU 11)

#### (1)	HUNT GROUP PROGRAM (MAIN MENU 11 – SUB MENU 1)

Stations in the system can be grouped so that incoming calls will search (hunt) for an idle station in the group. Three hunting processes can be assigned: Circular, Terminal, or UCD. Or 2 extra methods are also available: Ring, VM.

PROCEDURE

| HUNT GRP PGM/VM TBL PRESS FLEX KEY(1-2) | - Press flex button1 to select Hunt Group Program |
| --- | --- |
| HUNT GROUP PROGRAM PRESS FLEX KEY 1-4:GRP NO | - Press one of the flex buttons (1-4) to select group no. Flex button 1-4 means Hunt group 61-64. |
| HUNT GROUP 61 PRESS FLEX KEY(1-3) | - Press one of the flex buttons (1-3) to select terminal menu of Hunt Group Program. |
| HUNT GROUP 61 TYPE NONE DIAL DIGIT(1-5) | (4) LCD displays the current type. Dial digit (1-5) to select Hunt Group Type. There are 5 types. |
| HUNT GROUP 61 TYPE UCD DIAL DIGIT(1-5) | - Press [HOLD/SAVE] button for updating database permanently. |

 LCD Display of Terminal Menu in HUNT GROUP PROGRAM

| LCD DISPLAY | FLEX KEY |
| --- | --- |
| HUNT GROUP 61 TYPE TERM DIAL DIGIT(1-5) | - Hunt Group Type |
| - GRP 61 MEMBER(MAX:6) - 100 101 | - Hunt Group Member |
| - 61:CIR/TERM HUNT GRP ATTR - PRESS FLEX KEY(1-5) | - Hunt Group Attribute |

| FLEX KEY | TERMINAL MENU |  | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- | --- |
| 1 | Type |  | Cir/Term/UCD/ VM/Ring/None | None |  |
| 2 | Group Member |  | Station No. (Max 6 stations) | None |  |
| 3 | Attribute |  |  | None |  |
|  | Cir/ Term | 1. No Answer Timer | 0-99(sec) | 15 | If calls sent to the called station is unavailable or unanswered in this no answer time, the call is proceeded to the next listed station in the group. |
|  |  | 2. Ring Timer | 0-99(sec) | 60 | If this timer expires after a call comes in the group, the system announces the greeting if exists. |
|  |  | 3.Overflow Destination | Sta. No. HUNT No. DVU or DVU# SYS SPD No. |  | Destination Type - STA - HUNT - DVU or DVU# (# = Discon) - SYS SPD |
|  |  | 4.Overflow Timer | 000-600 | 180 | 1 second |
|  | UCD | 1. First Announcement Timer | 0-99(sec) | 05 | If all stations in the group are busy when a call is received for the group, the call may continue to wait for an available station in the group. If queued, the call may be sent to a UCD announcement when the queue period exceeds the queue to the 1st announcement timer. If the timer is set to 0, the call will receive the full first announce-ment prior to the hunting process. |
|  |  | 2. Second Announcement Timer | 0-99(sec) | 30 | The second announcement can be provided if the call continues to wait beyond the queue to the 2nd announcement timer. |
|  |  | 3. Announcement Repeat Timer | 0-99(sec) | 60 | The second message can be repeated until the call is answered or is disconnected when the queue period exceeds the Announcement Repeat Timer. |
|  |  | 4.Overflow Destination | Sta. No. HUNT No. DVU or DVU# SYS SPD No. |  | Destination Type 1. STA 2. HUNT 3. DVU or DVU# (# = Discon) 4. SYS SPD |
|  |  | 5. Overflow Timer | 001-600 | 180 | 1 second |

| - Flex Key | - TERMINAL MENU | - Default | - Range | - Remark |
| --- | --- | --- | --- | --- |

|  | VM | 1. Ring Timer | 0-99(sec) | 99 | If this timer expires after a call comes in the group, the system announces the greeting if exists. |
| --- | --- | --- | --- | --- | --- |
|  |  | 2. Put Mail Index | 1-4 | 1 | This index is one of the voice mail dialing tables. |
|  |  | 3. Get Mail Index | 1-4 | 2 | This index is one of the voice mail dialing tables. |
|  |  | 4.Overflow Destination | Sta. No. HUNT No. DVU or DVU# SYS SPD No. |  | Destination Type 1. STA 2. HUNT 3. DVU or DVU# (# = Discon) 4. SYS SPD |
|  |  | 5. Overflow Timer | 000-600 | 180 | 1 second |
|  | Ring | 1.Ring Announce Timer | 0-99 | 99 | 1 second |
|  |  | 2.Overflow Destination | Sta. No. HUNT No. DVU or DVU# SYS SPD No. |  | Destination Type 1. STA 2. HUNT 3. DVU or DVU# (# = Discon) 4. SYS SPD |
|  |  | 3.Overflow Timer | 000-600 | 180 | 1 second |

TABLE 7.2.39  Button Configuration for Station Group Program

(MAIN MENU 11-SUB MENU 1)

#### (2)	VOICE MAIL DIALING TABLE (MAIN MENU 11 – SUB MENU 2)

PROCEDURE

| HUNT GRP PGM/VM TBL PRESS FLEX KEY(1-2) |  |  | - Press flex button 2 to select Voice Mail Dialing Table. |
| --- | --- | --- | --- |
| VOICE MAIL DIALING TBL PRESS FLEX KEY(1-7) |  |  | - Press one of the flex buttons (1-7) to select table you want to change. |
| VM TABLE:PUT MAIL PRESS FLEX KEY(1-2) |  |  | - Press one of the flex buttons (1-2) to select between prefix and suffix. |
| PUT MAIL PREFIX(MAX:12) P# DIAL DIGIT |  |  | - LCD displays the current Prefix. Dial digits to enter new prefix. |
| PUT MAIL PREFIX(MAX:12) P#1 DIAL DIGIT |  |  | - Press [HOLD/SAVE] button for updating database permanently. |

| FLEX KEY | TERMINAL MENU | RANGE | DEFAULT | REMARK |
| --- | --- | --- | --- | --- |
| 1 | Put Mail | 1-2 | Prefix : P# Suffix : - | Flex 1 for Prefix, 2 for Suffix. Max. 12 digits |
| 2 | Get Mail | 1-2 | Prefix : P## Suffix : - |  |
| 3 | Busy | 1-2 | Prefix : P#*3P Suffix : - |  |
| 4 | No Answer | 1-2 | Prefix : P#*4P Suffix : - |  |
| 5 | Error | 1-2 | Prefix : P#*5P Suffix : - |  |
| 6 | DND | 1-2 | Prefix : P#*6P Suffix : - |  |
| 7 | Disconnect | - | ***** |  |

TABLE 7.2.40  Button Configuration for VM Table Program (MAIN MENU 11-SUB MENU 2)

### 7.2.12	DATABASE PRINT (MAIN-MENU 12)

In order to obtain a hard copy printout of the database, a printer must be connected to the RS-232C connector. If user entered Database Print menu while printing, error-tone is heard.

PROCEDURE

| PRINT DATABASE PRESS FLEX KEY (1-11) | - To print database, press one of flex buttons 1-11. Refer to TABLE 7.2.41 |
| --- | --- |
| PRINT DB: ALL DB PRESS [HOLD/SAVE] KEY | - To print out the selected database, press [HOLD/SAVE] button. |

| FLEX KEY | SUB-MENU | DATA | REMARK |
| --- | --- | --- | --- |
| 1 | Station Database Print | - | All Station (1-16) DB Print |
| 2 | CO Line Database Print | - | All CO (1-6) DB Print |
| 3 | ISDN Database Print | - |  |
| 4 | System Feature Database Print | - |  |
| 5 | System Timer Print | - |  |
| 6 | Toll Tables Print | - |  |
| 7 | Authorization Code Table Print | - |  |
| 8 | Flexible Station Numbering Print | - |  |
| 9 | LCR Database Print | - | Printed only when LCR feature is included. |
| 10 | HUNT Group/VM Table Print | - |  |
| 11 | All Database Print | - | Print above 9 menus serially. |

TABLE 7.2.41   Button Configuration for Database Print (Main-Menu 12)

(Printing Example)

This printing example is for GDK-16 system.

| Station Database | STATION DATABASE ------------------------------ <STA 100> 1. STATION ATTRIBUTE DND SPDA PAGE FWD PLA SPK ASPK IBOX AHLD DSEC DVU_ACC 3.1KHZ N Y N N Y Y Y Y Y N N N 2. ISDN ATTRIBUTE SUB_ADDR LONG/SHORT MSN COLR COLP CLIP CLIS N N N N Y Y N 02 3. COS : DAY(1)/NIGHT(1) 4. CO GROUP ACCESS : 1234 5. PRESET CALL FORWARD : NOT ASSIGNED 6. FLEX BUTTON ASSIGNMENT 1 2 3 4 5 6 7 8 CO1 CO2 CO3 CO4 STA100 STA101 STA102 STA103 9 10 11 12 13 14 15 16 STA104 STA105 STA106 STA107 STA108 STA109 STA110 STA111 17 18 19 20 21 22 23 24 STA112 STA113 STA114 STA115 USER USER USER USER 7. WARM LINE : NO 8. PAGE ZONE : 1 9. LANGUAGE CODE : NORWAY <STA 101> : : (Printed like above for another station to range end station) |
| --- | --- |

| CO Line Database | CO LINE DATABASE ----------------------------- <CO 1> 1. CO LINE ATTRIBUTE TYPE SIGNAL UNA FLASH DISA_ACT LOOP_SUPERVISN DID/DISA VOICE CO DTMF N LOOP N N N 2. ISDN CO LINE ATTRIBUTE COLP DID_RN CALL_NO_TYPE DID_USE 0 0 2 N 3. CO GROUP : 1 4. DISA TYPE : NO DISA 5. CO FLASH TIME : 10*10ms 6. CO LOOP SUPERVISION TIME : 5*100ms 7. CO RING ASSIGNMENT 100(A) 101(U) 102(U) 103(U) 104(U) 105(U) 106(U) 107(U) 108(U) 109(U) 110(U) 111(U) 112(U) 113(U) 114(U) 115(U) 61(U) 62(U) 63(U) 64(U) <CO 2> : : (Printed like above for another CO line) |
| --- | --- |

| ISDN Database | ISDN PROGRAM ------------------------ 1. TEI TYPE : PORT1(AUTO) PORT2(AUTO) PORT3(AUTO) 2. CALL SUB ADDR : N 3. INCOMING ZERO INSERTION : N 4. OUTGOING ZERO INSERTION : Y 5. OUTGOING CHECK DIGIT : 0 6. MY AREA CODE : NOT ASSIGNED 7. MY NATION CODE : NOT ASSIGNED 8. INTERNATIONAL CODE : NOT ASSIGNED 9. ISDN COLP TABLE (ENTRY:01-10, MAX:10 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 10. ISDN MSN TABLE (ENTRY:01-24, TEL_NO MAX:20 DIGIT) ENTRY CO TEL_NO DAY(STA MSN_NO) NIGHT(STA MSN_NO) -------------------------------------------------------------------------------------------------------------------------------- 1 : 1 12345678901234567890E 111 1 101 1 2 : 2 1234567890E 112 2 102 3 3 : - E ___ 0 ___ 0 4 : - E ___ 0 ___ 0 5 : - E ___ 0 ___ 0 6 : - E ___ 0 ___ 0 7 : - E ___ 0 ___ 0 8 : - E ___ 0 ___ 0 9 : - E ___ 0 ___ 0 10 : - E ___ 0 ___ 0 11 : - E ___ 0 ___ 0 12 : - E ___ 0 ___ 0 13 : - E ___ 0 ___ 0 14 : - E ___ 0 ___ 0 15 : - E ___ 0 ___ 0 16 : - E ___ 0 ___ 0 17 : - E ___ 0 ___ 0 18 : - E ___ 0 ___ 0 19 : - E ___ 0 ___ 0 20 : - E ___ 0 ___ 0 21 : - E ___ 0 ___ 0 22 : - E ___ 0 ___ 0 23 : - E ___ 0 ___ 0 24 : - E ___ 0 ___ 0 11. DID DIGIT CONVERSION RCV DIGIT NO : 2 DIGIT CONVERSION : ** 2nd DIGIT CONVERSION : 1234567890 -> 1234567890 12. ISDN TRANSFER CODE : *# 13. ISDN SUPPLEMENTARY SVC - ISDN SUPPLEMENTARY SVC TYPE : HOLD/RETRV:KEY_PAD , 3PTY:KEY_PAD - ISDN HOLD CODE : *75# - ISDN RETRIEVE CODE : *77# - ISDN 3-PARTY CONFERENCE CODE : *77# 14. LOOP BACK : NO |
| --- | --- |
| System Feature | SYSTEM FEATURE ---------------------------- 1. SYSTEM ATTRIBUTE HOLD_TYPE AUTO_PRIVACY EXT_NIGHT_RING MULTI_LINE_CONF SYSTEM Y N N CO_LINE_CHOICE OVR_1st_COGRP NIGHT_DVU MUSIC_ON_CAMPON ROUND-ROBIN Y N N 2. ATTENDANT STATION : 100 3. DATE & TIME : 98/04/21 14:35 4. LCD DISPLAY MODE : 12H DDMMYY 5. PABX ACCESS CODE : __ __ __ __ 6. DID/DISA DESTINATION BUSY CASE DESTINATION : TONE ERROR CASE DESTINATION : TONE 7. DISA RETRY COUNTER : 3 8. SMDR ATTRIBUTE ENABLE PRINTING : N PRINT TYPE : LONG DISTANCE PRINT INCOM CALL : N BAUD RATE : 9600 BPS SMDR SAVE : N RECORDS IN DETAILS : Y 9. PULSE DIAL SPEED RATIO : 66/33 10. ALARM ATTRIBUTE ENABLE CONTACT_TYPE SIGNAL_MODE N CLOSE REPEAT 11. EXTERNAL CONTROL CONTACT 1:NO 2:NO 12. BGM(Background Music) TYPE : 1(INTERNAL) 13. MOH(Music On Hold) TYPE : 1(INTERNAL) |

| System Feature (continued) | 14. SYSTEM GAIN -------------------------------------- RX DEV ¦ TX DEV ¦ GAIN -------------------------------------- DKTU ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ DTMF ¦ 8 ¦ TONE ¦ 32 ¦ DVU ¦ 29 ¦ MUSIC1 ¦ 29 ¦ MUSIC2 ¦ 29 SLT ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ DTMF ¦ 8 ¦ TONE ¦ 32 ¦ DVU ¦ 29 ¦ MUSIC1 ¦ 29 ¦ MUSIC2 ¦ 29 CO ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ DTMF ¦ 8 ¦ TONE ¦ 32 ¦ DVU ¦ 29 ¦ MUSIC1 ¦ 29 ¦ MUSIC2 ¦ 29 DVU ¦ DKTU ¦ 26 ¦ SLT ¦ 33 ¦ CO ¦ 33 ¦ DCO ¦ 33 ¦ MUSIC2 ¦ 8 DTMF RX ¦ SLT ¦ 26 ¦ CO ¦ 33 ¦ DCO ¦ 33 ------------------------------------- 15. FAX TRANSFER CO LINE : 3 16.EXTERNAL PAGE PORT : NOT ASSIGNED 17. DID/DISA FWD TO DVU BUSY CASE : N, NO-ANS CASE : N, ATTD NO-ANS CASE : N |
| --- | --- |

| System Feature (continued) | 18. CCR TABLE - DAY/NIGHT CCR TABLE 1.DAY TBL IDX1:HUNT 61 IDX2:____ IDX3:____ IDX4:____ IDX5:____ IDX6:INT PG1 IDX7:____ IDX8:____ IDX9:____ IDX10:____ 2.NIGHT TBL IDX1:____ IDX2:____ IDX3:____ IDX4:____ IDX5:____ IDX6:____ IDX7:____ IDX8:____ IDX9:____ IDX10:____ -OTHER CCR TABLE 1.CCR TABLE 1 IDX1:____ IDX2:____ IDX3:____ IDX4:____ IDX5:____ IDX6:____ IDX7:____ IDX8:____ IDX9:____ IDX10:____ ………… …………… 8.CCR TABLE 8 IDX1 :____ IDX2 :____ IDX3 :____ IDX4 :____ IDX5 :____ IDX6 :____ IDX7 :____ IDX8 :____ IDX9 :____ IDX10 :____ 9.CCR TABLE 9 IDX1 :____ IDX2 :____ IDX3 :____ IDX4 :____ IDX5 :____ IDX6 :____ IDX7 :____ IDX8 :____ IDX9 :____ IDX10 :____ 19.Admin Password(MAX:4DIGIT) : ____ 20. DVU SETTING - DATE/TIME PROMPT ORDER : DATE/TIME - DVU MSG RETRIEVE ORDER : FIFO - AUTO DELETE DAY OF MSG : 0 DAYS 21. WEEKLY TIME TABLE - MON -DAY:0900/NIGHT:1800 - TUE -DAY:0900/NIGHT:1800 - WED -DAY:0900/NIGHT:1800 - THU -DAY:0900/NIGHT:1800 - FRI -DAY:0900/NIGHT:1800 - SAT -DAY:____/NIGHT:00 - SUN -DAY:____/NIGHT:00 22. ACNR TONE CADENCE 1>RBT CADENCE:ON - 20/OFF- 10 2>BUSY CADENCE:ON - 25/OFF- 25 3>ERROR CADENCE:ON - 12/OFF- 12 4>S_DIAL CADENCE:ON -100/OFF- 0 ------------------------------------------------------ = SYSTEM SPEED = BIN NAME(MAX:16DIGIT) Number(MAX:24DIGIT) ----------- ------------------------------------------ 21 E E 22 E E ………… 97 E E 98 E E 99 E E |
| --- | --- |

| System Timer | SYTEM TIMER --------------------- SYSTEM TIMER 1 1. EXCLUSIVE HOLD RECALL TIMER : 60sec 2. SYSTEM HOLD RECALL TIMER : 30sec 3. TRANSFER RECALL TIMER : 30sec 4. I-RECALL TIMER : 30sec 5. ATTENDANT RECALL TIMER : 1min 6. CO RING DETECT TIMER : 2*100msec 7. CO RING STOP DETECTION TIMER : 6sec 8. PAUSE TIMER : 3sec 9. CO RELEASE GUARD TIMER : 1sec 10. CO DIAL DELAY TIMER : 5*100msec 11. PRESET CALL FORWARD TIMER : 10sec 12. CALL FORWARD BUSY/NO ANSWER TIMER : 10sec 13. DID/DISA NO ANSWER TIMER : 20sec 14. UNSUPERVISED CONF TIMER : 10min 15. MSG WAIT REMINDER TONE TIMER : 0min 16. HOOK SWITCH FLASH TIMER : 50*10msec 17. DOOR OPEN TIMER : 20*100msec 18. INTER DIGIT TIMER : 5sec 19. WARM LINE TIMER : 5sec 20. SMDR TIMER : 0sec 21. WAKE-UP FAIL RING TIMER : 20sec 22. FAX TONE DETECT TIMER : 5sec 23. FAX CO CALL TIMER : 2min 24. DVU USER RECORD TIMER : 30sec SYSTEM TIMER 2 - DVU FORWARD ANSWER TIMER : 4sec - DVU VALID USER MSG TIMER : 4sec - HUNT DISCONNECT TIMER : 1min - CALL PARK RECALL TIMER : 120sec - NORMAL RING FWD DVU TIMER : 4sec - CCR INTER DIGIT TIMER : 30sec - DISA DELAY TIMER : 0sec - ACNR PAUSE TIMER : 30sec - ACNR DELAY TIMER : 30sec - ACNR TONE DETECT TIMER : 30sec - ACNR NO ANSWER TIMER : 30sec - ACNR RETRY COUNTER : 3cnt - ACNR NO TONE DETECT COUNTER : 1cnt |
| --- | --- |
| Exception Tables | EXCEPTION TABLES ------------------------------ 1. ALLOW TABLE A (ENTRY:01-20, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 11:E 12:E 13:E 14:E 15:E 16:E 17:E 18:E 19:E 20:E 2. DENY TABLE A (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 3. ALLOW TABLE B (ENTRY:01-20, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 11:E 12:E 13:E 14:E 15:E 16:E 17:E 18:E 19:E 20:E 4. DENY TABLE B (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E CANNED TOLL TABLES ---------------------------------- 1. ALLOW TABLE (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 2. DENY TABLE (ENTRY:01-10, MAX:8 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E |
| Authorization Code Table | AUTHORIZATION CODE TABLE --------------------------------------------- (ENTRY:01-30, 5 DIGIT) 1:E 2:E 3:E 4:E 5:E 6:E 7:E 8:E 9:E 10:E 11:E 12:E 13:E 14:E 15:E 16:E 17:E 18:E 19:E 20:E 21:E 22:E 23:E 24:E 25:E 26:E 27:E 28:E 29:E 30:E |
| Flexible Numbering Plan | FLEXIBLE NUMBERING PLAN ------------------------------------------ 1.NUMBERING PLAN TYPE : BASIC 2.STA NUMBER DIGIT SIZE : 3 3.FLEXIBLE STA NUMBERING 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 |

| LCR Database | LCR DATABASE ----------------------- 1. LCR ACCESS MODE : DISABLE 2. DAY ZONE : 1:M/TU/W/TH/F/SA/SU 2: 3: 3. TIME ZONE FOR DAY ZONE 1 : 1:00-24 2:00-24 3:00-24 FOR DAY ZONE 2 : 1:00-24 2:00-24 3:00-24 FOR DAY ZONE 3 : 1:00-24 2:00-24 3:00-24 4. LDT TABLE ENTRY LCR TYPE CODE DMT_INDEX_OF_DAY_ZONE ----------------------------------------------------------------------------------------------- 0 BOTH 123456789012E 111213 212223 313233 1 BOTH E . . . . . . . . . . . . . . . . . . 2 BOTH E . . . . . . . . . . . . . . . . . . : : 98 BOTH E . . . . . . . . . . . . . . . . . . 99 BOTH E . . . . . . . . . . . . . . . . . . 5. DMT TABLE ENTRY ADDED_DIGIT R_POS R_NO A_POS CGRP ALT_DMT -------------------------------------------------------------------------------------------------------- 0 E 1 0 01 1 . . 1 12345678901234567890E 1 0 01 1 . . : : 98 3467992457E 2 0 01 1 . . 99 E 1 0 01 1 . . |
| --- | --- |
| Station Group/ VM Table | HUNT GROUP PROGRAM ------------------------------------- HUNT GROUP 61 - HUNT GROUP TYPE : NONE - HUNT GROUP MEMBER : NOT ASSIGNED - HUNT GROUP ATTRIBUTE : NOT ASSIGNED HUNT GROUP 62 - HUNT GROUP TYPE : CIR - HUNT GROUP MEMBER : 100 101 - HUNT GROUP ATTRIBUTE : CIR/TERM NO ANSWER TIMER : 5sec CIR/TERM RING TIMER : 20sec CIR/TERM OVERFLOW DESTINATION: STA 711 CIR/TERM OVERFLOW TIMER : 7sec HUNT GROUP 63 - HUNT GROUP TYPE : NONE - HUNT GROUP MEMBER : NOT ASSIGNED - HUNT GROUP ATTRIBUTE : NOT ASSIGNED HUNT GROUP 64 - HUNT GROUP TYPE : NONE - HUNT GROUP MEMBER : NOT ASSIGNED - HUNT GROUP ATTRIBUTE : NOT ASSIGNED VOICE MAIL DIALING TABLE ----------------------------------------- - PUT MAIL : PREFIX-P# , SUFFIX-E - GET MAIL : PREFIX-P##, SUFFIX-E - BUSY : PREFIX-P#*3P, SUFFIX-E - NO ANSWER : PREFIX-P#*4P, SUFFIX-E - ERROR : PREFIX-P#*5P, SUFFIX-E - DND : PREFIX-P#*6P, SUFFIX-E 7. DISCONNECT TBL : ***** |

| System Database | SYSTEM FEATURE --------------- 8.SMDR ATTRIBUTE ENABLE PRINTING : N PRINT TYPE : ALL CALL PRINT INCOM CALL : N BAUD RATE : 9600 BPS SMDR SAVE : N RECORDS IN DETAIL : N ………………….. 18. CCR(Customer Call Routin) TABLE ================================================ DVU Index : 1 -------------------------- E 1 : .... E 2 : .... E 3 : .... E 4 : .... E 5 : .... E 6 : .... E 7 : .... E 8 : .... E 9 : .... E 10 : .... …………………….. 19. Admin Password: 1234 20. DVU SETTING DATE/TIME PROMPT ORDER : DATE/TIME DVU MESSAGE RETRIEVE ORDER : FIFO AUTO DELETE DAY OF SAVED MESSAGE : 000 Days ------------------------------------------------------ = SYSTEM SPEED = BIN NAME(MAX:16CHAR) Number(MAX:24DIGIT) ----------- ------------------------------------------ 21 E E 22 PPUV 123SFP125 23 E E 24 E E 25 E E ……… ………. 95 E E 96 E E 97 E E 98 E E 99 E E |
| --- | --- |
| System Timer Database | SYSTEM TIMER 2 - CALL PARK RECALL TIMER : 120 sec - NORMAL RING TO DVIB TIMER : 4 sec - CCR INTER DIGIT TIMER : 2*100msec - CCR TIME OUT TIMER : 015 sec - DISA DELAY TIMER : 015 sec |

| Station Group/ VM Table | HUNT GROUP PROGRAM ------------------------------------- HUNT GROUP 61 - HUNT GROUP TYPE : CIR - HUNT GROUP MEMBER : 101 102 - HUNT GROUP ATTRIBUTE : - CIR/TERM NO ANSWER TIMER : 5sec - CIR/TERM RING TIMER : 20sec - Overflow Destination : STA 100 - Overflow Timer :180 sec HUNT GROUP 62 - HUNT GROUP TYPE : RING - HUNT GROUP MEMBER : 100 101 - HUNT GROUP ATTRIBUTE : - RING ANNOUNCE TIMER : 10sec - Overflow Destination : STA 100 - Overflow Timer :180 sec ………………. |
| --- | --- |